SA WG2 Meeting #136	S2-1911217

November 18-22, Reno, Nevada USA	(revision of )

Source:	Nokia, Nokia Shanghai Bell, Qualcomm

Title:	Way forward for Manufacturer assigned UE Radio Capability ID Format

Document for:	Discussion and Agreement

Agenda Item:	7.10.3

Work Item / Release:	RACS/ Rel-16

Abstract of the contribution: this paper proposes a Vendor Id based granularity for the UE Manufacturer assigned UE Radio Capability ID

# Discussion

At SA2#135 we have agreed that the basis of allocation of unique UE Radio Capability IDs should not be based on the TAC+SVN, rather some coarser granularity, as the sheer number of TACs and SVNs  combinations per TAC multiplied by the possible UE radio bundles and configurations that need to be considered would create a potentially very high number of IDs even for a single successful UE model of a UE Manufacturers.

We believe each UE vendor that manages a UE Manufacturer assigned UE Radio Capability ID database should do so with a view to optimise the number and allocation strategy of IDs according to its own policies. Hence, we propose that each Manufacturer should just claim one Vendor ID and associate a database of UE Manufacturer assigned UE Radio Capability IDs to a Vendor ID. In other words, the format of the UE Manufacturer assigned UE Radio Capability ID should be like in figure 1.

Figure 1-UE Manufacturer assigned UE Radio Capability ID format

The allocation of the Vendor ID could e.g. be based on a value of Private Enterprise Number issued by Internet Assigned Numbers Authority (IANA) in its capacity as the Private Enterprise Number administrator, as maintained at https://www.iana.org/assignments/enterprise-numbers/enterprise-numbers.

Other options may exist, like involving GSMA, but for now using the IANA PEN as the basis for Vendor IDs registration seems like an straightforward approach that also 3GPP is using in NPN context to allocate globally unique NIDs.

# Proposal

It is proposed that the UE Manufacturer assigned UE Radio Capability ID is based on the proposed high level format concept described above and the Vendor ID is a Private Enterprise Number issued by Internet Assigned Numbers Authority (IANA) in its capacity as the Private Enterprise Number administrator, as maintained at https://www.iana.org/assignments/enterprise-numbers/enterprise-numbers.