SA WG2 Meeting #S2-136	S2-1911293

18 - 22 November, 2019, Reno, Nevada, USA	(revision of S2-1909150)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1765 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Notification receiver information in a subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A subscription to notification only includes the so called notification endpoint (e.g. URI). In case of failure of this notification endpoint, the subscription to notification does not include the information to allow re-selection a new receiver for the expected notification. More information should be included in the subscription request to achieve reliability for the notifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the information to identify the notification receiver should include the information to allow the producer to discover from the NRF corresponding default notification endpoints. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of failure of the notification endpoint, notifications requests will fail, withouth the option to re-select an alternative destination. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2, new 5.21.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 1764 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 7.1.2	NF Service Consumer - NF Service Producer interactions

The end-to-end interaction between two Network Functions (Consumer and Producer) within this NF service framework follows two mechanisms, irrespective of whether Direct Communication or Indirect Communication is used:

-	"Request-response": A Control Plane NF_B (NF Service Producer) is requested by another Control Plane NF_A (NF Service Consumer) to provide a certain NF service, which either performs an action or provides information or both. NF_B provides an NF service based on the request by NF_A. In order to fulfil the request, NF_B may in turn consume NF services from other NFs. In Request-response mechanism, communication is one to one between two NFs (consumer and producer) and a one-time response from the producer to a request from the consumer is expected within a certain timeframe.

Figure 7.1.2-1: "Request-response" NF Service illustration

-	"Subscribe-Notify": A Control Plane NF_A (NF Service Consumer) subscribes to NF Service offered by another Control Plane NF_B (NF Service Producer). Multiple Control Plane NFs may subscribe to the same Control Plane NF Service. NF_B notifies the results of this NF service to the interested NF(s) that subscribed to this NF service. The subscription request shall include the notification endpoint (i.e. Notification Target Address) and a Notification Correlation ID (e.g. the notification URL) of the NF Service Consumer to which the event notification from the NF Service Producer should be sent to.

NOTE: The notification endpoint URL can be constructed by both the notification endpoint and the Notification Correlation ID.

The NF Service Consumer may also add either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id, which may be used by the NF Service Producer to discovery an alternative default notification endpoint. The producer will use both the notification endpoint and the Notification Correlation ID when sending a Notify to a NF Service Consumer. In addition, the subscription request may include notification request for periodic updates or notification triggered through certain events (e.g., the information requested gets changed, reaches certain threshold etc.). The subscription for notification can be done through one of the following ways:

-	Explicit subscription: A separate request/response exchange between the NF Service Consumer and the NF Service Producer; or

-	Implicit subscription: The subscription for notification is included as part of another NF service operation of the same NF Service; or

-	Default subscription: Registration of a notification endpoint for each type of notification the NF consumer is interested to receive, as a NF service parameter with the NRF during the NF and NF service Registration procedure as specified in TS 23.502 [3] clause 4.17.1.

Figure 7.1.2-2: "Subscribe-Notify" NF Service illustration 1

A Control Plane NF_A may also subscribe to NF Service offered by Control Plane NF_B on behalf of Control Plane NF_C, i.e. it requests the NF Service Producer to send the event notification to another consumer(s). In this case, NF_A includes the notification endpoint (i.e. Notification Target Address) and a Notification Correlation ID (e.g. the notification URL) of the NF_C in the subscription request. NF_A may also additionally include the notification endpoint and a Notification Correlation ID of NF A associated with subscription change related Event ID(s), e.g. Subscription Correlation ID Change, in the subscription request, so that NF_A can receive the notification of the subscription change related event. Additionaly, either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id may be included to allow the NF Service Producer to discovery alternative default notification endpoints.

Figure 7.1.2-3: "Subscribe-Notify" NF Service illustration 2

Routing of the messages for the NF interaction mechanisms above may be direct, as shown in the figures 7.1.2-1 to 7.1.2-3, or indirect. In case of Indirect Communication, an SCP is employed by the NF service consumer. The SCP routes messages between NF service consumers and NF service producers and may do discovery and associated selection of the NF service producer on behalf of a NF service consumer. Figure 7.1.2-4 shows the principle for a request-response interaction and figure 7.1.2-5 shows an example of a subscribe-notify interaction.

Figure 7.1.2-4: Request response using Indirect Communication

Figure 7.1.2-5: Subscribe-Notify using Indirect Communication

NOTE:	The subscribe request and notify request can be routed by different SCPs.

*** NEXT CHANGE ***

#### 5.21.3.X	Reliability for notification re-selection

When a NF Service Consumer uses a Subscription Service in a NF Service Producer, the NF Service Consumer may add either the NF instance Id and optionally the Service name, or alternatively (if applicable) NF Set Id or NF Service Set Id to allow the NF service producer to discover alternative notification endpoints. These notification endpoints are default notification endpoints that the NF Service Consumer may register in the NRF as part of the NF profile. The default notification endpoint may be either on NF level, or NF service level.

In case the provided notification endpoint by the NF Service Consumer to the NF Service Producer cannot be reached, the NF Service Producer may discover and use a default notification endpoint. In this case, the NF Service Producer may select a default notification endpoint within the construct (NF, NF service, NF Set, NF Service set) based on the information included in the subscription.

For default subscriptions, the default notification endpoints may be registered for the NF and/or NF service and if applicable within corresponding NF Set and/or NF Service Set. In this case, the NF Service Producer may select an alternative default notification endpoint within the construct (NF, NF service, NF Set, NF Service set).

*** END CHANGES ***