SA WG2 Meeting #136 	S2-1911450

18 - 22 Nov 2019, Reno, USA	 (revision of S2-190xxxx)

Source:	CATT

Title:	Solution for path selection between PC5 path and Uu path

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: this paper proposes to add a solution of Path Selection for KI #5.

Discussion

If two UEs are in proximity, the available paths for communication include Uu path and PC5 path. Which path is selected for communication should be considered, this pCR is proposed to solve the problem.

This contribution aims at addressing “Key Issue #5: Support direct communication path selection between PC5 and Uu” of TR 23.752.

Proposal

It is proposed to add a new solution for Key Issue #5.

First change

6.X	Solution for Key Issue #5: Solution to support path selection between PC5 and Uu

6.X.1	Description

This solution solves Key Issue #5 about path selection between PC5 path and Uu path. In this solution, UE selects communication path (i.e. PC5 path or Uu path) based on path selection policy received from PCF.

The path selection policy is decided by PCF based on AF request and RAN conditions (e.g. RAN user plane congestion information) and includes:

-	Traffic descriptor:

-	Application descriptors; or

-	IP descriptors.

-	Path preference: indicates the preferred path (i.e. PC5 path or Uu path) for the match traffic.

-	Validation criteria:

-	Time window: the time window when the matching traffic is allowed; or

-	Location criteria: the UE location where the matching traffic is allowed.

-	Received signal strength: the threshold of the signal strength between two UEs which are in proximity and presents only when the PC5 path is the preferred path.

Editor's Note:	How does PCF obtain the RAN conditions is FFS.

6.X.2	Procedures

6.X.2.1	PCF triggered Path Selection Policy provisioning

Path selection policy is one component of URSP (UE Route Selection Policy), so the procedure is the same as the UE Configuration Update procedure for transparent UE Policy delivery procedure (described in clause 4.2.4.3 in TS 23.502).

6.X.2.2	UE triggered Path Selection Policy provisioning

The procedure is the same as the procedure described in clause 6.2.4 in TS 23.287.

6.X.3	Impacts on Existing Nodes and Functionality

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

End of change