3GPP TSG-WG SA2 Meeting #136 	S2-1911637

# Reno, USA, November 18 – 22, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0101 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the UE Communication analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The purpose of the UE Communication analytics as defined in clause 6.7.3, TS 23.288 is to provide communication pattern parameters with the help of NWDAF, if the AF could not provide the parameters. However, if we check the input data in clause 6.7.3.2, the Communication pattern parameters from AF are included. It is unreasonable. - The data from AMF should also be taken into consideration when NWDAF derives the UE Communication analytics, however, there is no corresponding descriptions in the gereral and procedure clauses. - All the UE communication data in the input data are all from AF. However, the no-privacy related data is also very likely come from UPF due to MNO’s private implementation e.g. deep packet inspection. - All the input data is not SMF related, so it should be removed from the procedure. - The "Nnwdaf_AnalyticsInfo_Subscibe" servcice name in the procedure is not corrent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the Communication pattern parameters from the input data of UE Communication analytics. - Add descriptions related to data from AMF in the gereral and procedure clauses. - Add UPF as source of the UE communication data; - Revise the "Nnwdaf_AnalyticsInfo_Subscibe" into "Nnwdaf_AnalyticsSubscription_Subscibe". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Technically unreasonable and not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.7.3.1, 6.7.3.2, 6.7.3.4, 6.7.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.7.3	UE Communication Analytics

6.7.3.1	General

In order to support some optimized operations, e.g. customized mobility management or QoS improvement, in 5GS, an NWDAF may perform data analytics on UE communication pattern and user plane traffic, and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs. If consumer NF provides an Application ID, the NWDAF only considers the data from AF, UPF and AMF that corresponds to this application ID.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs.

-	Analytics Filter Information containing optional list of analytics subset(s)

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high).

# * * * * Next change * * * *

6.7.3.2	Input Data

The NWDAF supporting data analytic on UE communication shall be able to collect communication information for the UE from 5GC. The detailed information collected by the NWDAF includes service data related to UE communication as defined in the Table 6.7.3.2-1.

Table 6.7.3.2-1: Service Data from 5GC related to UE communication

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AF | Could be GPSI or external UE ID |
| Internal Group ID |  | To identify UE group if available |
| Application ID | AF / UPF | Identifying the application providing this information |
| UE communication (1..max) | AF / UPF | Communication description per application |
| >Communication start |  | The time stamp that this communication starts |
| >Communication stop |  | The time stamp that this communication stops |
| >UL data rate |  | UL data rate of this communication expected by the application |
| >DL data rate |  | DL data rate of this communication expected by the application |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar communication behavior. The UE whose communication behavior is unlike other UEs with the same TAC may be an abnormal one. |

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE communication information).

# * * * * Next change * * * *

6.7.3.4	Procedures

The NWDAF can provide UE communication related analytics, in the form of statistics or predictions or both, to a 5GC NF.

Figure 6.7.3.4-1: Procedure for UE communication analytics

1.	5GC NF to NWDAF: Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID=UE communication analytics, Target of Analytics Reporting=SUPI(s)).

5GC NF sends a request to the NWDAF for analytics on a specific UE(s), using either Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription_Subscribe service. The analytics type indicated by "Analytics ID" is set to "UE communication". The Target of Analytics Reporting is set to SUPI or a list of SUPI.

2a-b.	NWDAF to AF (Optional): Naf_EventExposure_Subscribe (Event ID, external UE ID).

In order to provide the requested analytics, the NWDAF may subscribe per application communication information from AFs for the UE. The Event ID indicates communication report for the UE which is requested by the 5GC NF in the step 1. The external UE ID is obtained by the NWDAF based on UE internal ID, i.e., SUPI.

This step is skipped if the NWDAF already has the requested analytics available or has subscribed to the AF.

2c-d.

NWDAF to AMF: Namf_EventExposure_Subscribe (Event ID, SUPI).

In order to provide the requested analytics, the NWDAF subscribes to AMF data from AMFs.

NOTE x:	The NWDAF determines the AMF serving the UE as described in subclause 6.2.2.1.

3.	The NWDAF derives requested analytics, in the form of UE communication statistics or predictions or both.

4.	NWDAF to 5GC NF: Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify.

The NWDAF provides requested UE communication analytics to the NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-6.	If the NF subscribed UE communication analytics at step 1, when the NWDAF generates new analytics, it provides the new generated analytics to the NF.

# * * * * Next change * * * *

6.7.4.4.1	NWDAF-assisted expected UE behavioural analytics

Figure 6.7.4.4.1-1: NWDAF assisted expected UE behavioural analytics procedure

1.	5GC NF (e.g., AMF, SMF, AF and UDM) to NWDAF: Nnwdaf_AnalyticsInfo_Request (Analytics ID, Target of Analytics Reporting=SUPI) or Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID, Target of Analytics Reporting =SUPI).

The Analytics ID is set to "UE Mobility" or to "UE Communication"," and the consumer request predictions.

2a-b.	[Conditional] The NWDAF requests to AMF Namf_EventExposure_Subscribe (Event ID, SUPI), to request location report for the UE, if the Analytics ID is set to "UE Mobility", unless the information is available.

2c-d.	[Conditional] The NWDAF requests to AF Naf_EventExposure_Subscribe (Event ID, SUPI), to request communication report for the UE, if the Analytics ID is set to "UE Communication", unless the information is available.

3.	The NWDAF derives requested analytics, i.e. predictions.

4.	NWDAF to 5GC NF: Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify.

The NWDAF provides requested Expected UE behaviour to the NF, using either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-6.	If the NF subscribed to at step 1, when the NWDAF generates new analytics, it provides the new generated analytics to the NF.

# * * * * End of changes * * * *