SA WG2 Meeting #S2-136	S2-1912209

18 - 22 November, 2019, Reno, USA 	was S2-1911025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0062 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to LCS Assistance Data Broadcast procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS |  |  |  |  |  | Date: |  |  | 2019-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN3 has endorsed a CR (R3-194935) to NRPPa protocol TS 38.455. The CR implies that a “Broadcast Procdure” in TS 23.273 sec 6.14.1 need to be bidirectional. This as the Assistance Information Transfer Procedures in sec 8.x of CR includes both an Assistance Information Control message sent to NG-RAN and an Assistance Information Feedback message sent from NG-RAN. The current procedure in 23.273 sec 6.14.1 is however unidirectional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the procedure in 23.273 sec 6.14.1 to make it bidirectional. The pattern from bidirectional procedure in 6.11.3 “Obtaining Non-UE Associated Network Assistance Data” is reused to avoid stage 3 impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 23.273 would not align to 38.305 or to stage 3 in TS 38.455. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.14.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Release changed to Rel-16. Description of information sent in step 4 modified. |  |  |  |  |  |  |  |  |

***** BEGIN 1st CHANGE *****

### 6.14.1	Broadcast of Assistance Data by an LMF

The following procedure is used by the LMF to support broadcasting of network assistance data to target UEs. This procedure is not associated with a UE location session. It is used in an AMF LCS stateless manner to send network assistance data to an NG-RAN node for broadcasting by the NG-RAN node to target UEs. NG-RAN positioning procedures related to LMF and NG-RAN node communication are specified in TS 38.455 [15].

Figure 6.14.1-1: Broadcasting Network Assistance Data

1.	The LMF invokes the Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request the transfer of a Network Assistance Data message to an NG-RAN node (gNB or ng-eNB) in the NG-RAN. The service operation includes the Network Assistance Data message and the target NG-RAN node identity. The Network Assistance Data message includes assistance data which may be optionally ciphered.

Editor's note:	It is FFS if AMF can be used to determine the NG-RAN nodes to transfer the Network Assistance Data.

2.	The AMF forwards the Network Assistance Data message to the target NG-RAN node indicated in step 1 in an N2 Transport message. The AMF includes a Routing identifier, in the N2 Transport message, identifying the LMF.

3.	The NG-RAN node broadcasts the assistance data contained in the Network Assistance Data message.

4.	The target NG-RAN node may return feedback on assistance information broadcasting to the AMF in a Network Assistance Feedback message. The target NG-RAN node shall also include the Routing identifier in the N2 Transport message received in step 2.

5.	The AMF invokes the Namf_Communication_N2InfoNotify service operation towards the LMF indicated by the routing identifier received in step 4. The service operation includes the Network Assistance Data Feedback message received in step 4.

***** END OF CHANGES *****