SA WG2 Meeting #136 	S2-1912777

18 – 22 Nov 2019, Reno, NV, USA	(e-mail revision 1 of S2-1912545)

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation of KI #2 conclusions

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_5GSAT_ARCH / Rel-16

Abstract of the contribution: conclusions on mobility key issue #2

# Discussion

Clause 7.2 evaluates three mobility solutions as follows:

- Solution #1 is simple for the 3GPP system, but it relies on so called static TA concept.

- Solution #7 works with moving TA concept but it requires additional functionality from UE and AMF.

- Solution #9 is to large extent in the RAN area. SA2 has sent LS to RAN2 asking for their evaluation of the radio related aspects of this solution. SA2 cannot conclude on this candidate solution without the guidance form the RAN experts.

It is proposed to reflect these aspects in the conclusions part of the TR 23.737 as shown below.

## 8.2	Conclusion on solutions for Key Issue #2

Key Issue #2 studied Mobility Management with moving satellite coverage area. 3GPP system should adapt to being usable over variety of satellite types without putting pre-requisite conditions on the capabilities of the satellites, or the satellite constellation to be used. The system impact on 3GPP system depends on what kind of satellites are used, and therefore, the following conclusions are made.

Both Solutions #1 and #9 have  been identified as having no impacts on the existing 3GPP specifications of the 5G-CN. It is thus concluded that no normative work is needed in SA2 for these solutions. Feedback and evaluation of these solutions has also been requested to the RAN Working Group.

Solutions #7 and #8 have both impacts on the 5G-CN. Dependency with respect to Moving Tracking Areas  in the case of Satellite based RAN has been identified for these solutions. Before concluding on the normative work for Solution #7 or Solution #8, input from RAN WGs is needed, e.g. if Moving Tracking Areas would be selected as a basis for normative activity during Rel. 17.