3GPP TSG-WG SA2 Meeting #136 	S2-1911171

# Reno, NV, USA, November 18 - 22, 2019	(revision of S2-1908940)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1727 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates for Bridge Dealy information reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Which NF is responsible for the 5GS Bridge Delay information mapping based on the cumulative rateRatio between TSN time and 5G time: The UE-DS-TT Residence Time and PDB are used by the AF to caculate the 5GS independentDelayMax/ independentDelayMin value of bridge for each port pair and for each traffic class. There may be frequency offset between TSN clock and 5G clock. The UE-DS-TT Residence Time and PDB used by the TSN AF should be in reference to TSN time. The SMF or the AF need to map the UE-DS-TT Residence Time and PDB from the 5G clock to a TSN clock and then the AF can calculates the 5GS independentDelayMax/independentDelayMin value based on the TSN time. Alternative 1: The AF is responsible for mapping the UE-DS-TT Residense time and PDB from the 5G clock to the TSN clock. Alternative 2: The SMF is responsible for mapping the UE-DS-TT Residense time and PDB from the 5G clock to the TSN clock. The SMF provides the above components in reference to TSN time to the AF via PCF during PDU Session establishment. Alternative 1 will introduce an additional subscription &notification of the cumulative rateRatio between the SMF and AF. Considering SA2 has agreed that the TSCAI is adjusted by the SMF based on the time offset and cumulative reteRatio between TSN time and 5G time. It is better to support the timing information mapping on only one NF. Therefore, Alternative 2 is proposed. - The update of cumulative reteRatio and 5GS Bridge delay information: The UPF can measure the cumulative reteRatio between TSN time and 5G time. The UPF may update the cumulative reteRatio to the SMF. If the SMF received a cumulative reteRatio update from the UPF, it determines whether to update the 5GS Bridge delay information to the PCF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The SMF maps the UE-DS-TT Residence Time and PDBs supported by the PDU Session from the 5G clock to a TSN clock based on the cumulative reteRatio between TSN time and 5G time, and the he SMF reports the UE-DS-TT Residence Time and PDBs (and the corresponding 5QIs) in reference to TSN time to the AF via PCF. - The cumulative rateRatio is reported from the UPF which can measure the cumulative rateRatio between TSN time and 5G time. - If the SMF received a cumulative rateRatio update from the UPF, it determines whether to update the 5GS Bridge delay information to the PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GS independentDelayMax/ independentDelayMin values reported to the TSN CNC are not correct and the delay requirement of the TSC traffic may not be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.502 CR 1700 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.27.5	5G System Bridge delay

In order for 5G System to participate as a TSN bridge according to gate schedules specified, the 5GS bridge is required to provide 5GS Bridge Delays for each port pair and traffic class to an IEEE TSN system. In order to determine 5GS bridge delays, following components are needed:

1.	UE-DS-TT Residence Time: the time taken within the UE and DS-TT to forward a packet between the UE/DS-TT port.

2.	The PDB as defined in clause 5.7.3.4: packet delay between the UE and the UPF that terminates the N6 interface. The PDB is determined from the 5QI (as in standardized 5QI to QoS characteristics mapping as in clause 5.7.4) assigned for the TSC QoS Flow. It is assumed that the PDB includes the UPF Residence Time and NW-TT Residence time.

Residence times are the same for uplink and downlink traffic. The TSN AF calculates the 5GS independentDelayMax/independentDelayMin value of bridge for each port pair and for each traffic class using the above components.

The SMF provides the above components in reference to the TSN clock to the AF via PCF during PDU Session establishment.  Before providing the above comonents to the AF, the SMF maps the UE-DS-TT Residence Time and the PDB values from the 5G clock to a TSN clock to the 5G clock based on cumulative rateRatio between TSN time and 5G timeas measured and reported by the UPF. The corresponding 5QIs are also provided with the PDBs by the SMF.

In the case of change of cumulative rateRatio between TSN time and 5G time, the UPF updates the cumulative rateRatio to SMF. If the SMF received a cumulative rateRatio update from the UPF, it determines whether to update the 5GS Bridge delay information to the PCF based on local policy.

NOTE 1:	With supporting the hold and forward buffering mechanism in UE-DS-TT and NW-TT, the values of independentDelayMax and independentDelayMin for 5GS Bridge can be the same.

NOTE 2:	In order to prevent frequent updates from the UPF, the UPF sends the cumulative rateRatio only when the difference between the current measurement and the previously reported measurement is larger than a threshold.

The dependentDelayMax and dependentDelayMin for 5GS Bridge are derived from frame length-dependent delays in DS-TT and in NW-TT for inputting the packet via the ingress port and outputting the packet via the egress port.

NOTE 3:	Whether the values of dependentDelayMax and dependentDelayMin are the same is dependent on implementation.

Since Residence times may vary among UEs and among UPFs, the 5GS bridge delay is determined after the PDU Session Establishment for the corresponding UPF and the UE by the AF. UE-DS-TT Residence Time is provided at the time of PDU Session Establishment by the UE to the network.

The PDU Session serving for TSC only supports PDU Session type of Ethernet and SSC mode 1 in this release of the specification.

# * * * * End of changes * * * *