3GPP TSG-SA WG2 Meeting #136	 		S2-1911693

Reno, Nevada, USA, November 18-22 2019

Title:	draft LS on Version ID in PLMN Assigned UE radio Capability ID format

Release:	Rel-16

Work Item:	RACS

Source:	SA2

To:	CT4

Cc:

Contact Person:

#### Name:			Alessio. Casati

Tel. Number:

E-mail Address:			Alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

SA2 has discussed it is a realistic assumption that during its lifetime a UCMF may change the PLMN Assigned UE Radio Capability ID assignment algorithm. Also, a PLMN operator may decide to change UCMF vendor. The problem then arises that upon change of UCMF or assignment algorithm at time T, the Set of PLMN assigned UE radio capability IDs that are allocated form time T may overlap with those allocated before time T. This can create a permanent malfunction of the PLMN assigned UE Radio capability ID scheme as UEs in the field may signal IDs that now have acquired a totally different meaning in the system. In other words, we need a mechanism to detect stale IDs in the UEs but also in the RAN and CN caches.

Hence, CT4 is kindly requested to consider that the format of the PLMN assigned ID ought to include a Version identification to detect stale IDs.

2. Actions:

To CT4:

ACTION: SA2 kindly requests to take the above into account and add a Version ID to the PLMN assigned UE Radio Capability ID format.

3. Date of Next TSG-SA WG2 Meetings:

SA2#136-AH			2020-01-13	2020-01-17	 TBD, KR

SA2#137			2019-02-24	2019-02-28	 India