3GPP TSG-SA WG2 Meeting #136 	S2- 1911703

18 - 22 November, 2019, Reno, Nevada, US

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1980 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MDBV mapping and configuration for TSC QoS Flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MDBV identifies the maximum cumulative payload (from all TSN streams that map to a TSN traffic class and its corresponding 5G QoS flow) that can be sent per instance of traffic transmission over the radio interface for any given 5G QoS flow and therefore limits the throughput of a QoS flow. At the 5GS QoS capability report phase, currently MDBV is not part of QoS capability report (sent to the AF), and it is not part of minimum set of TSN QoS-related parameters for QoS mapping either. Although MDBV is part of 5QI, but only the bridge delay and priority related parameters are reported to AF during the QoS capability report phase. The mapping of a TSC traffic class or TSC streams to a 5G QoS flow requires to match bridge delay, priority, and MDBV in the same time. During the capability report phase, the MDBV value of a reported 5QI has been decided already. There is a risk that the maximum value of TSC burst size of a traffic class (aggregated TSC streams) is larger than any reported MDBV of 5QIs. Therefore, it is not possible to find a 5QI that matches MDBV and in the same time also matches bridge delay and priority requirements for the corresponding traffic class. If AF doesn’t provide TSC burst size information, the default value for the TSC 5QI may not be enough to accommodate aggregated TSC streams. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case the Maximum Burst Size of the aggregated TSC streams in the traffic class is provided by CNC via TSN AF to PCF, PCF can derive the required MDBV taking the Maximum Burst Size and TSCAI as input. If the default MDBV associated with a 5QI in the QoS mapping table can’t satisfy the aggregated TSC Burst Size, the PCF provides the derived MDBV in the PCC rule and then the SMF performs QoS Flow binding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the TSN-5GS QoS mapping leading to stage 3 delay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### 5.28.4	QoS mapping tables

The mapping tables between the traffic class and 5GS QoS Profile is provisioned and further used to find suitable 5GS QoS profile to transfer TSN traffic over the PDU Session. QoS mapping procedures are performed in two phases: (1) QoS capability report phase as described in clause 5.28.1, and (2) QoS configuration phase as in clause 5.28.2:

(1)	The TSN Translator AF shall be pre-configured via OAM the mapping table and represents what is being reported or exposed to the TSN system (i.e. the CNC) through the relevant TSN information objects, such as the Traffic Class Table for every port, see the IEEE 802.1Q [X] and IEEE 802.1Qcc [95].

(2)	CNC distributes the TSN QoS requirements and TSN scheduling parameters (specific for current node) to 5G virtual bridge via TSN AF. Alternatively, the 5GS virtual bridge may pre-request or query CNC for the TSN QoS and traffic information.

The PCF mapping table provides a mapping from TSN QoS information to 5GS QoS profile. Based on trigger from TSN AF, the PCF may trigger PDU session modification procedure to establish a new 5G QoS flow for the requested traffic class according to the selected QoS policies from the TSN AF traffic requirements.

Figure 5.28.4-1 illustrates the functional distribution of the mapping tables.

Figure 5.28.4-1: QoS Mapping Function distribution between PCF and TSN AF

The minimum set of TSN QoS-related parameters that are relevant for mapping the TSN QoS requirements in the 5GS are: traffic classes and their priorities per port, bridge delays per port pair and traffic class (independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), and propagation delay per port (txPropagationDelay).

QoS mapping table between port traffic classes and 5QI should be matching the delay and priority, while preserving the priorities in the 5GS. An operator enabling TSN services via 5GS can choose up to eight traffic classes to be mapped to 5GS QoS profiles.

Once the 5QIs to be used for TSN streams are identified, then it is possible to enumerate as many bridge port traffic classes as the number of selected 5QIs.

Once the CNC has received the necessary information, it proceeds to calculate scheduling and paths. The configuration information is then set in the bridge per port and per traffic class. The most relevant information received is the scheduling for every traffic class and port of the bridge. At this point, it is possible to retrieve the real QoS requirements by identifying the traffic class of the port. Then the traffic class to 5QI mapping can be performed using the QoS mapping table in the TSN AF. Subsequently, the TSC QoS flow can be configured using the 5QI retrieved from the QoS mapping table. This feedback approach uses the reported information to the CNC and the feedback of the configuration information coming from the CNC to perform the mapping and configuration in the 5GS. The scheduling configuration information per traffic class is mapped to trigger creation/modification of a QoS flow in 5GS.

In case the Maximum Burst Size of the aggregated TSC streams in the traffic class is provided by CNC via TSN AF to PCF, PCF can derive the required MDBV taking the Maximum Burst Size and TSCAI as input. If the default MDBV associated with a 5QI in the QoS mapping table can not satisfy the aggregated TSC Burst Size, the PCF provides the derived MDBV in the PCC rule and then the SMF performs QoS Flow binding as specified in clause 6.1.3.2.4 of TS 23.503 [45].

*** END CHANGES ***