SA WG2 Meeting #136 	S2-1911746

# November 18-22th, 2019, Reno, NV, USA

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1848 | rev | 6 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the Inter PLMN UP functionality in the architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia?, Nokia Shanghai Bell?, Deutsche Telekom? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2019-09-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | S3-193096/S2-1908713 “LS to SA2 on UP gateway function” (now called Inter PLMN User Plane functionality / IPUPS in 23.501) where SA3 requests SA2 to specify the interface between an UPF supporting the IPUPS and a 5GC control plane entity (SMF as per agreed Solution #20 of SA3 TR ) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds the IPUPS in the over all architecture Adds IPUPS functionality support as a criteria to select an UPF In rev 6(changes are highlighted in green): propose no N4 interface between SMF and IPUPS UPF. IPUPS UPF is deployed on the path between V-UPF and H-UPF, transparent option. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of inter PLMN UP Security mechanisms on GTP-u traffic defined in 3GPP specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2 ; 4.2.4 ; 5.8.2.X (new) ; 6.2.3 ; 6.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2 - UPF dedicated to support of IPUPS |  |  |  |  |  |  |  |  |

FIRST CHANGE

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GLAN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5QI	5G QoS Identifier

AF	Application Function

AMF	Access and Mobility Management Function

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

FAR	Forwarding Action Rule

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GFBR	Guaranteed Flow Bit Rate

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LMF	Location Management Function

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

N3IWF	Non-3GPP InterWorking Function

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NSI ID	Network Slice Instance Identifier

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

PCF	Policy Control Function

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

QFI	QoS Flow Identifier

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TAC	IMEI Type Allocation Code

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSN	Time Sensitive Networking

TSP	Traffic Steering Policy

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

NEXT CHANGE (2)

4.2.4	Roaming reference architectures

Figure 4.2.4-1 depicts the 5G System roaming architecture with local breakout with service-based interfaces within the Control Plane.

Figure 4.2.4-1 Roaming 5G System architecture- local breakout scenario in service-based interface representation

NOTE 1:	In the LBO architecture. The PCF in the VPLMN may interact with the AF in order to generate PCC Rules for services delivered via the VPLMN. The PCF in the VPLMN uses locally configured policies according to the roaming agreement with the HPLMN operator as input for PCC Rule generation. The PCF in VPLMN has no access to subscriber policy information from the HPLMN.

NOTE 2:	An SCP can be used for indirect communication between NFs and NF services within theVPLMN, within the HPLMN, or in within both VPLMN and HPLMN. For simplicity, the SCP is not shown in the roaming architecture.

Figure 4.2.4-3 depicts the 5G System roaming architecture in the case of home routed scenario with service-based interfaces within the Control Plane.

Figure 4.2.4-3 Roaming 5G System architecture - home routed scenario in service-based interface representation

NOTE 3:	An SCP can be used for indirect communication between NFs and NF services within theVPLMN, within the HPLMN, or in within both VPLMN and HPLMN. For simplicity, the SCP is not shown in the roaming architecture.

Figure 4.2.4-4 depicts 5G System roaming architecture in the case of local break out scenario using the reference point representation.

Figure 4.2.4-4: Roaming 5G System architecture - local breakout scenario in reference point representation

NOTE 2:	The NRF is not depicted in reference point architecture figures. Refer to Figure 4.2.4-7 for details on NRF and NF interfaces.

NOTE 3:	For the sake of clarity, SEPPs are not depicted in the roaming reference point architecture figures.

The following figure 4.2.4-6 depicts the 5G System roaming architecture in the case of home routed scenario using the reference point representation.

Figure 4.2.4-6: Roaming 5G System architecture-Home routed scenario in reference point representation

For the roaming scenarios described above each PLMN implements proxy functionality to secure interconnection and hide topology on the inter-PLMN interfaces.

Figure 4.2.4-7: NRF Roaming architecture in reference point representation

NOTE 4:	For the sake of clarity, SEPPs on both sides of PLMN borders are not depicted in figure 4.2.4-7.

In roaming scenarios, the I-NEF may be deployed. The I-NEF is described in clause 6.2.5a. Figure 4.2.4-8 depicts the roaming architecture for Network Exposure Function, using reference point representation.

Figure 4.2.4-8: Roaming architecture for Network Exposure Function in reference point representation

NOTE 5:	The reference architecture in figure 4.2.4-8 supports service based interfaces on the I-NEF.

Operators can deploy UPFs supporting the Inter PLMN UP Security (IPUPS) functionality at the border of their network to protect their network from inter PLMN N9 traffic in home routed roaming scenarios. Such UPFs may be UPFs with several UPF features or UPFs dedicated to IPUPS functionality. Figure 4.2.4-9 depicts the home routed roaming architecture with the deployment option of UPF dedicated to support of IPUPS functionality, using service-based interface representation.

The IPUPS functionality is specified in clause 5.8.2.X and TS 33.501 [29].

Figure 4.2.4-9 Roaming 5G System architecture - home routed roaming scenario in service-based interface representation employing UPF dedicated to IPUPS

NEXT CHANGE (3) all text is new

5.8.2.X	Inter PLMN User Plane Security functionality

Operators can deploy UPF(s) supporting the Inter PLMN User Plane Security (IPUPS) functionality at the border of their network to protect their networks from inter PLMN N9 traffic.

The IPUPS functionality corresponds to following features:

- secure traffic transmsion between VPLMN and HPLMN

The IPUPS functionality is further specified in TS 33.501 [29].

Operators can deploy UPF dedicated to the IPUPS functionality or deploy UPF supporting IPUPS functionality together with other functionalities for the same data traffic.

NOTE: UPF supporting IPUPS is deployed on the path between V-UPF and H-UPF, transparent to V-UPF and H-UPF.

NEXT CHANGE (3B)

6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functionalities may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	Allocation of UE IP address/prefix (if supported) in response to SMF request.

-	External PDU Session point of interconnect to Data Network.

-	Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session, support of traffic forwarding within a 5G VN group (UPF local switching, via N6, via N19)).

-	Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition), and packet filtering.

-	User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

-	Uplink Traffic verification (SDF to QoS Flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

-	Sending and forwarding of one or more "end marker" to the source NG-RAN node.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Packet duplication in downlink direction and elimination in uplink direction in GTP-U layer.

-	TSN Translator functionality to hold and forward user plane packets for de-jittering when 5G System is integrated as a bridge with the TSN network.

-	High latency communication, see clause 5.31.8.

NOTE:	Not all of the UPF functionalities are required to be supported in an instance of user plane function of a Network Slice.

- Inter PLMN UP Security (IPUPS) functionality, specified in clause 5.8.2.X.

NEXT CHANGE (4)

6.3.3.3	Selection of an UPF for a particular PDU Session

If there is an existing PDU Session, and the SMF receives another PDU Session request to the same DNN and S-NSSAI, and if the SMF determines that interworking with EPC is supported for this PDU Session as specified in clause 4.11.5 of TS 23.502 [3], the SMF should select the same UPF, otherwise, if the SMF determines that interworking with EPC is not supported for the new PDU Session, a different UPF may be selected.

For the same DNN and S-NSSAI if different UPF are selected at 5GC, when the UE is moved to EPC network, there is no requirement to enforce APN-AMBR. Whether and how to apply APN-AMBR for the PDN Connection associated with this DNN/APN is implementation dependent, e.g. possibly only per PDU Session AMBR enforcement applies.

The following parameter(s) and information may be considered by the SMF for UPF selection and re-selection:

-	UPF's dynamic load.

-	Analytics (i.e. statistics or predictions) for UPF load as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

-	UPF's relative static capacity among UPFs supporting the same DNN.

-	UPF location available at the SMF.

-	UE location information.

-	Analytics (i.e. statistics or predictions) for UE's mobility as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

-	Capability of the UPF and the functionality required for the particular UE session: An appropriate UPF can be selected by matching the functionality and features required for an UE.

-	Data Network Name (DNN).

-	PDU Session Type (i.e. IPv4, IPv6, IPv4v6, Ethernet Type or Unstructured Type) and if applicable, the static IP address/prefix.

-	SSC mode selected for the PDU Session.

-	UE subscription profile in UDM.

-	DNAI as included in the PCC Rules and described in clause 5.6.7.

-	Local operator policies.

-	S-NSSAI.

-	Access technology being used by the UE.

-	Information related to user plane topology and user plane terminations, that may be deduced from:

-	5G-AN-provided identities (e.g. CellID, TAI), available UPF(s) and DNAI(s);

-	Identifiers (i.e. FQDN or IP address) of N3 terminations provided by a W-AGF or a TNGF;

-	Information regarding the user plane interfaces of UPF(s). This information may be acquired by the SMF using N4;

-	Information regarding the N3 User Plane termination(s) of the AN serving the UE. This may be deduced from 5G-AN-provided identities (e.g. CellID, TAI);

-	Information regarding the N9 User Plane termination(s) of UPF(s) if needed;

-	Information regarding the User plane termination(s) corresponding to DNAI(s).

-	RSN when redundant handling is applicable.

-	Information regarding the ATSSS Steering Capability of the UE session (ATSSS-LL capability, MPTCP capability, or both).

-	Support for UPF allocation of IP address/prefix.

-	Support of the IPUPS functionality, specified in clause 5.8.2.X.

-	Support for High latency communication (see clause 5.31.8).

NOTE:	How the SMF determines information about the user plane network topology from information listed above, and what information is considered by the SMF, is based on operator configuration.

A W-AGF or aTNGF may provide Identifiers of its N3 terminations when forwarding over N2 uplink NAS signalling to the 5GC. The AMF may relay this information to the SMF, as part of session management signalling for a new PDU Session.

END OF CHANGES