SA WG2 Meeting #S2-136	S2-1911989

18 - 22 November, 2019, Reno USA

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1854 | rev | 3 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Managament of GBR QoS Flows at handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At mobility GBR QoS Flows are subject to admission control at the target RAN that may result in rejection of such QoS Flows. Two issues are are identified: - target RAN rejects unnecessarily a GBR QoS Flow instead of applying same functionality as specified by Notification Control logic, i.e. accept the GBR QoS Flow, operate on the supported QoS level even when GFBR can’t be sustained and and finally strive to get back to the GFBR. - source RAN has no visibility in the QoS level that can be supported by target RAN and thus can’t perform a proper target RAN selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR corrects the management of GBR QoS Flows during the handover reducing the amount of unnecessary GBR QoS Flow rejections by: - introducing information exchange about the QoS level that can be supported by target to the source side in the handover preparation phase; - introducing the possibility to accept a GBR QoS Flow by a target that currently can’t sustain the GFBR of that GBR QoS Flow. To address the above following changes are introduce by this CR. - To have a better understanding what target RAN to select, a mechanism is introduced that enables a potential target RAN to provide information to the source RAN about the QoS level that can be supported. - To eliminate the need to reject a GBR QoS Flow due to temporary limitations in the QoS level that can be supported by a target RAN, functionality based on Notification Control logic that allows the target RAN to accept the GBR QoS flow even when the GFBR can’t be sustained, is introduced. Post-handover the target RAN may, if needed, issue a Notification Control towards the 5GC including the information about GFBR, PDB, PER that currently can be fulfilled and alternatively also providing the reference to the Alternative QoS profile that matches GFBR, PDB, PER that currently can be fulfilled. The evaluation on the target side takes the priority order of the Alternative QoS profiles into consideration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Selection of inappropriate target RAN at handover resulting in unnecessary rejection of GBR QoS Flows. Missing support to accept a GBR QoS Flow at temporary lack of support for the associated GFBR resulting in unnecessary rejection of GBR QoS Flows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 3: clarified that the reference to Alt QoS Profile is included in Path Switch Request and HO Notify. Removal of the attribute values reporting |  |  |  |  |  |  |  |  |

*******BEGIN OF CHANGES********

#### 5.7.2.4	Notification control

The QoS Parameter Notification control indicates whether notifications are requested from the NG-RAN when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. Notification control may be used for a GBR QoS Flow if the application traffic is able to adapt to the change in the QoS (e.g. if the AF is capable to trigger rate adaptation).

The SMF shall only enable Notification control when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow. The Notification control parameter is signalled to the NG-RAN as part of the QoS profile.

If, for a given GBR QoS Flow, Notification control is enabled and the NG-RAN determines that the GFBR can no longer be guaranteed, NG-RAN shall send a notification towards SMF and may provide values for the GFBR, the PDB and the PER parameters it can guarantee. Furthermore, the NG-RAN shall keep the QoS Flow (i.e. while the NG-RAN is not delivering the requested GFBR for this QoS Flow), unless specific conditions at the NG-RAN require the release of the NG-RAN resources for this GBR QoS Flow, e.g. due to Radio link failure or RAN internal congestion. The NG-RAN should try to guarantee the GFBR again.

NOTE 1:	NG-RAN can decide that the GFBR can no longer be guaranteed based on, e.g. measurements like queuing delay or system load.

If the NG-RAN has received Alternative QoS profile(s) for this QoS Flow and supports the Alternative QoS profile handling, the NG-RAN shall, before sending a notification that the GFBR can no longer be guaranteed towards the SMF, check whether the values of the GFBR, the PDB and the PER parameters that the NG-RAN can guarantee match any of the Alternative QoS profile(s), and if there is a match, the NG-RAN shall indicate the reference to the Alternative QoS profile(s) together with the notification to the SMF.

Upon receiving a notification from the NG-RAN that the GFBR can no longer be guaranteed, the SMF may forward the notification to the PCF, see TS 23.503 [45]. 5GC may initiate N2 signalling to modify or remove the QoS Flow.

When the NG-RAN determines that the GFBR can be guaranteed again for a QoS Flow (for which a notification that the GFBR can no longer be guaranteed has been sent), the NG-RAN shall send a notification, informing the SMF that the GFBR can be guaranteed again and the SMF may forward the notification to the PCF, see TS 23.503 [45]. The NG-RAN shall send a subsequent notification that the GFBR can no longer be guaranteed whenever necessary.

NOTE 2:	It is assumed that NG-RAN implementation will apply some hysteresis before determining that the GFBR can be guaranteed again and therefore a frequent signalling of GFBR can be guaranteed again followed by GFBR can no longer be guaranteed is not expected.

NOTE 3:	If the QoS Flow is modified, the NG-RAN restarts the check whether the GFBR can no longer be guaranteed according to the updated QoS profile. If the Notification control parameter is not included in the updated QoS profile, the Notification control is disabled.

During a handover, the Source NG-RAN does not inform the Target NG-RAN about whether the Source NG-RAN has sent a notification that the GFBR for a QoS Flow can no longer be guaranteed. The Target NG-RAN performs admission control rejecting any QoS Flows for which resources cannot be permanently allocated. If the Target NG-RAN is not able to guarantee the GFBR, PDB or PER included in the QoS profile and if Alternative QoS profiles are provided to the Target NG-RAN and the Target NG-RAN supports Alternative QoS profiles, the Target NG-RAN checks whether the GFBR, the PDB and the PER parameters that it can fulfil match any of the Alternative QoS profile(s) taking the priority order into account. If there is a match between one of the Alternative QoS Profiles and the GFBR, the PDB and the PER parameters that Target NG-RAN can fulfil, the Target NG-RAN shall indicate the reference to the Alternative QoS profile(s) it can currently fulfil to the Source NG-RAN.

NOTE:	The Target NG-RAN aims at fulfiling the GFBR, the PDB and the PER of the QoS profile again as specified above.

Target NG-RAN rejects QoS Flows for which the Target NG-RAN is not able to guarantee the GFBR, PDB or PER included in the QoS profile and for which no match any of the Alternative QoS profile(s) is available. The accepted QoS Flows and the accepted QoS Flow(s) for which GFBR, PDB, PER can’t be guaranteed together with the reference(s) to the Alternative QoS Profile(s), are included in the N2 Path Switch Request or N2 Handover Notify message from the Target NG-RAN to the AMF. The SMF shall interpret the fact that a QoS Flow is listed as transferred QoS Flow without a reference to an Alternative QoS Profile in the Nsmf_PDUSession_UpdateSMContext Request received from the AMF as a notification that GFBR can be guaranteed again for this QoS Flow. The SMF shall notify the PCF about QoS Flows for which GFBR can be guaranteed again as well as about the QoS Flows for which a reference to an Alternative QoS Profile has been provided.

*******END OF CHANGES********