3GPP TSG-SA2 Meeting #136	S2-1912263_was_S2-1911434

# Nov 18 - 22, 2019, Reno, USA

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1912 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PCF discovery via BSF to consider eSBA producer binding indication principles – BSF Services (23.502) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the “principles for binding, selection and reselection” in subclause 6.3.1.0 of TS 23.501, the NF service producer may provide a binding indication to the NF service consumer. Thus, the PCF as an NF service producer can provide a binding indication to the SMF/SCP for the Npcf_SmPolicyControl service. If the PCF provides a binding indication to the SMF/SCP, the SMF/SCP may re-select for a subsequent related service request another PCF instance based on the binding indication for the ongoing SM policy association/PDU session, e.g. when the original PCF instance becomes unavailable. The purpose of the BSF is to enable AF or NEF to discover the PCF instance serving an ongoing PDU session (for the so-called session binding). To enable this discovery, the PCF instance serving a PDU session registers itself at PDU session establishment at the BSF. However, in addition to that, Subclause 6.3.1.0 of TS 23.501 says “The NF service producer may provide a binding indication to the NF service consumer as part of the Direct or Indirect Communication procedures, to be used in subsequent related service requests.” According that, the PCF may register a binding indication at the BSF. Doing that will enable the PCF to indirectly provide its binding indication to the AF or NEF so that it’s consistent with the information provided to the SMF over N7. NOTE: In addition to registering a binding indication at the BSF, the PCF registers PCF address(es) and PCF id at the BSF. This is to keep backward compatibility with R15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PCF may register a binding indication at the BSF. The BSF then provides this information as part of the discovery procedure to AF or NEF. Rev s2-1912263: 1. Using the correct agreed baseline (SA2 #135 S2-1909101) 2. Retricting the binding indication to only NF Instance or NF Set rows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If an AF or NEF uses the BSF after the PCF instance within a set has changed, it will obtain an outdated PCF instance that may no longer be available. There will also be an inconsistency between the information provided by the PCF to the SMF with regarding to binding and what is provided to the BSF for consumption by the AF/NEF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.13.2.2, 5.2.13.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 5.2.13.2.2	Nbsf_Management_Register service operation

Service Operation name: Nbsf_Management_Register.

Description: Registers the tuple (UE address(es), SUPI, GPSI, DNN, DN information (e.g. S-NSSAI), PCF address(es) PCF id, PCF  PCF NF Instance binding indication or PCF NF Set binding indication as defined in TS 23.501 [2] Table 6.3.1.0) for a PDU Session.

Inputs, Required: UE address(es), PCF address(es)

UE address can contain IP address/prefix or MAC address as defined in TS 23.501 [2]. It can optionaly refer to Frame Routes. W-5GAN specific UE IP address information is specified in TS 23.316 [53].

Frame Routes are defined in Table 5.2.3.3.1-1.

Inputs, Optional: DNN, SUPI, GPSI, DN information (e.g. S-NSSAI), PCF NF Instance binding indication or PCF NF Set binding indication, PCF ID (see subclause 6.3.1.0 of TS 23.501 [2]).

Outputs, Required: Result indication, Binding Identifier for a PDU Session.

Outputs, Optional: None.

2nd change

##### 5.2.13.2.4	Nbsf_Management_Discovery service operation

Service Operation name: Nbsf_Management discovery

Description: Discovers the PCF and PCF set selected for the tuple (UE address(es), SUPI, GPSI, DNN, DN information (e.g. S-NSSAI)).

Inputs, Required: UE address (i.e. IP address or MAC address), DNN [Conditional], DN information (e.g. S-NSSAI) [Conditional]

Inputs, Optional: SUPI, GPSI

Outputs, Required: PCF address(es)

Outputs, Optional: PCF ID, PCF NF Instance binding indication or PCF NF Set binding indication (see subclause 6.3.1.0 of TS 23.501 [2]).

End of changes