3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2001856

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2088 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding two new event IDs for AMF event exposure. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 6.7.2.2, TS 23.288, the NWDAF can obtain two parameters “Type Allocation code (TAC)” and “Frequent mobility re-registration” from the AMF as the input data for UE moblity analytics generation. And in clause 6.2.2, TS 23.288, it is speficified that the NWDAF can collect data from NF using on NF event exposue service. However, the two parameters “Type Allocation code (TAC)” and “Frequent mobility re-registration” are not included in the event IDs that AMF can expose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding two new event IDs “Type Allocation code (TAC)” and “Frequent mobility re-registration” for AMF event exposure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing AMF event IDs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.2.2.3.1	General

Service description: This service enables an NF to subscribe and get notified about an Event ID.

Following UE access and mobility information event are considered (Event ID is defined in clause 4.15.1 and Table 4.15.3.1-1):

-	Location changes (TAI, Cell ID, N3IWF/TNGF node, UE local IP address and optionally UDP source port number, Area Of Interest);

-	UE moving in or out of a subscribed "Area Of Interest" as described in clause 5.6.11 in TS 23.501 [2];

-	Number of UEs served by the AMF and located in "Area Of Interest";

-	Time zone changes (UE Time zone);

-	Access Type changes (3GPP access or non-3GPP access);

-	Registration state changes (Registered or Deregistered);

-	Connectivity state changes (IDLE or CONNECTED);

-	UE loss of communication;

-	UE reachability status;

-	UE indication of switching off SMS over NAS service;

-	Subscription Correlation ID change (implicit subscription);

-	Type Allocation code (TAC);

-	Frequent mobility re-registration;

-	Subscription Correlation ID addition (implicit subscription) ; and,

-	User State Information in 5GS.

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides some examples on how the conditions to match for event reporting can be specified for various Event IDs for AMF exposure.

NOTE:	The conditions to match can be set based on AMF-associated expected UE Behaviour parameter(s) to only notify the event when the UE's behaviour deviates from its expected UE behaviour as described in TS 23.288 [50].

Table 5.2.2.3.1-1: Example of Event Filters for AMF exposure events

| Event ID | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Area of Interest | <Parameter Type = TAI, Value = TA1> <Parameter Type = PRA ID, Value = PRA ID value> |
| Access Type | <Parameter Type=AN Type, Value=3GPP Access"> |
| Location | <Parameter Type=TAI, Value=wildcard> (to report any TAI change) |
| Location | <Parameter Type=TAI Value=abnormal> (to report only when the TAI deviates from expected values based on Expected UE Moving Trajectory). |

The following service operations are defined for the Namf_EventExposure service:

-	Namf_EventExposure_Subscribe.

-	Namf_EventExposure_UnSubscribe.

-	Namf_EventExposure_Notify.

# * * * * End of changes * * * *