SA WG2 Meeting #137E	S2-2002202

February 24 – 27, 2020, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	Revised SID: Study on enhanced support of Industrial IoT - TSC/URLLC enhancements

Document for:	Approval

Agenda Item:	9.1

Work Item:	FS_IIoT

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Study on enhanced support of Industrial IoT

## Acronym: FS_IIoT

## Unique identifier: 	{A number to be provided by MCC at the plenary}

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  |  |  |  |
| Don't know | X |  | X |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 720005 | New Services and Markets Technology Enablers (SMARTER) | Stage 1 work item |
|  | New WID on LAN support in 5G (5GLAN) | Stage 1 work item |
| 800007 | Service requirements for cyber-physical control applications in vertical domains (cyberCAV) | Stage 1 work item |
| 800028 | Study on 5GS Enhanced support of Vertical and LAN Services | Antecedent Stage 2 study item |
| 820017 | 5GS Enhanced support of Vertical and LAN Services | Antecedent Stage 2 work item |
|  | Study on enhancements for cyber-physical control applications in vertical domains (FS_eCAV). | Agreed Stage 1 study item |
|  | TR 22.827 Feasibility Study on Audio-Visual Service Production | Agreed Stage 1 study item |

## 3	Justification

Rel-16 normative work was based on conclusion of the FS_Vertical_LAN study described in TR 23.734. However, some aspects of TSC (time sensitive communication) could not be agreed to be part of Rel-16 study conclusion.  In addition, SA1 is working on Enhanced Cyber Physical Control applications and Audio-Visual Service Production.

Rel-16 starts to introduce some generic enablers (e.g. TSCAI) for native Time Sensitive communication using 5G System. However, the integration of 5GS is specifically introduced for IEEE TSN networks only. Furthermore, it relies on the application layer protocol and DS-TT/NW-TT to support certain functionalities (e.g. appropriate routing and delivery of packets, synchronization). Enhancements to Time sensitive communication needs to be studied.

Enhanced Cyber Physical Control applications needs to be studied:

- Enhancements to support Industrial Ethernet integration, which includes uplink, UE - UE time synchronization, enhanced support for time-sensitive networking (TSN) with UE – UE routing via same UPF;

New service requirements from Audio-Visual Service Production application needs to be studied.

## 4	Objective

Study enhancements to 5G System that would enable enhanced support of Time Sensitive Communication and deterministic applications.

The following aspects are in scope of the study:

1.	Enhanced support of integration with IEEE TSN:

- Support for uplink synchronization via 5GS.

- Support for multiple working clock domains connected to the UE (considering uplink synchronisation with UE as master).

- Support for Time Synchronization of UE(s) with the TSN GM attached to the UE side via 5G System.

2.  Enhanced support of deterministic applications:

- UE-UE TSC communication via same UPF

- Exposure of network capability to support Time sensitive communication, more specifically deterministic services and Time synchronization.

3. Consider the following aspects:

- Specific TSC related requirements needed for Audio Visual Service Production.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 23.700-20 | Study on enhanced support of IIoT in 5G System | SA#86 March 2020 | SA#87 June 2020 | Nokia, Devaki Chandramouli |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

## 6	Work item Rapporteur(s)

Nokia, Devaki Chandramouli, devaki.chandramouli@nokia.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

Potential impacts to SA3, SA5. Potential interactions with SA1 during the course of the work.

## 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

NTT DoCoMo

Interdigital

Verizon

SK Telecom

ETRI

NICT

AT&T

Telecom Italia

BMWi

KDDI

Siemens

KT

China Mobile

ITRI

ZTE

Volkswagen AG

Bosch

Deutsche Telecom

Samsung

Huawei

ZTE

Mediatek

Sennheiser

Intel