SA WG2 Meeting #137E (e-meeting)	S2-2002040

# Feb 24 - 27, 2020, Elbonia		                                          			  (revision of S2-200)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | DRAFT | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of different slices over different Non 3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, 5WWC |  |  |  |  |  | Date: |  |  | 2020-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Per Current 3GPP specifications, - the AMF receives from the 5G AN (in N3GPP case: N3IWF / TNGF / W-AGF) information on the slices (S-NSSAI) supported by the 5G AN TA(s). This is done as part of NG set up request (38.413) >>>>TAI Slice Support List; M; Slice Support List 9.3.1.17; Supported S-NSSAIs per TA.; -. - all the 5G AN (N3IWF / TNGF / W-AGF) correspond to the same unique TA This mean that support of slices has to be homogeneous between (Un)trusted Non 3GPP access (N3IWF/TNGF) and wireline access (W-AGF) as well as between different wireline AN of a PLMN; This looks like an un-necessary restriction |
| Summary of change: |  | Support the possibility for operators to allocate a TAI per non 3GPP 5G AN (e.g. N3IWF / TNGF / W-AGF) : each non 3GPP 5G AN is locally configured with its own TAI whose value is provided (as in case of 3GPP access) to AMF over NGAP/N2 in NG SET UP message. Different non 3GPP 5G AN (e.g. different N3IWF / TNGF / W-AGF) can thus advertise different TAI values. Then the same algorithm than in case of 3GPP access can apply for: - the determination of the supported S-NSSAI over Non 3GPP access - the determination of I-SMF involvement for a PDU Session over Non 3GPP access (ETSUN) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of slices has to be homogeneous between (Un)trusted Non 3GPP access (N3IWF/TNGF) and wireline access (W-AGF) as well as between different wireline AN of a PLMN.. |
| Clauses affected: |  | 5.3.2.3 ; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

FIRST CHANGE

#### 5.3.2.3	Registration Area management

Registration Area management comprises the functions to allocate and reallocate a Registration area to a UE. Registration area is managed per access type i.e., 3GPP access or Non-3GPP access.

When a UE registers with the network over the 3GPP access, the AMF allocates a set of tracking areas in TAI List to the UE. When the AMF allocates registration area, i.e. the set of tracking areas in TAI List, to the UE it may take into account various information (e.g. Mobility Pattern and Allowed/Non-Allowed Area (refer to clause 5.3.4.1)). An AMF which has the whole PLMN as serving area may alternatively allocate the whole PLMN ("all PLMN") as registration area to a UE in MICO mode (refer to clause 5.4.1.3).

The 5G System shall support allocating a Registration Area using a single TAI List which includes tracking areas of any NG-RAN nodes in the Registration Area for a UE.

TAI(s) dedicated to Non-3GPP access, N3GPP TAI, may be defined in a PLMN and apply within thisPLMN: each non 3GPP 5G AN (e.g. N3IWF / TNGF / W-AGF) is locally configured with its own TAI whose value is provided (as in case of 3GPP access) to AMF as part of the N2 Configuration procedure (see 23.502 [2] clause4.2.7.1).

When a UE registers with the network over the Non-3GPP access, the AMF allocates to the UE a registration area that only includes the N3GPP TAI corresponding to the Non 3GPP 5G AN (e.g. N3IWF / TNGF / W-AGF). . This N3GPP TAI corresponds to the value indicated by the Non 3GPP 5G AN.

When generating the TAI list, the AMF shall include only TAIs that are applicable on the access type (i.e. 3GPP access or Non-3GPP access) where the TAI list is sent.

NOTE:	To prevent extra signalling load resulting from Mobility Registration Update occurring at every RAT change, it is preferable to avoid generating a RAT-specific TAI list for a UE supporting more than one RAT.

For all 3GPP Access RATs in NG-RAN and for Non-3GPP Access, the 5G System supports the TAI format as specified in TS 23.003 [19] consisting of MCC, MNC and a 3-byte TAC only.

The additional aspects for registration management when a UE is registered over one access type while the UE is already registered over the other access type is further described in clause 5.3.2.4.

To ensure a UE initiates a Mobility Registration procedure when performing inter-RAT mobility to or from NB-IoT, a Tracking Area shall not contain both NB-IoT and other RATs cells (e.g. WB-E-UTRAN, NR), and the AMF shall not allocate a TAI list that contains both NB-IoT and other RATs Tracking Areas.

The AMF determines the RAT type the UE is camping on based on the Global RAN Node ID associated with the N2 interface and additionally the Tracking Area indicated by NG-RAN. When the UE is accessing NR using unlicensed bands, as defined in clause 5.4.8, an indication is provided in N2 interface as defined in TS 38.413 [34].

NEXT CHANGE

END OF CHANGES