SA WG2 Meeting #138E (e-meeting)	S2-2002690

# April 20 - 24, 2020, Elbonia		                                          			  (revision of S2-2002690)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2225 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of ETSUN within and between PLMN(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2020-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The conditions within a PLMN to support Deployments Topologies with specific SMF Service Areas of the feature are not defined. Whether and How to support Home Routing is not defined when the VPLMN requires ETSUN for a PDU Session while the HPLMN has not deployed ETSUN: the AMF in the VPLMN may initiate a V-SMF change that is not supported by the H-SMF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define - The conditions within a PLMN to support Deployments Topologies with specific SMF Service Areas of the feature. - How to support Home Routing is not defined when the VPLMN requires ETSUN for a PDU Session while the HPLMN has not deployed ETSUN; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unspecified how to deal with non homogeneous support of ETSUN especially between HPLMN and VPLMN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 23.502 CR 2174 to add the indication of support of ETSUN by the H-SMF in Table 5.2.2.2.2-1 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE ()

### 5.34.x	Support of the Deployment Topologies with specific SMF Service Areas feature within and between PLMN(s)

When Deployments Topologies with specific SMF Service Areas need to be used in a PLMN for a S-NSSAI, all AMF serving this S-NSSAI are configured to support Deployments Topologies with specific SMF Service Areas

NOTE:	The specifications do not support AMF selection related with Deployment Topologies with specific SMF Service Areas

For HR roaming, the AMF discovers at PDU Session establishment whether a H-SMF supports V-SMF change based on feature support indication received from the NRF, possibly via the SCP. When the V-PLMN requires Deployments Topologies with specific SMF Service Areas but no H-SMF can be selected that supports V-SMF change, a H-SMF not supporting V-SMF change may be selected by the VPLMN. To support this case, the following applies

- The information that the H-SMF does not support V-SMF change may also be discovered by the AMF from the V-SMF after an EPS to 5GS mobility

- The information that the H-SMF does not support V-SMF change is propagated between AMF(s) at inter AMF mobility.

- In this release of the specifications, when an AMF detects the need to change the V-SMF while the H-SMF does not support V-SMF change, the AMF shall not trigger V-SMF change but shall trigger the release of the PDU Session.

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES