3GPP TSG-SA2 Meeting #138-E	S2-2002734

# Online, , 20th Apr 2020 - 23rd Apr 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2183 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handover of a PDU Session procedure between 3GPP and trusted non-3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Handover procedures between 3GPP and trusted non-3GPP access are missing in clause 4.9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause 4.9.x for “Handover of a PDU Session procedure between 3GPP and trusted non-3GPP access”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If handover procedures between 3GPP and trusted non-3GPP access are missing, the specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

4.9.x	Handover of a PDU Session procedure between 3GPP and trusted non-3GPP access

4.9.x.1	General

Clause 4.9.x defines the procedures to support hand over a PDU Session between 3GPP access and trusted non-3GPP access by describing the differences compared to the defined procedures in clause 4.9.2.

4.9.x.2	Handover of a PDU Session procedure from trusted non-3GPP to 3GPP access (non-roaming and roaming with local breakout)

The handover of a PDU Session procedure from trusted non-3GPP to 3GPP access (non-roaming and roaming with local breakout) shall be supported as specified in clause 4.9.2.1 for the handover from untrusted non-3GPP to 3GPP (non-roaming and roaming with local breakout) access with the following modifications:

-	The untrusted non-3GPP access is substituted by a trusted non-3GPP access point (TNAP).

-	The N3IWF is substituted by the TNGF.

4.9.x.3	Handover of a PDU Session procedure from 3GPP to trusted non-3GPP access (non-roaming and roaming with local breakout)

The handover of a PDU Session procedure from 3GPP to trusted non-3GPP access (non-roaming and roaming with local breakout) shall be supported as specified in clause 4.9.2.2 for the handover from 3GPP to untrusted non-3GPP access (non-roaming and roaming with local breakout) with the following modifications:

-	The untrusted non-3GPP access is substituted by a trusted non-3GPP access point (TNAP).

-	The N3IWF is substituted by the TNGF.

-	The PDU Session is activated in step 2 as specified in clause 4.12a.5.

4.9.x.4	Handover of a PDU Session procedure from trusted non-3GPP to 3GPP access (home routed roaming)

The handover of a PDU Session procedure from trusted non-3GPP to 3GPP access (home routed roaming) shall be supported as specified in clause 4.9.2.3.1 for the handover from untrusted non-3GPP to 3GPP access (home routed roaming) with the following modifications:

-	The untrusted non-3GPP access is substituted by a trusted non-3GPP access point (TNAP).

-	The N3IWF is substituted by the TNGF.

4.9.x.5	Handover of a PDU Session procedure from 3GPP to trusted non-3GPP access (home routed roaming)

The handover of a PDU Session procedure from 3GPP to trusted non-3GPP access (home routed roaming) shall be supported as specified in clause 4.9.2.4.1 for the handover from 3GPP to untrusted non-3GPP access (home routed roaming) with the following modifications:

-	The untrusted non-3GPP access is substituted by a trusted non-3GPP access point (TNAP).

-	The N3IWF is substituted by the TNGF.

-	The PDU Session is activated in step 2 as specified in clause 4.12a.5.

End of change