3GPP SA WG2 Meeting #138E 	S2-2002946

April 20 - 24, 2020, Elbonia              	(revision of S2-20xxxxx)

# Source:	Huawei, Hisilicon

# Title:	New WID on enhanced application level descriptor in UE related rules

# Document for:	Endorsement

Agenda Item:	9.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Enhanced application level descriptor in UE related rules

Acronym: EALD

Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-17.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X |  | X |  |
| No |  |  | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

3	Justification

The Application Descriptor has been introduced as one form of Traffic Descriptor in URSP in Rel-15. Application Descriptor is an efficient granularity for network to enforce traffic matching, especially for several core features of the 5G system, e.g. slicing and edge computing.

As discussed in S2-2002943, currently, the application descriptor defined by OSid+OSappid does not work well because

- The OSid is a poorly defined parameter and cannot reflect the diversity of environments of devices that are supposed to be used in 5G systems now and in the future. Applications may run on a variety of OSes, some of which are proprietary and domain-specific. For instance, the same type of V2X terminal may support variety of existing and future OSes.

- Within a single OS, there is no trustable unique APP ID, e.g. Android supports many APK stores. In practice, currently most OSes will not allocate a unique APP ID to its own software, but use the “name” of software to differentiate them. For example, in Android, the APP Developer could use “Packet Name” to name their applications and the corresponding Application Store will ensure such Packet Name is unique within the Application Store, but not considering other Application Stores.

Hence, new type of application level descriptor is needed. The new type of allocation level descriptor should be unique, and independent from the OS and/or software repository. Furthermore, the application level descriptor should support the applications both deployed by operators themselves and developed by 3rd parties. So, the application level descriptor should allow both global (operator-independent) and local (operator-specific, i.e. HPLMN of the UE) identities. Global identities would be assigned by a single registrar (TBD, perhaps GSMA) and local identities would be assigned uniquely by the operator of which the UE is subscribed.

4	Objective

The objectives for this work item are:

Define a new type of application level descriptor, the new application level descriptor should be defined with the following: <global/local> <application describing string>

  - where global: defined by a central independent registrar, such as GSMA if they are willing to take the role.

  - where local: defined by the HPLMN of the UE: this value is only valid in conjunction with the PLMN ID (or PLMN ID+NID) of the HPLMN of the UE.

  - application describing string: a string which is unique with respect to the registrar

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.503 | Define a new type of application level descriptor in URSP. A draft CR to TS 23.503 implementing the objective of this WID can be found in S2-2002944. | TSG#89 |  |
| 23.501 | Define a new type of application level descriptor in ATSSS rule. A draft CR to TS 23.503 implementing the objective of this WID can be found in S2-2002945. | TSG#89 |  |

6	Work item Rapporteur(s)

Haiyang Sun, sunhaiyang3@huawei.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

Huawei

Hisilicon