3GPP TSG-WG SA2 Meeting #138e 	S2-202970

# E-Meeting, 20th – 24th April, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0165 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for the Network Performance analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 6.6, first, the definition for load and its relationship with the Network Performance analytics are not clear. It seems that all the Network Performance analytics (such as gNB status information, gNB resource usage, Communication performance and Mobility performance) could be the load information except for the number of UEs. But the "load" could be ambiguous definition compared with the "NF load" as defined in clause 6.5. Second, almost all the analytics parameters in the Network Performance now are not Core Network Function related but RAN related. In fact, the NF load such as UPF load could also be a Network Performance parameter. Furthermore, similar to the Area Subset, if an Analytics target period is provided, the NWDAF could also divided it into one or multiple period subset to help e.g. PCF determine more accurate BDT policy. Last but not least, in addition to the gNB resource usage, others analytics parameters should also be an average value within the Area Subset during the Analytics target period subset. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the "load" in the Network Performance analytics; - Add the "UPF load" and "Analytics target period susbet" as the analytics parameters; - Clarify the alaytics parameters of Network Performance analytics could be an average value; - Some others editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Technically not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.6	Network Performance Analytics

6.6.1	General

With Network Performance Analytics, NWDAF provides either statistics or predictions on the gNB status information, gNB resource usage, UPF load, communication performance and mobility performance in an Area of Interest; in addition, NWDAF it may provide statistics or predictions on the number of UEs that are located in that Area of Interest.

The service consumer may be an NF (e.g. PCF, NEF, AF), or the OAM.

The consumer of these analytics may indicate in the request:

-	Analytics ID set to "Network Performance";

-	Target of Analytics Reporting containing either a UE, or Internal Group Identifier that refers to the group for which the analytics on the number of UEs that are located in the Area of Interest at the time indicated in the Analytics target period is requested or any UE;

-	Analytics Filter Information containing:

-	Optionally, maximum number of objects;

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target Of Event Reporting is set to "any UE", optional otherwise);

-	Optionally, the subset of analytics that are requested among those specified in clause 6.6.3;

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics information (see clause 6.6.3) in order to be notified by the NWDAF;

-	An Analytics target period indicates the time period over which the statistics or prediction are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

6.6.2	Input Data

The NWDAF collects Load and Performance information in an Area of Interest from the sources listed in Table 6.6.2-1 and number of UEs within Area of Interest from the sources listed in Table 6.6.2-2.

Table 6.6.2-1: Load and Performance information collected by NWDAF

| Load information | Source | Description |
| --- | --- | --- |
| Status, load and performance information | OAM | Statistics on RAN status (up/down), load (i.e. Radio Resource Utilization) and performance per Cell Id in the Area of Interest as defined in TS 28.552 [8]. |
| NF Load information | NRF | Load per NF |

Table 6.6.2-2: Number of UEs in Area of Interest information collected by NWDAF

| Number of UEs information | Source | Description |
| --- | --- | --- |
| Number of UEs | AMF | Number of UEs in an Area of Interest |

The NWDAF shall be able to collect UE mobility information as stated in clause 6.7.2.2.

6.6.3	Output Analytics

The NWDAF shall be able to provide both statistics and predictions on Network Performance.

Network performance statistics are defined in Table 6.6.3-1.

Table 6.6.3-1: Network performance statistics

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Observed statistics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area of interest as defined in clause 6.6.1 |
| > Analytics target period subset | Time window within the requested Analytics target period as defined in clause 6.6.1. |
| > gNB status information | Average ratio of gNBs that have been up and running during the entire Analytics target period in the area subset |
| > gNB resource usage | Average usage of assigned resources (CPU, memory, disk) |
| > UPF Load | Average load of UPFs in the Area subset (average, peak) |
| > Number of UEs | Average number of UEs observed in the area subset |
| > Communication performance | Average ratio of successful setup of PDU Sessions |
| > Mobility performance | Average ratio of successful handover |

Network performance predictions are defined in Table 6.6.3-2.

Table 6.6.3-2: Network performance predictions

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Predicted analytics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area of interest as defined in clause 6.6.1 |
| > Analytics target period subset | Time window within the requested Analytics target period as defined in clause 6.6.1. |
| > gNB status information | Average ratio of gNBs that will be up and running during the entire Analytics target period in the area subset |
| > gNB resource usage | Average usage of assigned resources (CPU, memory, disk) (average, peak) |
| > UPF Load | Average load of UPFs in the Area subset (average, peak) |
| > Number of UEs | Average number of UEs predicted in the area subset |
| > Communication performance | Average ratio of successful setup of PDU Sessions |
| > Mobility performance | Average ratio of successful handover |
| > Confidence | Confidence of this prediction |

NOTE 1:	The predictions are provided with a Validity Period, as defined in clause 6.1.3.

NOTE 2:	The analytics on number of UEs are related to the information retrieved from the AMFs.

The number of network performance information entries is limited by the maximum number of objects provided as input parameter.

The NWDAF provides Network Performance Analytics to a consumer at the time requested by the consumer in the Analytics target period:

-	Analytics ID set to "Network Performance".

-	Notification Target Address including the address of the consumer.

-	Notification Correlation Id, for the consumer to correlate notifications from NWDAF if subscription applies.

-	Analytics specific parameters at the time indicated in the Analytics target period.

6.6.4	Procedures

Figure 6.6.4-1: Procedure for subscription to network performance analytics

1.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID="Network Performance", Target of Analytics Reporting, Analytics Filter="Area of Interest", "Reporting Thresholds" and Analytics target Period(s)) to the NWDAF.

2a-2d.	The NWDAF discovers from NRF the AMF(s) belonging to the AMF Region(s) that include(s) the Area of Interest and subscribes to NF load and status information from NRF about these AMF(s).

3a-3b.	The NWDAF subscribes to OAM services to get the status and load information and the resource usage on the Area of Interest in clause 6.6.2, following the procedure captured in Clause 6.2.3.2.

4a-4b.	The NWDAF collects the number of UEs located in the Area of Interest from AMF using Namf_EventExposure_Subscribe service, including the Target of Event Reporting provided as an input parameter (i.e. any UE or Internal Group Identifier).

5.	The NWDAF derives the requested analytics.

6.	The NWDAF sends Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response (Network Performance analytics,  Subscription Correlation Id, Probability of assertion).

7-8. A change of network performance information, i.e. change in the gNB status information, gNB resource usage, UPF load, communication performance and mobility performance in the area of interest at the observed period, is detected by OAM, or a change in the NF load information is reported by NRF, and is notified to NWDAF.

9.	The NWDAF derives new analytics taking into account the most recent data collected.

10.	When relevant according to the Analytics target period and Reporting Thresholds, the NWDAF provides a notification using Nnwdaf_AnalyticsSubscription_Notify (Network Performance analytics, Subscription Correlation Id, Probability of assertion).

# * * * * End of changes * * * *