SA WG2 Meeting #S2-138E	S2-2003500

20 - 24 April, 2020, Electronic, Elbonia	(revision of S2-2003028)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2257 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarification on CN tunnel for EPS during the I-SMF insertion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Huawei, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The I-SMF handling is simliar to V-SMF in home routed roaming. For home routed roaming scenario, if the EBI is assigned successfully, the PGW-C+SMF prepares the CN Tunnel Info for each EPS bearer and provides it to V-SMF. Thus when the UE moves to EPC, the V-SMF does not need to interact with the PGW-C+SMF to get the EPS bearer context(s). (see step 8 in clause 4.11.1.4.1) This also applied to ETSUN case. There is still a scenario, in which how to handle is not clear. In the LBO or non-roaming scenario, there is no I-SMF and the PDU session has been allocated the EBI. It means the PGW-C+SMF does not prepare the CN Tunnel info for each bearer. Later if there is a need to insert an I-SMF, there are several options: - PGW-C+SMF prepares the CN Tunnel Info for each EPS bearer and provides it to I-SMF; - Do nothing in I-SMF insertion. When the next EBI assignment is successfully, the PGW-C+SMF prepares the CN Tunnel Info for each EPS bearer and provide it to I-SMF This CR proposes the option 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | During I-SMF insertion, if the PGW-C+SMF has not prepared the CN Tunnel Info for each EPS bearer, the PGW-C+SMF requests the PGW-U+UPF to allocate the CN Tunnel for each EPS bearer for the PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear, when the I-SMF is inserted, whether the PGW-C+SMF prepares the CN Tunnel Info for each EPS bearer or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.12.4, 4.23.12.x (new), 4.23.12.x.1 (new), 4.23.12.x.2 (new), 4.23.12.x.3 (new), 4.23.12.x.4 (new), 4.23.12.x.5 (new), 4.23.12.x.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

4.23.12.4	Procedures for EPS bearer ID allocation

The EPS Bearer ID allocation procedure, EPS Bearer ID transfer procedure and EPS Bearer ID revocation procedure are perfomed as described for the home-routed roaming case in clause 4.11.1.4, with the following differences:

-	H-SMF is replaced by SMF.

-	V-SMF is replaced by I-SMF.

-	In Step 8 of clause 4.11.1.4.1, in addition to home routing roaming scenario, for scenario of deployments topologies with specific SMF Service Areas, if EBI is assigned successfully, the PGW-C+SMF also prepares CN Tunnel Info for each EPS bearer and provides this information to the I-SMF.

NEXT CHANGE

4.23.12.X	Impact to 5GC procedures

4.23.12.X.1	General

This clause captures impact to 5GC procedures in other subclauses of clause 4.23 to support interworking with N26.

4.23.12.X.2	UE Triggered Service Request with I-SMF insertion/change/removal

The following impact is applicable for UE triggered Service Request with I-SMF insertion in clause 4.23.4.3:

-	In step 20a, when an I-SMF is inserted, if EBI(s) have been allocated before but the PGW-C+SMF has not prepared the CN Tunnel Info for each EPS bearer, the PGW-C+SMF requests the PGW-U+UPF to allocate the CN Tunnel for each EPS bearer for PDU Session(s). PGW-U+UPF allocates the PGW-U tunnel info for the EPS bearer and sends it to the PGW-C+SMF.

-	In step 20c, the PGW-C+SMF provides also CN Tunnel Info for each EPS bearer to the I-SMF.

NOTE:	The CN Tunnel Info for each EPS bearer provided to the I-SMF is to prepare for UE mobility to EPC network so that the I-SMF does not need interact with the PGW-C+SMF to get the EPS bearer context(s) at mobility to EPC.

4.23.12.X.3	PDU Session establishment procedure

The following impact is applicable for PDU Session establishment in clause 4.23.5.1:

-	If I-SMF is involved in the PDU Session establishment, after EBI has been  successfully allocated, the PGW-C+SMF also prepares CN tunnel info for each EPS beare and provides it to the I-SMF r, see clause 4.23.12.4.

4.23.12.X.4	PDU Session modification procedure

The following impact is applicable for PDU Session modification in clause 4.23.5.3:

-	If I-SMF is involved in the PDU Session modification, if EBI needs to be allocated and the allocation is successful, the PGW-C+SMF also prepares CN tunnel info for each EPS bearer and provides it to the I-SMF, see clause 4.23.12.4.

4.23.12.X.5	Inter NG-RAN node N2 based handover with I-SMF insertion/change/removal

The following impact is applicable for Inter NG-RAN node N2 based handover with I-SMF insertion in clause 4.23.7.3.2:

-	In step 7c, at I-SMF insertion, if EBI(s) have been allocated before but the PGW-C+SMF has not prepared the CN Tunnel Info for each EPS bearer, the PGW-C+SMF requests the PGW-U+UPF to allocate the CN Tunnel for each EPS bearer for PDU Session(s). PGW-U+UPF allocates the PGW-U tunnel info for the EPS bearer and sends it to the PGW-C+SMF.

-	In step 7f, the PGW-C+SMF provides also the CN Tunnel Info for each EPS bearer to the I-SMF.

4.23.12.X.6	Xn based handover with insertion of I-SMF

The following impact is applicable Xn based handover with insertion of I-SMF in clause 4.23.11.2:

-	In step 6, at I-SMF insertion, if EBI(s) have been allocated before but the PGW-C+SMF has not prepared the CN Tunnel Info for each EPS bearer, the PGW-C+SMF requests the PGW-U+UPF to allocate the CN Tunnel for each EPS bearer for PDU Session(s). PGW-U+UPF allocates the PGW-U tunnel info for the EPS bearer and sends it to the PGW-C+SMF.

-	In step 9, the PGW-C+SMF provides also the CN Tunnel Info for each EPS bearer to the I-SMF.

END OF CHANGES