3GPP TSG-SA2 Meeting #138-E	S2-2002733

# Online, 20th Apr 2020 - 23rd Apr 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0145 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of OAM as the NWDAF service consumer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As description in Clause 5，the NWDAF provides analytics to 5GC NFs, and OAM. So, the OAM shall be added to the Table 7.1-1 as the NWDAF service consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The OAM is added to the Table 7.1-1 as the NWDAF service consumer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description about NWDAF service consumer is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

7.1	General

The following table illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM |

The following table shows the analytics information provided by NWDAF service:

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level of a Network Slice Instance reported either as notification of crossing of a given threshold or as periodic notification (if no threshold is provided). |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice instance or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice instance. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |

**** End of Change ****