SA WG2 Meeting #S2-138E	S2-2002925

20 - 24 April, 2020, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2289 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on QoS support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2020-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. The SMF only needs to provide the same QFI in both 3GPP and non-3GPP accesses for a Non-GBR QoS Flow, for the GBR QoS flow is only transmitted over one access. 2. When dynamic PCC is not deployed, the SMF also needs to decide to which access network to provide the QoS profile for the GBR QoS Flow based on its local policy. 3. The SMF may decide to move the GBR QoS Flow to the other access based on the report from the AMF (e.g. UE reachability report), besides the report from the UPF. 4. When moving the GBR QoS Flow to the other access, the SMF needs to update the ATSSS rules to the UE and the N4 rules to the UPF. Currently only updating the ATSSS rules to the UE is mentioned in a NOTE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify that the SMF shall provide the same QFI in 3GPP and non-3GPP accesses only for a Non-GBR QoS Flow, so that the same QoS is supported for the Non-GBR QoS Flow in both accesses. 2. Clarify that when dynamic PCC is not deployed, the SMF also needs to decide to which access network to provide the QoS profile for the GBR QoS Flow based on its local policy. 3. Add that the SMF may decide to move the GBR QoS Flow to the other access based on the report from the AMF. 4. Clarify in normative text that when moving the GBR QoS Flow to the other access, the SMF needs to provide the updated ATSSS rules and N4 rules to the UE and the UPF respectively. And modify the NOTE 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear / incomplete specifications on QoS support for Non-GBR and GBR QoS flows of the MA PDU session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.4 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

5.32.4	QoS Support

The 5G QoS model for the Single-Access PDU Session is also applied to the MA PDU Session, i.e. the QoS Flow is the finest granularity of QoS differentiation in the MA PDU Session. One difference compared to the Single-Access PDU Session is that in a MA PDU Session there can be separate user-plane tunnels between the AN and the PSA, each one associated with a different access. However, the QoS Flow is not associated with specific access, i.e. it is access agnostic, so the same QoS is supported when the traffic is distributed over 3GPP and non-3GPP accesses. The SMF shall provide the same QFI in 3GPP and non-3GPP accesses for a Non-GBR QoS Flow so that the same QoS is supported for the Non-GBR QoS Flow in both accesses.

A QoS Flow of the MA PDU Session may be either Non-GBR or GBR depending on its QoS profile.

For a Non-GBR QoS Flow, the SMF provides a QoS profile to 5G-AN(s) during MA PDU Session Establishment or MA PDU Session Modification procedure:

-	During MA PDU Session Establishment procedure, the QoS profile to both ANs if the UE is registered over both accesses.

-	During MA PDU Session Modification procedure, the QoS profile is provided to the 5G-AN(s) over which the user plane resources are activated.

For a GBR QoS Flow, the SMF shall provide a QoS profile to a single access network as follows:

-	If the PCC rule allows a GBR QoS Flow in a single access, the SMF provides the QoS profile for the GBR QoS Flow to the access network allowed by the PCC rule.

-	If the PCC rule allows a GBR QoS Flow in both accesses or dynamic PCC is not deployed, the SMF decides to which access network to provide the QoS profile for the GBR QoS Flow based on its local policy (e.g. the access where the traffic is ongoing according to the Multi Access Routing rules).

For a GBR QoS Flow, traffic splitting is not supported because the QoS profile is provided to a single access network at a given time. If the UPF determines that it cannot send GBR traffic over the current ongoing access e.g. based on the N4 rules and access availability and unavailability report from the UE as described in clause 5.32.5.3, the UPF shall send an Access Availability report to the SMF. If the AMF determines that the UE is unreachable over one access (e.g. no Paging response from the UE, the UE is in CM-IDLE on non-3GPP access), the AMF notifies the SMF if the SMF has subscribed to the event of UE reachability. Based on the report from the UPF or the AMF, the SMF decides whether to move GBR QoS Flows to the other access:

-	if the PCC rule allows the GBR QoS Flows only on this access, the SMF shall release the resources for the GBR QoS Flow and report to the PCF about the removal of the PCC rule.

-	if the corresponding PCC rule allows the GBR QoS Flow on both accesses and the other access is not available, the SMF shall release the resources for the GBR QoS Flow and report to the PCF about the removal of the PCC rule.

-	if the PCC rule allows the GBR QoS Flow on both accesses and the other access is available, the SMF shall try to move the GBR QoS Flow to the other access. The SMF may trigger a PDU session modification procedure to provide the QoS profile to the other access and release the resources for the GBR QoS Flow in the current access, and to provide the updated ATSSS rules and N4 rules to the UE and the UPF respectively.

-	If Notification Control parameter is not included in the PCC rule for the GBR QoS Flow and the other access does not accept the QoS profile, the SMF shall release the resources for the GBR QoS Flow and report to the PCF about the removal of the PCC rule.

-	if the Notification Control parameter is included in the PCC rule, the SMF shall notify the PCF that GFBR can no longer be guaranteed. After the other access accepts the QoS profile, the SMF shall notify the PCF that GFBR can again be guaranteed. If the other access does not accept the QoS profile, the SMF shall delete the GBR QoS Flow and report to the PCF about the removal of the PCC rule.

NOTE 1:	The SMF provides the ATSSS rules and N4 rules in a way that  a GBR QoS Flow is only allowed to be transmitted over a single access.

When the MA PDU Session is established or when the MA PDU Session is modified, the SMF may provide QoS rule(s) to the UE via one access, which are applied by the UE as specified in clause 5.7.1.4. The QoS rule(s) provided by SMF via one access are commonly used for both 3GPP access and non-3GPP access, so the QoS classification is independent of ATSSS rules.

The derived QoS rule generated by Reflective QoS is applied independently of the access on which the RQI was received. When the MPTCP functionality is used in the UE, the UE shall use the IP address/prefix of the MA PDU Session and the final destination address to generate the derived QoS rule.

When MPTCP functionality is enabled for the MA PDU Session:

-	any QoS rules or PDRs that apply to the MA PDU Session IP address/prefix and port also apply to the "link-specific multipath" addresses/prefixes and ports used by the UE to establish MPTCP subflows over 3GPP and non-3GPP accesses; and

-	any QoS rules or PDRs that apply to the IP address/prefix and port of the final destination server in DN also apply to the IP address and port of the MPTCP proxy for corresponding MPTCP subflows that are terminated at the proxy.

NOTE 2:	How these associations are made is left up to the UE and UPF implementations.

******************** End of Change ********************