SA WG2 Meeting #138E	S2-2003047

# April 20 - 24, 2020, Elbonia  														(revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0172 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Nnwdaf service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are a couple of parameters missing from the NWDAF service operations specification in clause 7: - Reporting Thresholds, Maximum number of objects and Maximum number of SUPIs can be used by the analytics consumer as inputs for Nnwdaf_AnalyticsSubscription_Subscribe and can be provided per individual Analytics ID. - Maximum number of objects and Maximum number of SUPIs can be used by the analytics consumer as inputs for Nnwdaf_AnalyticsInfo_Request and can be provided per individual Analytics ID. - Validity period missing from Nnwdaf_AnalyticsSubscription_Notify and as output for Nnwdaf_AnalyticsInfo_Request. Nnwdaf_AnalyticsSubscription_Notify and output for Nnwdaf_AnalyticsInfo_Request refer to "Timestamp for the analytics information" while clause 6.1.3 refers to "timestamp of analytics generation". The notes added via agreed CR in S2-2001989 (SA2#137E) should also apply to Nnwdaf_AnalyticsInfo_Request, i.e. Target of Analytics Reporting can be provided per individual Analytics ID in a set of Analytics IDs, and Analytics Filter Information can be provided per individual Analytics ID in a set of Analytics IDs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reporting Thresholds, Maximum number of objects and Maximum number of SUPIs added as inputs for Nnwdaf_AnalyticsSubscription_Subscribe and a note clarifies that these can be provided per individual Analytics ID. - Maximum number of objects and Maximum number of SUPIs added as inputs for Nnwdaf_AnalyticsInfo_Request and a note clarifies that these can be provided per individual Analytics ID. - Validity period added as input to Nnwdaf_AnalyticsSubscription_Notify and as output for Nnwdaf_AnalyticsInfo_Request, with a note clarifying that Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics. - Notes added to Nnwdaf_AnalyticsInfo_Request description to clarify that Target of Analytics Reporting can be provided per individual Analytics ID in a set of Analytics IDs, and Analytics Filter Information can be provided per individual Analytics ID in a set of Analytics IDs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete service operations specification leading to issues at stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2, 7.2.4, 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (1)

### 7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Subscribe.

Description: Subscribes to NWDAF analytics with specific parameters.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2, Target of Analytics Reporting, Notification Target Address (+ Notification Correlation ID), Analytics Reporting Parameters, Analytics target period.

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID in a set of Analytics IDs.

Inputs, Optional: Analytics Filter Information, Subscription Correlation ID (in the case of modification of the analytics subscription), preferred level of accuracy of the analytics, Reporting Thresholds, Maximum number of objects, Maximum number of SUPIs.

NOTE 2:	Analytics Filter Information, Reporting Thresholds, Maximum number of objects and Maximum number of SUPIs can be provided per individual Analytics ID in a set of Analytics IDs.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription).

Outputs, Optional: None.

NEXT CHANGE (2)

### 7.2.4	Nnwdaf_AnalyticsSubscription_Notify service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Notify.

Description: NWDAF notifies the consumer instance of the analytics that has subscribed to the specific NWDAF service.

Inputs, Required: Set of the tuple (Analytics ID, Analytics specific parameters), Notification Correlation Information.

Inputs, Optional: Timestamp of analytics generation, validity period, probability assertion.

NOTE 1:	Some NWDAF output analytics already include confidence of predictions, which provides the same information as probability assertion.

NOTE 2:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

NEXT CHANGE (3)

### 7.3.2	Nnwdaf_AnalyticsInfo_Request service operation

Service operation name: Nnwdaf_AnalyticsInfo_Request.

Description: the consumer requests NWDAF operator specific analytics.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2, Target of Analytics Reporting, Analytics target period.

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID in a set of Analytics IDs.

Inputs, Optional: Analytics Filter Information, preferred level of accuracy of the analytics, time when analytics information is needed, Maximum number of objects, Maximum number of SUPIs.

NOTE 2:	Analytics Filter Information, Maximum number of objects and Maximum number of SUPIs can be provided per individual Analytics ID in a set of Analytics IDs.

Outputs, Required: Set of the tuple (Analytics ID, Analytics specific parameters).

Outputs, Optional: Timestamp of analytics generation, validity period, probability assertion.

NOTE 3:	Some NWDAF output analytics already include confidence of predictions, which provides the same information as probability assertion.

NOTE 4:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

END OF CHANGES