SA WG2 Meeting #139E	S2-2003733

Elbonia, June 1- 12, 2020	(revision of S2-200xxxx)

Source:	Apple

Title:	KI #7, New Sol: AMF assistance to NG-RAN during PDU Session establishment procedure

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes a solution for 5GC assisted cell selection to access network slice. The solution assumes that Allowed S-NSSAI consists of slices which cannot operate on the same frequency band. The solution also assumes that NSSF maintains the mapping between the S-NSSAI and the allowed frequency bands to operate.

# 1. Introduction

As highlighted in this key issue, an operator may only support certain network slices on certain operating frequency bands. One registration area may consist of multiple NG-RAN cells belonging to different operating bands, and thus it is possible that support for a particular S-NSSAI in a registration area is non-homogenous.

In this paper, we propose AMF shall provide assistance to NG-RAN about the S-NSSAI and the operating frequency band(s) in which the slice is allowed to operate as part of the PDU session establishment procedure. NG-RAN may use this assistance to steer the UE to the appropriate operating frequency band to access the slice.

* * * * Start of Changes * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution#'s | Solution Titles | Key Issue#'s |
| --- | --- | --- |
| 1 | PCF measurement based Network Slice SLA control for Maximum Number of UEs parameter | 1 |
| 2 | Max number of UEs per Network Slice control at registration | 1 |
| 3 | AMF/NSSF based counting of UEs in a Network Slice | 1 |
| 4 | NWDAF enhancements for supporting of network slice quota on the maximum number of UEs | 1 |
| 5 | NWDAF enhancements for supporting of network slice quota on the maximum number of PDU Sessions | 2 |
| 6 | PCF-based counting of PDU Sessions in a Network Slice | 2 |
| 7 | Support of Network Slice SLA for Maximum Number of PDU sessions parameter | 2 |
| 8 | AMF and O&M based solution | 1, 2 & 4 |
| 9 | Monitoring multiple quotas of number of UEs/PDU Sessions per S-NSSAI at NWDAF | 1, 2 & 4 |
| 10 | Max number of PDU Sessions per Network Slice control via NSQ function | 2 |
| x | AMF assistance to NG-RAN during PDU Session establishment procedure | 7 |

NEXT CHANGE (All text is new)

## 6.X	Solution #x: AMF assistance to NG-RAN during PDU Session establishment procedure

### 6.X.1	Introduction

This solution is for Key Issue #7, "Support of 5GC assisted cell selection to access network slice". The solution is based on the following architectural assumptions:

-	The UE may simultaneously register to slices that are not all accessible on the same operating band(s). In other words, the UE may have S-NSSAIs in the Allowed NSSAI that are not all available in a common operating band.

-	The UE may simultaneously establish and access PDU Sessions on slices that are not all supported on the same operating band.

-	NSSF maintains the mapping between S-NSSAI and allowed frequency bands to operate.

-	AMF caches the information of the mapping between S-NSSAI and allowed frequency bands to operate.

### 6.X.2	High Level Description

As part of the PDU Session establishment procedure, AMF provides to NG-RAN the allowed frequency band to operate for the S-NSSAI in this PDU Session, as part of the PDU Resource Setup Request.

Based on AMF’s assistance, NG-RAN informs the UE to use a specific frequency band when sending and receiving user plane packets for the particular S-NSSAI.

Editor’s Note: How the NG-RAN steers the UE (e.g. handover, configuring dual connectivity to simultaneously access two operating bands, etc.) should be defined in RAN WGs.

### 6.X.3	Procedures

The procedure in Figure 6.X.3-1 shows a signalling flow in which the AMF provides the "Allowed list of operating bands" to the NG-RAN as part of PDU Session Resource Setup Request.

Figure 6.X.3.1 – Providing "Allowed list of operating bands" to NG-RAN as part of PDU Session Resource Setup Request

0.	AMF has a copy of the mapping table (from NSSF) of S-NSSAI(s) and the allowed list of frequency bands to operate.

1.	UE sends PDU Establishment Request for the S-NSSAI.

NOTE:	Based on the S-NSSAI included by the UE in RRCConnectionSetup, NG-RAN may steer the UE to access the new operating band which supports the S-NSSAI user is interested in accessing.

Editor’s Note: How the NG-RAN steers the UE (e.g. handover) should be defined in RAN WGs.

2.	PDU Session establishment procedure continues as defined in TS 23.502 [6], clause 4.3.2.2.1, steps 2-11.

3.	AMF provides NG-RAN the following parameters required for the PDU Session via PDU Session Resource Setup Request: PDU Session ID, CN Tunnel Info, QoS profile with QFI, S-NSSAI, Allowed list of operating bands, NAS PDU, UL NG-U UP TNL Information.

4.	NG-RAN responds with PDU Session Resource Setup Response that it is able to support the requested configuration.

5.	NG-RAN sends RRC Reconfiguration message to inform the UE to appropriately steer traffic for the S-NSSAI via the Allowed list of frequency band.

Editor’s Note: How the NG-RAN steers the UE (e.g. handover, configuring dual connectivity to simultaneously access two operating bands, etc.) should be defined in RAN WGs.

6.	AMF forwards the PDU Session Establishment Accept to UE.

### 6.X.4	Impacts on existing entities and interfaces

-	NSSF shall be able to provide the supported operating frequency band(s) for each allowed S-NSSAI(s) to the AMF.

- 	AMF shall be able to provide supported operating frequency band(s) for S-NSSAI(s) to the NG-RAN as part of  PDU Session Resource Setup Request.

- 	NG-RAN shall be able to steer the UE to appropriate operating frequency band which supports the S-NSSAI included in the RRC connection procedure.

### 6.X.5 	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * * End of Changes * * * *