3GPP TSG-WG SA2 Meeting #139E e-meeting 	S2-2003872

Elbonia, June 1 – 12, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#12, New Sol: Congestion Info as input for RFSP generation

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution introduces a solution for Key Issue #12.

1. Introduction/Discussion

This solution is proposed to address key issue #12.

The RFSP mechanism has been supported in the current specification. It can be beneficial for the PCF to derive suitable RFSP index based on the congestion information (e.g. the slice is in Severe Congestion/General Congestion/no Congestion) of the slice provided by NWDAF. For instance, the PCF could generate a RFSP index, which shows the frequency corresponding to the congested slice as low priority. The User Data Congestion Analytics could also obtain congestion analytics on a given slice as described in clause 6.8.1 of TS 23.288 [5]. Hence, it is proposed to reuse User Data Congestion Analytics as input for RFSP index generation.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-91.

# * * * * First change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 11 |  |  |  |
| 2 | 4 |  |  |  |
| 3 | 15 |  |  |  |
| 4 | 12 |  |  |  |
| x | 12 |  |  |  |

# * * * * Second change * * * *

6.X	Solution for Key Issue 12:  User Data Congestion Analytics for NWDAF-assisted RFSP policy

6.X.1	Description

This solution is proposed to address Key Issue #12: NWDAF-assisted RFSP policy.

This solution specifies for how NWDAF can provide User Data Congestion Analytics as defined in clause 6.8 of TS 23.288 [5] to a service consumer to derive a suitable RFSP index. The Analytics Filter Information could contain S-NSSAI.

6.X.2	Input Data

The same as clause 6.8.2 of TS 23.288 [5].

6.X.3	Output Analytics

The same as clause 6.8.3 of TS 23.288 [5].

6.X.4	Procedures

The same as clause 6.8.4 of TS 23.288 [5].

6.X.5	Impacts on services, entities and interfaces

PCF:

-	Using the output of User Data Congestion Analytics of NWDAF to make decision on the RSFP index.

# * * * * End of changes * * * *