SA WG2 Meeting #139E	S2-2004049

June 1 – 12, 2020	(revision of S2-20nnnnn)

Source:	MediaTek Inc.

Title:	FS_MUSIM: KI #1, Sol #8: Update to clarify UE impact

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_MUSIM / Rel-17

Abstract: This contribution clarifies UE impact for Solution#8.

# 1 	Proposal

Solution #8 “MT Notification via N3IWF” is defined in TR23.761 §6.8. The documented UE impact are very slim and inaccurate.

# pCR 23.761 (all new text)

## 6.8	Solution #8: MT Service Notification through N3IWF

### 6.8.4	Impacts on services, entities and interfaces

SMF:

-	Support of MT data handling info from AMF, and

-	decides how to handle the PDU session after receiving the MT data handling info, based on release assistance information from UE, in N11 message.

-	initiates MT data handling instruction to UPF

-	receiving return indication and initiate normal MT data handling instruction to UPF

AMF:

-	Handles the NAS procedures from N3IWF and mark the UE/PDU sessions for future MT service notification delivery.

-	forwards MT data handling info, based on the release assistance info from UE, to SMF.

-	using NAS notification instead of paging based on previous marking in case of MT service delivery.

UE:

-	Deactivation of paging reception in idle mode in a PLMN and support N3IWF access to a PLMN via 3GPP access of another PLMN and related procedures for NAS notification

-	Supports indication of MUSIM MT assistance info to the network (i.e. activation) and release of such indication (i.e. deactivation).