SA WG2 Meeting #139E	S2-2004150

1st June – 12th June, 2020

Source:	Samsung

Title:	KI #6, New Sol: UAV-UAVC association control

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS/ Rel-17

Abstract of the contribution: This contribution proposes a new solution for controlling UAV and UAV controller association.

- Discussion

A UAV UE and a UAV controller UE are new types of UEs that request support of additional functionalities for a specialized service.  One of the requirements is controlling the connectivity between UAV and UAV controller based on their association status. While the basic paring procedures between UAV and UAV controller may be performed using on-site direct communication, their mutual association for UAS service needs to be authorized by the USS/UTM to enforce the permitted usage and related regulatory requirements.

A UAV UE or a UAV controller UE may communicate with the USS/UTM to request authorization of its association with the corresponding party, using 3GPP or non3GPP communication. Further, it can be assumed that the authorization of association may be performed during the UAS registration of the UAV UE or a UAV controller UE, or using a separate procedure defined for the purpose. The registration and association status of the peer UAV or UAV controller UE is crucial in determining initiation of PDU session establishment procedure at the UAV (controller) UE as well as authorizing the requested PDU session at the network. That is, if the UAV UE starts PDU session establishment procedure for communication with a paired UAV controller that is not yet registered or associated, the procedure should not be allowed by the network. Otherwise, the resources for the network and the UE will be wasted.

This contribution proposes a solution for controlling UAV and UAV controller connectivity based on the UAV and UAV  controller association status.

2.	Proposal

It is proposed to adopt the following changes in the TR 23.754.

*** Start of the change (all new text) ***

6.X	Solution X: UAV-UAVC Association and Connectivity Control

6.X.1	Introduction

This solution is for Key Issue #6: UAV controller and UAV association.

One of the UAS requirements is controlling the connectivity between UAV and UAV controller based on their association status. While the basic paring procedures between UAV and UAV controller may be performed using, e. g., on-site direct communication, their mutual association for UAS service needs to be authorized by the USS/UTM to enforce the permitted usage and related regulatory requirements.

A UAV UE or a UAV controller UE may communicate with the USS/UTM to request authorization of its association with the corresponding party, using 3GPP or non3GPP communication. Further, it can be assumed that the authorization of association may be performed during the UAS registration of the UAV UE or a UAV controller UE, or using a separate procedure defined for the purpose. The registration and association status of the peer UAV or UAV controller UE is used in determining initiation of PDU session establishment procedure at the UAV (controller) UE as well as authorizing the requested PDU session at the network. That is, if the UAV UE starts PDU session establishment procedure for communication with a paired UAV controller that is not yet registered or associated, the PDU session should not be allowed by the network. Otherwise, the resources for the network and the UE will be wasted. When the UAV or UAV controller is registered and authorized for the association by the UTM, the counter party and the 5G network needs to be informed appropriately.

6.X.1	Functional Description

In the solution, a UAV and a UAV controller UE performs UAS registration in the USS/UTM to get authorized for UAS service. The UAS registration procedure may be performed before UE’s registration with the 3GPP network using a non3GPP networks connectivity. In the case, the UAV and the UAV controller are required to perform re-registration with the USS/UTM to notify the change of network connectivity when they complete initial registration in the 3GPP network.

The UAV and the UAV controller UE provide information on its desired counter party UEs to the USS/UTM during the UAS registration.

A UAV or a UAV controller initiates a PDU session establishment with DN authorization towards the USS/UTM to request authorization of association with a counter party UE selected from the received list.

The USS/UTM evaluates and sends the allowed list of counter party UEs (among the available UEs) to the UAV and the UAV controller UE.

The USS/UTM informs the counter party UE of the association request and triggers the PDU session establishment by the counter party UE.

The USS/UTM delivers the IP addresses of the UAV and the UAV controller UE to the 3GPP network to request update of the PDU sessions.

The USS/UTM provides an activation signalling for UAS communication to the UAV and the UAV controller UE when it receives completion of resource setup from the network or the two UEs.

6.x.3	Procedures

Figure 6.x.3-1: UAV-UAV controller association and C2 communication setup

- A UAV UE performs initial registration in the 3GPP network and establish a PDU session for communication between the UAV and the USS/UTM.

- The UAV UE performs UAS Registration with USS/UTM using application layer signalling to get authentication and authorization for the UAS service. If the UAV UE has already registered at the USS/UTM it may skip authentication and authorization but just include its updated network connectivity information. The UAV may also include information on its counter party (UAV controller) UEs in the Registration signalling message, and receives information on allowed counter party UEs from the USS/UTM. The USS/UTM selects the registered UEs among the requested counter party UEs, and decides the allowed counter party UEs from the list of the registered UEs

3-4. The UAV controller performs the same procedures as described in step 1-2 for the UAV UE.

- The  UAV controller UE initiates a PDU Session Establishment with DN authorization procedure to request USS/UTM authorization on association with the UAV UE.

- The UAV controller provides the UAV UE ID to the USS/UTM during the authorization procedure. The authentication/authorization procedure may be simplified just to handle association authorization. Upon successful authorization, the USS/UTM provides the counter party UE ID to the network for use in connectivity control.

- The UAV receives a PDU Session Establishment Response message from the 3GPP network for communication  with the selected counter party UE.

- The 3GPP network provides the IP address of the UAV UE to the USS/UTM.

- The USS/UTM sends an application signalling to the counter party UE to trigger the PDU Session Establishment. The message includes the initiating UAV controller UE ID.

- Same DN authorization procedure as for step 5-8 is performed for the UAV UE. The USS/UTM receives the IP address of the UAV UE from the 3GPP network.

11-12. The USS/UTM sends a UAS Association Update message to the 3GPP network to delivered the IP
addresses of the UAV UE and the UAV controller UE.

13-14. The 3GPP network performs the PDU session modification procedure to update packet filter information for 
UAS communication.

- The UAV and the UAV controller UE send the USS/UTM a notification of the completion of communication paths for UAS communication.

- The USS/UTM sends the activation signalling for the UAS communication to the UAV and the UAV controller UE, after receiving the notification messages from the two UEs.

6.x.4	Impacts on existing entities and interfaces

Editor's note:	This clause describes impacts on existing entities and interfaces.

6.X.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

*** End of the change ***