3GPP TSG-SA WG2 Meeting #139E	S2-2004269

# Electronic meeting, 1-12 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.737 | CR | 0017 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI #X, #1, #2, #10, New Sol: Solution for Key Issue for Minimization of 5GCN Impacts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2020-06-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Minimizing 5GCN impacts requires the RAN side to emulate terrestrial 5G NR access as far as possible and hide satellite specific aspects as far as possible from a 5GCN. One key part of this would be enabling fixed TAs and fixed cells from the perspective of a 5GCN. There is already a solution for this in TR 23.737 (Solution #12) but this solution is restrictive and does not fully ellaborate on impacts it would have to a UE, RAN and 5GCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new solution is added based in part on Solution #12 but with more flexibility and less 5GCN, RAN and UE impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Minimizing 5GCN impact will be more difficult or not possible to achieve. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * * all new text

## 6.A	Solution #A: Emulating Terrestrial 5G access to a 5GCN

### 6.A.1	Description

### 6.A.1.1	Architecture

This is a candidate solution for Key Issue #X and, as consequence of their being subsets of KI #X, also of Key Issues #1, #2 and #10. It is noted that the term “satellite Node B” (sNB) is used here to refer to a gNB that has been enhanced to support 5G NR satellite access.

The solution ensures that on the 5GCN side, a 5G satellite RAN can be treated almost the same as a terrestrial NG-RAN providing a new RAT or new set of RATs, e.g. as for Solution #2. This is achieved, in part, by using fixed TAs and fixed “virtual” cells, e.g. as for Solution #12, which allows a 5G satellite NG-RAN to appear, from a 5GCN perspective, to provide access to UEs located in normal terrestrial TAs and cells and hides details of moving NGSO SVs and moving radio cells from the 5GCN.

It is noted that certain parts of this solution depend on support by 3GPP RAN.

The solution incorporates certain previously defined options and solutions while excluding others. As an example, transparent SV mode and regenerative SV mode with a split architecture are included (or allowed) while regenerative SV mode with a non-split architecture is excluded because it would require significant 5GCN impact to manage access to moving NGSO satellites from the 5GCN. Table 6.A.1-1 shows the previously defined options and solutions which are included versus excluded.

Table 6.A.1-1: Previous Options and Solutions Included versus Excluded for Solution #A

| Tyoe of Solution or Option | Included Options and Solutions | Excluded Options and Solutions |
| --- | --- | --- |
| RAN Architecture | Transparent SVs with fixed sNBs e.g. as in Figures 6.12.1-1 and B.2-2 and Figures 5.1-1, 5.1-2, 5.1-3 and 5.1-4 in TR 38.821 [7]. Regenerative SVs with fixed sNB-CUs, e.g. as in Figure B.2-3 and Figures 5.2.2-1, 5.2.2-2, 5.2.2-3 and 5.2.2-4 in TR 38.821 [7]. | Regenerative SVs with sNBs entirely within SVs, e.g. as in Figure B.2-4 and Figures 5.2.1-1, 5.2.1-2, 5.2.1-3, 5.2.1-4 and 5.2.1-5 in TR 38.821 [7]. |
| Cells and TAs | Fixed TAs and Fixed Cells as in Solution #12 from a 5GCN perspective. Mobile radio cells from a RAN perspective. | Non-fixed TAs and radio cells from a 5GCN perspective. |

### 6.A.1.2	Fixed TAs and Fixed Cells

A solution based on Solution #12 is proposed with some differences.

Fixed TAs may be defined using either a regular array of rectangular based or hexagonal based grid points as described for Figures 6.12.1.1-1 and 6.12.1.1-2, where the TAs have the rectangular or hexagonal shapes thereby produced. Alternatively, fixed TAs may be define as polygons (by providing the coordinates of the vertices) as implied by the Geographical Zones approach for Solution #12 in clause 6.12.1.2. The grid point approach can consume minimal data and signalling because the regularity of the arrays only requires definition of a few parameters (e.g. inter-grid point spacings and orientation), but the polygon approach is more flexible and can exactly align a TA boundary with the border of a country or some other natural or artificial boundary such as a coastline, bank of a river or perimeter of a building or campus.

For fixed cells, only a regular array or rectangular based or hexagonal based grid points as described for Figures 6.12.1.1-1 and 6.12.1.1-2 is included due to the savings in data and signaling. While a more flexible polygon approach could be used to define fixed cells, there is no advantage because cells can also be constrained to lie within fixed TAs (which can be aligned with national borders and other lines as just described) and the additional data to define cell vertices would hugely increase complexity including processing and signalling resources needed for support. For eaxmple, if the number of TAs in a network is 100 times the number of cells, defining the TAs as polygons but not the cells could reduce data storage and signalling by almost 100 times (e.g. if a common grid point based array of cells is applied to all TAs as described below).

A third difference to Solution #12 is that definitions of fixed cells and fixed TAs are allowed to be independent of one another. A consequence is that a fixed cell may not wholly lie inside just one fixed TA but may overlap with two or more TAs. This independence can make definition much simpler. For example, if a fixed TA is defined as an irregular polygon, attempting to define a large number of cells that must all be wholly included within the TA could be extremely complex. But if cells are defined separately from TAs, the complexity disappears. Although this means that some cells will not each be part of just one TA, this may not matter since global cell IDs do contain a TAI and are thus not required to identify a unique TA. Instead, cell IDs and TAIs may continue to be used for their respective purposes without mutual conflict (e.g. to support paging and mobility in the case of TAIs and regulatory services and approximate location in the case of cell IDs).

However, if it is preferred to define and identify cells which are each wholly within a single TA, the independent definition of cells and TAs just described can be slightly extended. This can be done by defining cell IDs as having two components – a  base ID corresponding to an initial independent definition of a “base cell” (e.g. a rectangle or hexagon for a regular array of grid points) and a colour code which is appended to the base ID and corresponds to a TA within which a portion of the base cell is included. This is illustrated in Figure 6.A.1.2-1 for a rectangular base cell which overlaps with 3 separate TAs. The base cell is split into 3 separate cell portions which can each be assigned a unique ID using the colour codes of the TAs. These 3 cell portions are then treated as 3 separate fixed cells. In a general case, the serving cell ID for any UE would then be determined from both the base cell (e.g. the rectangular cell shown in Figure 6.A.1.2-1) and the TA (e.g. TA 1, 2 or 3 in in Figure 6.A.1.2-1) within which the UE is located. The colour codes would ensure unique cell IDs as long as no two TAs which share part of the same boundary or the same vertex have the same colour code and as long as base cells are small enough to overlap with no more than one common vertex for the TAs. If there is a further (optional) restriction that no more than 3 TAs share a common vertex (which means in general that the 3 TAs will also share part of a common boundary), then the requirement of having different colour codes becomes equivalent to colouring a map with no two adjacent countries sharing the same colour. This corresponds to the well known Four Colour Theorem which states that four colours are enough. Hence, with these restrictions, four TA colour codes (requiring just 2 bits) would be enough to generate unique cell IDs. (This is also incidentally the reason for choosing the term “colour code”.) In this case, base cell IDs (e.g. the ”cccccc” in Figure 6.A.1.2-1) can comprise 34 bits – leading to normal 36 bit cell IDs as used for 5G NR when extended with the 2 bit colour code.

Figure 6.A.1.2-1: Creation of Unique Cells and Unique Cell IDs with independent Fixed TAs and Cells

### 6.A.1.3	RAN support of fixed TAs and fixed Cells

Mainly to support paging of UEs in the same was as for terrestrial NR access, each sNB (with transparent mode) or sNB-CU (with regenerative mode with split architecture) needs to have a defined and well known service area comprising one or more fixed TAs. This does not, however, preclude supporting the same TA by two or more sNBs or two or more sNB-CUs or having some variation in the current radio coverage area of an sNB due to variation in the radio beams coverage of SVs controlled by the sNB. But it does generally preclude sNBs from shifting support between different TAs at different times.

With a well defined association between fixed TAs and sNBs, a serving AMF can direct a paging request for a UE only to those sNBs supporting the current TA for the UE just as for paging of a UE from certain gNBs in the case of terrestrial NR access.

sNBs (or sNB-CUs) would also broadcast, in a SIB for each supported radio cell, the TA(s) currently supported by that radio cell. This may assist a UE in determining its current TA as well indicating to a UE whether a registration is needed for a change of TA.

The support of fixed TAs by sNBs or sNB-CUs means that at least fixed TA areas must be known by sNBs or sNB-CUs in order to determine the TA(s) supported by each radio cell and to maintain radio cell coverage within the supported TA(s) only.

### 6.A.1.4	UE and 5GCN support of fixed TAs and fixed Cells

A serving AMF provides a geographic definition of the fixed TAs allowed for a UE and associated fixed cells as part of Registration. This data can be pre-configured in an AMF and does not need to be interpreted or processed – making support quite simple.

A UE with a location capability can periodically determine its current fixed TA and current fixed serving cell. The current fixed TA can be used to determine when a new registration is needed – e.g. if a UE moves outside of its allowed set of TAs. The current fixed serving cell can be used to support regulatory services as described for Solution #12 and can also be included for mobile originating services where defined in order to provide location information to the network (e.g. 5GCN and/or RAN).

For a UE without a location capability, an sNB or sNB-CU can determine a current fixed TA and possibly a current fixed cell from the current radio cell and radio beam used by a UE. This may be enough to support UE access though regulatory services may be provided less precisely.

* * * * End of Changes * * * *