SA WG2 Meeting #139E e-meeting	S2-2004500

June 1 – 12, 2020, Elbonia	(revision of S2-2004324)

Source:	vivo, Tencent

Title:	KI #6, Sol #7: Update to clarify ENs

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: Update to clarify ENs for solution 7 of local multicast service discovery.

1. Introduction

It is proposed to update solution 7 as follow to clarify all ENs:

1. Modify the figure to align with both architecture alternatives, which resolves EN in 6.7.1, the 2nd and 3rd ENs in 6.7.2.

2. Clarify only registered UE instead of all UE will be updated of the service parameters, which resolves the 1st EN in 6.7.2.

3. Modify that the registration to the Content Provider has two option: application layer option and network layer option, to make the solution more flexible.

4. Split step 6 into two steps, one for TAI/cell id detection based on SIB1, the other for TMGI detection based on SIB other, which resolves 4th EN in 6.7.2.

5. Clarify that cell ID can be used if using TAI to define the service boundary impacts the TA planning, which resolves the last EN in 6.7.2.

2. Proposal

It is proposed to accept the following update to solution 7 in TR 23.757.

FIRST CHANGE

## 6.7	Solution #7: Local multicast service discovery and provisioning

### 6.7.1	Functional description

This solution addresses KI#6 "Local MBS service".

This solution proposes local MBS service discovery and provisioning procedures. It is proposed to use NAS signalling or application level interaction to provision local multicast service boundary information to the UE, who is interested in the local multicast service. When the UE enters into the multicast service boundary, it detects the broadcast signalling (i.e. SIB) for local multicast service discovery. The architecture of this solution can be based on the two architecture alternatives (see Annex A).

### 6.7.2	Procedures

Figure 6.7.2-1 shows the procedure of local multicast service discovery and provisioning for receiving multicast data:

Figure 6.7.2-1: Local multicast service discovery procedure

1.	The UE, who interests in the local multicast service, may register to the AF of the local MBS Content Provider and may get information for receiving the local multicast service data from the Content Provider, e.g., application layer security information, TMGI, service boundary (e.g. cell IDs, geo area information), etc.

2.	The AF may provide service parameters including service boundary (e.g. cell IDs, geo area information) to NEF, the NEF may store the information in the UDR (translation maybe needed).

3.	The UE may retrieve service parameters including service boundary at any time anywhere in the PLMN via control plane, e.g. initiates User Multicast Service Create procedure as described in solution 4. Or, the UDR may notify the (MB-)SMF that subscribed event for multicast service context change of the updated service parameters, the (MB-)SMF queries the UDM for each member of the TMGI to get the serving AMF and notifies the UE of service boundary for the TMGI, e.g. via Namf_Communication_N1N2MessageTransfer service operation. Or, the UE may request service parameters update via control plane to get the updated service boundary information, e.g. initiates User Multicast Service Update procedure as described in solution 4.

4.	The UE starts to detect whether it is in the Service Boundary, e.g. based on the TAI or cell ID received in the SIB1. If the UE does not entered into the service boundary, the UE does not need to resolve SIB other.

5.	When the UE is in the local multicast service boundary, the UE starts to discover the local multicast service based on SAI/TMGI detection, i.e. based on SAI/TMGI received in SIB other.

6.	If the UE detects radio resource information for data transmission of the local multicast service, it starts receiving the service data. If the UE does not detects radio resource, the UE performs user multicast session join for activate the data transmission via the RAN node, e.g. initiates User Multicast Session Join procedure as described in solution 4 specified in clause 6.4.

### 6.7.3	Impacts on services, entities and interfaces

UE:

1. Support multicast communication service detection triggered by service boundary entering/leaving.

END OF CHANGES