3GPP TSG-SA WG2 Meeting #139E 	S2-2003899

June 1 – 12, 2020, Elbonia	 (revision of S2-xxxxxxx)

Source:	CATT

Title:	KI#16, New solution for KI#16: NWDAF assisted UP optimization for EC

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes a solution to optimize UP path towards EC server based on NWDAF output.

Discussion

EC in 5GS aims to improve user experience on content access by providing computing/storage capability closing to RAN. However, in some specific area, there may be multiple EC servers providing EC service or multiple anchor UPFs being able to connect to the EC servers. Therefore, the anchor UPF and the serving EC server must be carefully selected to promise user experience.

Another aspect, due to limited service area of an EC server and UE mobility, UP anchor change and application server relocation may be frequent, which has significant impacts on 5G system performance, and then user experiences may downgrade. Therefore, in order to promise user experience on visiting EC applications in a 5GS with support of EC, anchor UPF must be carefully selected to minimize the number of UPF anchor change, and correspondingly, the EC server on the application layer need also be selected carefully to minimize the number of application server relocation.

In order to achieve this, it is proposed to use NWDAF analytics to optimize the selection of anchor UPF and target EC server. From our perspective, apart from UE mobility analytics defined in Rel 16, the NWDAF need also provide following analytics information:

| Information | Description |
| --- | --- |
| EC Application ID |  |
| S-NSSAI |  |
| EC Service experiences (0-x) | List of observed service experience information for the EC Application |
| >Service Experience | service experience on visiting the EC Application |
| >APP location | Application location, could be identified by DNAI |
| >Serving anchor UPF | The involved anchor UPF |
| >Spatial Validity Condition | Indicates the specific area where the service experience can be promised |
| >Temporal Validity Condition | Indicates the specific time period in which the service experience can be promised |
| >Confidence |  |

Instead of providing service experience analytics, the NWDAF can provide performance analytics, which allows the SMF to select an anchor UPF with special preference, e.g. selecting a UPF which can provide the largest traffic rate. The EC performance analytics is shown below:

| Information | Description |
| --- | --- |
| EC Application ID |  |
| S-NSSAI |  |
| EC performance (0-x) | List of EC performance for the EC application |
| >APP location | Application location, could be identified by DNAI |
| >Serving anchor UPF | The involved anchor UPF |
| >Performance | Performance indicators |
| >> Average Traffic rate | Average traffic rate for visiting the EC application |
| >> Maximum Traffic rate | Maximum traffic rate for visiting the EC application |
| >>Average Packet Delay | Average packet delay for visiting the EC application |
| >>Maximum Packet Delay | Maximum packet delay for visiting the EC application |
| >> Average Packet Loss Rate | Average packet loss for visiting the EC application |
| >Spatial Validity Condition | Indicates the specific area where the service experience can be promised |
| >Temporal Validity Condition | Indicates the specific time period in which the service experience can be promised |
| >Confidence |  |

The NWDAF analytics need to be provided to the SMF and the EC server, so that the SMF can select the anchor UPF which can provide best user experience on the designated area, and the most appropriate target DNAI when UPF change happens.

Proposal

It is proposed to include following new solution into the TR 23.700-91.

FIRST CHANGE (all new text)

## 6.X	Solution for Key Issue #16: NWDAF assisted UP optimization for EC

### 6.X.1	Description

Editor's note:	Describe the solutions. (sub) clause(s) may be added to capture details, procedural flow etc.

In order to promise user experience, when a UE is visiting EC server, the 5GS needs to select an appropriate anchor UPF to serve the UE. The appropriate anchor UPF here means that, the UP path via this anchor UPF can satisfy the QoS requirement well, and the number of anchor UPF relocation can be minimized.

In this solution, it is proposed to rely on NWDAF analytics to select anchor UPF and DNAI to promise user experience. Besides UE mobility analytics defined in Rel 16, an new analytics named “EC service experience/ performance analytics” is introduced, which contains the service experience/performance information for an EC application per specific area per time period.

The UE mobility analytics and “EC service experience/ performance analytics” will be notified to the SMF and EC server. Based on such NWDAF analytics, the SMF can select the anchor UPF which can provide best user experience to the UE along the UE moving trajectory and determine the most appropriate target DNAI; the EC server can determine the target DNAI and/or target EC server if EC server relocation is required.

The input data for generating the “EC service experience/ performance analytics” could be collected from 5GC and EC server. The input data collected from 5GC is defined in the Table 6.4.2-2 in TS 23.288 with some additions, and the input data collected from EC server is defined in the Table 6.4.2-1 in TS 23.288. The additions to the Table 6.4.2-2 in TS 23.288 is shown in the table 6.x.1-1.

Table 6.x.1-1 Additions to the Table 6.4.2-2 in TS 23.288

| Information | Source | Description |
| --- | --- | --- |
| Content defined in the Table 6.4.2-1 in TS 23.288 | Content defined in the Table 6.4.2-1 in TS 23.288 | Content defined in the Table 6.4.2-1 in TS 23.288 |
| Anchor UPF ID | SMF | The anchor UPF of the PDU session containing the QoS flow |

The EC service experience analytics is shown in the Table 6.x.1-2.

Table 6.x.1-2 EC service experience analytics (statistic/prediction)

| Information | Description |
| --- | --- |
| EC Application ID |  |
| S-NSSAI |  |
| EC Service experiences (0-x) | List of observed service experience information for the EC Application |
| >Service Experience | service experience on visiting the EC Application |
| >APP location | Application location, could be identified by DNAI |
| >Serving anchor UPF | The involved anchor UPF |
| >Spatial Validity Condition | Indicates the specific area where the service experience can be promised |
| >Temporal Validity Condition | Indicates the specific time period in which the service experience can be promised |
| >Confidence |  |

The EC service performance analytics is shown in the Table 6.x.1-3.

Table 6.x.1-2 EC service performance analytics (statistic/prediction)

| Information | Description |
| --- | --- |
| EC Application ID |  |
| S-NSSAI |  |
| EC performance (0-x) | List of EC performance for the EC application |
| >APP location | Application location, could be identified by DNAI |
| >Serving anchor UPF | The involved anchor UPF |
| >Performance | Performance indicators |
| >> Average Traffic rate | Average traffic rate for visiting the EC application |
| >> Maximum Traffic rate | Maximum traffic rate for visiting the EC application |
| >>Average Packet Delay | Average packet delay for visiting the EC application |
| >>Maximum Packet Delay | Maximum packet delay for visiting the EC application |
| >> Average Packet Loss Rate | Average packet loss for visiting the EC application |
| >Spatial Validity Condition | Indicates the specific area where the service experience can be promised |
| >Temporal Validity Condition | Indicates the specific time period in which the service experience can be promised |
| >Confidence |  |

### 6.X.2	Procedures

Figure 6.X-2: Procedure for NWDAF providing data analytics for an EC Application

1.	Consumer NF (e.g. SMF, AF) sends an Analytics request/subscribe (Analytics ID = UE mobility and EC Service experience/performance) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2-4.The NWDAF performs data collection if it hasn’t subscribe the input data for the requested analytics. The NWDAF has to collect data from AMF, SMF and AF.

5.	the NWDAF derives the requested analytics.

6.	The NWDAF provide the analytics result to the consumer NF, including UE mobility analytics and EC service experience/performance analytics.

7.	During the PDU session establishment or anchor UPF change (as specified in clause 4.3.5 of TS 23.502) procedure, the SMF, as an analytics consumer, can use UE mobility analytics to predict UE location, and then check EC service experience/performance analytics, by taking UE location into account, to find which UPF and DNAI can serve the UE with the best user experience or service performance.

The AF as an analytics consumer can also use UE mobility analytics and EC service experience/performance analytics to determine which DNAI is more suitable if EC server relocation is required.

### 6.X.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

End of CHANGE