3GPP TSG-WG SA2 Meeting #139E e-meeting 	S2-2004062

Elbonia, June 1 – 12, 2020	(revision of S2-200xxxx)

Source:	China Mobile, Huawei

Title:	KI #1, New Sol: Dynamic multicast/broadcast transmission

Document for:	Discussion/Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS/Rel-17

Abstract of the contribution: Discuss a new scenario that MBS is not pre-defined, it is triggered temporarily and dynamically. And a possible solution is proposed.

- Discussion

1.1 Lessons from LTE MBMS

In LTE, MBMS services are pre-defined and each MBMS service will be assigned with a unique identity such as TMGI. UE is informed with the available MBMS services via such as announcement procedure, then UE joins a multicast group to receive the MBMS data. Although LTE MBMS is fully discussed and specified, we should notice that LTE MBMS is not commercially used due to the following reasons：

- Multicast does not share the common architecture and interfaces as unicast;

- Mobile operators could not find use cases and business model under the LTE MBMS framework, since the most of attractive applications/services are not pre-defined. Support of some scenarios such as public safety is not sufficient enough for mobile operators to introduce multicast framework.

Based on the experience of LTE MBMS above, it’s necessary to consider the business model of NR multicast service seriously. If the NR MBS discussed currently still only focuses on the pre-defined MBS like LTE MBMS, it could be difficult to come to the final commercial utilization of mobile operators.

Observation 1: It’s difficult for operator to come to the commercial usage of NR MBS if we only focus on the pre-defined MBS is studied in NR.

1.2 New scenarios for NR MBS

Besides Public safety and TV broadcasting, many new scenarios with multicast attribute appear in 5G network, such as live concerts/sport events or live video streaming via smart phone. All these new scenarios have the characteristics listed below:

- The service has high requirements on the data rates or latency;

- There may be a lot of users to use data/voice communication at the same time, for example, some of users are watching high-definition real-time video of a backstage and some of them are performing basic communication, such as voice call or internet surfing;

- The users may be gathered temporally and separate with each other after a period of time.

Taking live concert as an example. The concert host may provide different view of the backstage real-time video with high-definition and low latency, for a better experience. And the concert audiences may pay for the real-time video service, and watch the video via their smart phones. Now this kind of service is provided by unicast, which results in the waste of the resources of Uu and the core network backhaul. What’s more, network congestion may be caused, and even the basic communication could not be provided.

Besides, for the live video watching, a large proportion of of the service used to watch is smart phone, and when there are too many people watching the same live video in the same cell, for example, in a school dormitory building, it may also cause the waste of resources and network congestion mentioned above.

To solve this problem simply, we can define these services as multicast services, and then transmit them as the current discussion. However, there are some problems:

- For content provider: most of the services have a mature working model with unicast. If the service is defined as multicast service, it means the content provider need to do a lot of change to match the multicast transmission, which is not welcome. And as we discussed above, And as we disussed above, the multicast group is formulated by chance and the scenarios/use cases are rather various, therefore, defining them as multicast service may cause a lot of overhead.

- For the 5G network: the previous LTE MBMS data transmition in the EPC is based on the IP multicast, which is not supported in the current network.

- For users: if some of the services is defined as MBS other than normal services, it may change the users’ operation habits.

Observation 2: not all services or scenarios with multicast character suits to be pre-defined as MBS.

A better way is to not pre-define such services as MBS, but the multicast transmission could be triggered temporarily and dynamically by the network for resource saving and congestion avoiding.

With this way, UE doesn’t need to do special things like obtaining the network announcement or perform the joining procedure, but request for the service as normal. From the content provider perspective, there’s no pre-defined multicast service, which means it does not need to do changes like supporting MBS-specific signalling and data processing. Such proposal does not prevent the future enhancement that content provider freely makes statistics and helps the core network establish a multicast transmission. In our view, this is a more feasible way for the multicast commercial use.

- Proposal

It is proposed to adopt the following solution in TR 23.757 to support none pre-defined MBS service.

* * * First Change* * * *

6.x	Solution X: Dynamic multicast/broadcast transmission

6.X.1	Key Issue mapping

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

This solution addresses the following aspects of KI1:

-	When and how to trigger the establishment of an MBS session/tunnel?

6.x.2	Functional Description

6.x.2.1	System Architecture

The architecture functional entities are described in Annex A.1 "5G MBS system architecture based on unicast 5GC".

6.x.3	Procedure

Editor's Note: This clause describes high-level procedures and information flows for the solution.

6.X.3.1	 dynamic multicast session/tunnel setup/modification

Figure 6.X.3-1: multicast session/tunnel setup/modification

This key steps of this solution as:

-	The UE establishes a PDU session for unicast flow, and the SMF is selected.

-	The Content Provider could optionally make some statistics for different services. The statistics done by the content provider may be triggered actively or passively.

-	The Content Provider indicates the SMF the multicast service information, e.g., statistics result, UE number of requesting the service, the list of UE IDs, and other service level information, such as IP address information. Afterwards SMF could further, based on such info, to determine the establishment/modification for the multicast service.

-	The SMF sends to RAN the binding relationship between the UE and the service.

-	RAN gets the binding information of UEs, service and related tunnel/session.

-	RAN decides how to deliver the multicast flow: PTM or PTP and sends reconfiguration to UEs if necessary.

Editor's note: The details of this procedure will be studied in the RAN WGs.

6.X.4	Impacts Analysis

Editor's note: This clause describes impacts to existing entities and interfaces.

Editor's note: Further impacts on NFs would be added later.

SMF: The SMF needs to support the processing of multicast, e.g., sends to RAN the binding relationship, receives the information of the Content Provider.

RAN: The RAN needs to support the processing of multicast, e.g., binding, selecting PTP or PTM bearers that are used for multicast data transmission to UEs.

* * * End of Changes* * * *