SA WG2 Meeting #140-e	S2-2004859

August 19th -September 2nd 2020, Online	(revision of S2-20xxxx)

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	KI#7: solution 29 update

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2

Abstract of the contribution: xxx

# Discussion

This solution allows to have some cells in a TA that do not support the S-NSSAI the TA supports. This is incorrect. Even the proposed solution on how to deal with that is incorrect:

" However, this solution allows for the case where some cell(s) in the tracking area(s) do not support all of the slices in the Allowed NSSAI. If some cell(s) in the tracking area(s) do not support all of the slices in the Allowed NSSAI, then the RFSP Index will be selected such that the UE will not select operating bands (i.e. cells) that do not allow the UE to simultaneously access all of the slices in the Allowed NSSAI.

"

There is no guarantee the RFSP provided by the CN will result in the UE getting CellRelectionPriorityList in RRC connection release that is perfectly aligned. But, more importantly, the cell reselection by the UE is not guaranteed to never select a non-supporting cell. If the UE selects that cell it will not perform a Registration to update the network and the UE is still assumed to be able to use the S-NSSAI. De facto for the rest of the network this becomes a supporting cell. Hence the definition of non-supporting cell becomes uncertain from a system standpoint.

# Proposal

The following clarifications are proposed to be added to TR 23.700-40

## 6.29	Solution#29: Operating Band Information is Provided with the Configured NSSAI

### 6.29.1	Introduction

This solution is for Key Issue #7, "Support of 5GC assisted cell selection to access network slice". The solution is based on the following architectural assumption:

-	The solution introduces the restriction that the UE shall not simultaneously register to slices that are not all accessible on the same operating band(s). In other words, the UE shall not have S-NSSAIs in the Allowed NSSAI that are not all available in a common operating band.

-	The following Rel-15/16 principles shall not be changed

-	The UE can establish and maintain PDU Session(s) with any slice(s) that are in its Allowed NSSAI.

-	The Allowed NSSAI indicates the S-NSSAIs values the UE can use in the Serving PLMN for the current Registration Area.

-	A TA is the set of all cells that broadcast the same TAC.

### 6.29.2	High-level Description

The main idea of the solution is that the UE is provisioned with the operating band(s) that are allowed for each S-NSSAI in the PLMN. This information is sent to the UE in the Configured NSSAI and Mapping of Configured NSSAI. It is then understood that the UE will only request slices that are all accessible in the same operating band(s) and the AMF will include in the Allowed NSSAI only S-NSSAI(s) that are all accessible in the same operating band(s).

Per existing Rel-15/16 behaviour, the RFSP Index will be determined based on the Allowed NSSAI.

Per existing Rel-15/16 behaviour, the serving AMF shall determine a Registration Area such that all S-NSSAIs of the Allowed NSSAI for this Registration Area are available in all Tracking Areas of the Registration Area.

When the UE is provisioned with a Configured NSSAI or Mapping of Configured NSSAI (i.e. pre-provisioned, provisioned during Registration, or provisioned during a UE Configuration Update), the UE is also provisioned with the operating band(s) of each S-NSSAI in the Configured S-NSSAI or Mapping of Configured NSSAI. In the case where the operator slice deployment is governed by frequency ranges (e.g. FR1 and FR2), the AMF may indicate that an S-NSSAI is supported on [FR1 only], [FR2 only] or [FR1+FR2] instead of explicitly listing out all the operating bands where each S-NSSAI is supported. The Default Configured NSSAI is not associated with any specific operating band information but may be associated with frequency range information.

The UE may take into account the operating band(s) of each S-NSSAI when the UE selects a cell to camp on and/or generates a Requested NSSAI, e.g. UE selects S-NSSAIs which are associated with the operating band that the UE is currently using and generates a Requested NSSAI based on the operating band.

If the UE is interested to access an S-NSSAI which is not supported on the currently camped operating band and not in the Allowed NSSAI, the UE may choose to re-select to the new operating band, initiate the radio connection, and send a Mobility Registration Update Request and include the S-NSSAI in the Requested NSSAI.

### 6.29.3	Procedures

The Registration Accept message and UE Configuration Update Command from the AMF to the UE in TS 23.502 [6] are updated to include permissible operating band(s) of each S-NSSAI in the Configured S-NSSAI and to include permissible operating band(s) of each S-NSSAI in the Mapping of Configured S-NSSAI.

In the Registration and UE Configuration Update procedures, the AMF will only include S-NSSAIs in the UE's Allowed NSSAI that are all available via the UE's current operating band.

The AMF may be locally provisioned with operating band information for each S-NSSAI.

### 6.29.4	Impacts on services, entities and interfaces

AMF:

-	Provides the UE with permissible operating band(s) for each S-NSSI in the Configured NSSAI and Mapping of Configured NSSAI.

UE:

-	During the Registration and UE Configuration Update procedures, receives permissible operating band(s) for each S-NSSAI in the Configured NSSAI and Mapping of Configured NSSAI.