SA WG2 Meeting #140E	S2-2005040

19 AUG –  2 SEP 2020

Source:	III,

Title:	KI #20, New Sol: NWDAF assisting in detecting anomaly events for the user plane

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2/ Rel-17

Abstract of the contribution: This contribution proposes a new solution for NWDAF assisting in detecting anomaly events for the user plane.

- Discussion

The SMF is responsible for instructing the user plane (UP) function about how to classify arriving user data traffic based on Packet Detection Rule (PDR). The parameters provided within a PDR describe how the UP function shall treat a packet that matches the detection rule. Also, the Session Reporting Rules (SRR) contains information to request the UP function to detect and report events for a PDU session that are not related to specific PDRs of the PDU session. Leveraging the capabilities of PDR and SRR, the NWDAF can collect information about the UP, and analyse or predict the anomaly behaviour and events in the UP.

2.	Proposal

It is proposed to include the added text into the TR 23.700-91 as a solution for Key Issue #20.

* * * * First Change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 11 |  |  |  |
| 2 | 4 |  |  |  |
| 3 | 15 |  |  |  |
| 4 | 12 |  |  |  |
| X | 20 |  |  |  |

* * * * Second Change  * * * *

6.X	Solution X: NWDAF assisting in detecting anomaly events for the user plane

6.X.1	Description

This solution is for Key Issue #20: NWDAF assisting in detecting anomaly events for the user plane.

The PDR contains information to achieve the following: i) Traffic detection (e.g. classifying traffic of IP type, Ethernet type, or unstructured type); ii) Traffic reporting (e.g. allowing SMF support for charging); iii) QoS enforcement; and iv) Traffic routing for Uplink Classifier(UL-CL) or IPv6 multi-homing. The Session Reporting Rules (SRR) contains information to request the UP function to detect and report events for a PDU session that are not related to specific PDRs of the PDU session. The following provides descriptions of events the SRR should provide in the report: i) Per QoS flow per UE QoS Monitoring Report. ii) Change of 3GPP or non-3GPP access availability, for PDU session.

With the capabilities of collecting application service related information from OAM and AF as well as collecting traffic characteristic information about the UP leveraging PDR and SRR, NWDAF can analyse or predict the anomaly behaviour and events in the UP.

6.X.2	Input Data

The detailed data are described in Table 6.X.2-1.

Table 6.X.2-1 input data to detecting anomaly events for the user plane

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | AF | A time stamp associated with the collected information. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QFI | SMF | QoS Flow Identifier |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |
| N4 Session ID | UPF | Identifies the N4 session associated to this PDR/SRR |
| Rule ID | UPF | Unique identifier to identify this rule. |
| QoS Monitoring per QoS flow Control Information | UPF | Indicates the UPF to apply the QoS Monitoring report for one or more QoS Flows. |
| Precedence | UPF | Determines the order, in which the detection information of all rules is applied. |
| Outer header removal | UPF | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. |
| Forwarding Action Rule ID | UPF | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |
| List of Usage Reporting Rule ID(s) | UPF | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |
| List of QoS Enforcement Rule ID(s) | UPF | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |
| Packet Detection Information. | UPF |  |
| >Source interface | UPF | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". |
| >UE IP address | UPF | One IPv4 address and/or one IPv6 prefix with prefix length |
| >Network instance | UPF | Identifies the Network instance associated with the incoming packet. |
| >CN tunnel info | UPF | CN tunnel info on N3, N9 interfaces, i.e. F-TEID. |
| >Packet Filter Set | UPF | Packet Filter Set |
| >Application ID | UPF | Application ID |
| >QoS Flow ID | UPF | Contains the value of 5QI or non-standardized QFI. |
| >Ethernet PDU Session Information | UPF | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session |
| >Framed Route Information | UPF | Refers to Framed Routes |

6.X.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.X.3-1.

- Table 6.X.3-1. Output for detecting anomaly events for the user plane

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided |
| List of user plane anomaly event analytics (1..max) |  |
| >Applicable Time Window | The time period that the analytics applies to |
| >Network Status Indication | Anomaly event |
| > Confidence | Confidence of this prediction |

Editor’s Note: To predict the anomaly event or not is FFS.

6.X.4	Procedures

- The OAM is requesting a detection of anomaly event to the NWDAF.

- The NWDAF send Event Exposure Subscribe to AF to gathering the relative parameters for application server.

- The AF response and send the related information via Event Exposure Notify message.

- The NWDAF send Event Exposure Subscribe to SMF to gathering the relative parameters for PDU session and traffic.

- SMF forward the PDR and SSR package to NWDAF after receiving the NWDAF subscribing traffic situations via Event Exposure Notify message.

Editor’s note: Whether SMF using Nsmf_EventExposure_Notify message to forward the Usage Information or not is FFS.

- The NWDAF collection the characteristics from traffic and applications to detect anomaly event.

- The NWDAF reports the data analytics to provide anomaly event statistics or predictions.

6.X.5	Impacts on services, entities and interfaces

NWDAF:

- Need to provide a new analytic for anomaly event detection information

SMF:

- Need to forward user plane packets to NWDAF.

AF:

- Need to forward application server information to NWDAF.

* * * * End of change * * * *