SA WG2 Meeting #140E e-meeting 	S2-2005200

Elbonia, Aug 19 – Sep 01, 2020	 (revision of S2-20xxxxx)

Source:	CATT

Title:	KI#3, New Sol: Service continuity for Layer-3 UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This paper proposes service continuity solution options for Layer-3 UE-to-Network Relay.

1	Introduction

One of the issues for KI#3 UE-to-Network Relay is about service continuity.

It can be seen from the definition of “service continuity” in TS 22.261 and TS 23.501 that, service continuity does not require IP address preservation and can be achieved at different levels.

This contribution proposes to two options to achieve service continuity for Layer-3 UE-to-Network Relay: i.e. application layer mechanism and 3GPP defined SSC modes.

2	Proposal

It is proposed to add a new solution for Key Issue #3 service continuity into TR 23.752.

First change (all text new)

6.X	Solution #X: Service Continuity for Layer-3 UE-to-Network Relay

6.X.1	Description

This solution addresses Key Issue #3 on service continuity for UE-to-Network Relay.

“Service continuity” refers to “the uninterrupted user experience of a service that is using an active communication when a UE undergoes an access change without, as far as possible, the user noticing the change.” as defined in TS 22.261 [3]. It is further elaborated that service continuity can be achieved by using different means: “Moreover, some applications require the network to ensure seamless mobility of a UE so that mobility is hidden from the application layer to avoid interruptions in service delivery while other applications have application specific means to ensure service continuity. But these other applications may still require the network to minimize interruption time to ensure that their application-specific means to ensure service continuity work effectively.”

TS 23.501 [6] also gives a similar definition of “service continuity” by “The uninterrupted user experience of a service, including the cases where the IP address and/or anchoring point change.” as well as the definition of “session continuity” by “The continuity of a PDU Session. For PDU Session of IPv4 or IPv6 or IPv4v6 type "session continuity" implies that the IP address is preserved for the lifetime of the PDU Session.”

It can be seen from the above that “service continuity” is different from “session continuity” by definition, and service continuity does not require IP address preservation and can be achieved at different levels by using different mechanisms.

For the service continuity for Layer-3 UE-to-Network Relay, both application layer mechanism and 3GPP defined mechanism can be used and further illustrated in the following section.

6.X.2	Procedures

The following two options are provided to achieve service continuity for Layer-3 UE-to-Network Relay.

Option 1: Service continuity is achieved using application layer mechanism

The different application layer mechanisms can be used for different use cases to achieve service continuity for Layer-3 UE-to-Network Relay:

- For Mission Critical Service in Public Safety, service continuity can be achieved by the application layer mechanism, e.g. as described in Annex B in TS 23.280 [xx].

- For commercial IMS use cases, service continuity can be achieved using mechanisms described in TS 23.237 [yy].

- For commercial use cases with application layer out of 3GPP scope (e.g. non IMS), service continuity can be achieved using similar way, e.g. QUIC.

It is noted that all these existing application layer mechanisms can be reused for Layer-3 UE-to-Network Relay without any enhancements in this study item.

Option 2: Service continuity is achieved using 3GPP defined mechanisms

The Session and Service Continuity (SSC) modes are defined in TS 23.501 [6] and can be reused to achieve service continuity for the Layer-3 UE-to-Network Relay.

For the baseline Layer-3 UE-to-Network Relay architecture in solution#6, the SSC mode 3 can be used to achieve service continuity when Remote UE performs path switching between direct and indirect network communication path. For the service continuity from direct to indirect network communication path, the Remote UE may establish PC5 connection with UE-to-Network Relay (which establishes PDU session for relaying) before it releases the PDU session in the direct network communication path. For the service continuity from indirect to direct network communication path, the Remote UE may establish a new PDU session in the direct network communication path before it releases the PC5 connection with the UE-to-Network Relay (which may release the PDU session for relaying).

The SSC mode 1 is not pursued for solution#6 as the PDU sessions established by Remote UE and UE-to-Network Relay are deemed as different PDU sessions and the anchor UPF may not be kept.

For the Layer-3 UE-to-Network Relay architecture using N3IWF in solution#23, as the UE-to-Network Relay access using N3IWF can be seen as untrusted non-3GPP access the “Handover of a PDU Session procedure between 3GPP and untrusted non-3GPP access” defined in TS 23.502 [8] clause 4.9.2 can be reused for Layer UE-to-Network Relay using N3IWF.

6.X.3	Impacts on Existing Nodes and Functionality

UE and CN need to support the SSC modes and procedures as described in clause 6.X.2.

Second change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS); Stage 1".

[3]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[4]	3GPP TS 22.115: "Service aspects; Charging and billing; Stage 1".

[5]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[6]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[7]	3GPP TR 22.842: "Study on Network Controlled Interactive Service (NCIS) in the 5G System (5GS); Stage 1".

[8]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[9]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[10]	IETF RFC 826: "An Ethernet Address Resolution Protocol".

[11]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[12]	3GPP TS 29.343: "Proximity-services (ProSe) function to ProSe application server aspects (PC2); Stage 3".

[13]	3GPP TS 32.277: "Telecommunication management; Charging management; Proximity-based Services (ProSe) charging".

[14]	3GPP TR 23.713: "Study on extended architecture support for proximity-based services".

[15]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[16]	3GPP TS 32.256: "Charging management; 5G connection and mobility domain charging; Stage 2".

[17]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system, charging service; Stage 3".

[18]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[19]	3GPP TS 22.186: "Service requirements for enhanced V2X scenarios".

[20]	3GPP TS 24.334: " Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects; Stage 3".

[21]	3GPP TS 24.333: " Proximity-services (ProSe) Management Objects (MO)".

[22]	3GPP TS 32.290: " Services, operations and procedures of charging using Service Based Interface (SBI)".

[23]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[24]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[25]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[26]	3GPP TR 23.733: "Study on architecture enhancements to Proximity Services (ProSe) UE-to-Network relay".

[27]	3GPP TR 36.746: "Study on further enhancements to LTE Device to Device (D2D), UE to network relays for Internet of Things (IoT) and wearables".

[28]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[xx]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[yy]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

End of changes