3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005295

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Tencent

Title:	Clarification on Multicast/Broadcast Communication Service and EN Resolving

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution propose clarification to definition of broadcast and multicast service area and resolve the EN for these two definitions.

1. Discussion

In current TR 23.757, regarding to the definitions of broadcast and multicast service, there is an EN as follows:

Editor's note:  The definitions of broadcast service area and multicast service area may be updated further according to 5MBS study.

In our view, current definitions of broadcast and multicast communication service is not clear and accurate.

For broadcast communication service, actually not all the UEs within a geographical area need to be authorized.  Instead, the authorized UEs in the geographical area are able to receive the broadcast service but not the broadcast server or network is actually unaware whether these authorized UEs in the geographical area are receiving the broadcast service.  Thus we propose to add NOTE 1 to the definition of Broadcast communication service.

For multicast communication service, we think compared with the broadcast communication service, the server or the network may know whether the authorized UEs are receiving the data being delivered. Thus we propose to add NOTE 2 to make this more clear.

With these two NOTEs for clarification, we propose to remove the EN regarding to the definition of broadcast and multicast service.

2. Text Proposal

It is proposed to capture the following changes to TR 23.757.

# * * * * First change * * * *

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Broadcast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast coverage area are authorized to receive the data).

NOTE 1: For broadcast communication service, the server or network may not be aware of whether the authorized UEs are actually receiving the data being deliver in broadcast approach.

Broadcast service area: The area within which data of one or multiple Broadcast session(s) are sent.

Broadcast session: A session to deliver the broadcast communication service. A broadcast session is characterised by the content to send and the geographical area where to distribute it.

Multicast service area: The area within which data of one or multiple Multicast session(s) are sent.

MBS session: A multicast session or a broadcast session.

Multicast communication service: A communication service in which the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast coverage are authorized to receive the data).

NOTE 2: For multicast communication service, the server or network can be aware of whether the authorized UEs are actually receiving the data being delivered in multicast approach.

NOTE 3:	The transport in the 5GC for broadcast and multicast services will be determined as part of this study. A multicast communication service could for instance use a lower layer unicast or multicast transfer in the 5GC and the access interfaces.

Multicast session: A session to deliver the multicast communication service. A multicast session is characterised by the content to send, by the list of UEs that may receive the service and optionally by a multicast area where to distribute it.

Receive Only Mode: A UE configuration option that allows a UE to receive only broadcast service without the need to access and register with the PLMN offering the MBS service. Use of Receive Only Mode does not require USIM for the UE.

Shared MBS Network: A network shared by multiple PLMNs that provides multicast or broadcast services. At least the northbound data entrance point in the 5GS is shared.

5GC Individual MBS traffic delivery: 5G CN receives a single copy of MBS data packets and delivers separate copies of those MBS data packets to individual UEs via per-UE PDU session.

NOTE 4:	It will be determined based on the selected solutions whether the 5GC Individual delivery method is supported.

5GC shared MBS traffic delivery: 5G CN receives a single copy of MBS data packets and delivers a single copy of those MBS data packets to a RAN node.

NOTE 5:	For 5GC shared MBS traffic delivery the RAN node either delivers a single copy of MBS data packets over common radio resources to a set of UEs (PTM) or delivers separate copies of MBS data packets over dedicated radio resources to individual UEs (PTP).

# * * * * End of changes * * * *