SA WG2 Meeting #140E (e-meeting)	S2-2005297

August 19 – September 1, 2020, Elbonia	(revision of S2-200xxxx)

Source:	LG Electronics

Title:	KI #2, Evaluation and Conclusion for KI#2

Document for:	Approval

Agenda Item:	8.2 Study on enhanced support of Non-Public Networks

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This paper proposes evaluation and conclusion for KI#2.

Background

Proposal

It is proposed to capture the following solution to TR 23.700-07.

* * * * Start 1st Change * * * *

7	Evaluation

7.2	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

Editor's note:	This clause will provide a general evaluation and comparison of the solutions per Key Issue #<X>.

Solution #13, #14, #15, #16, #17, #18 and #X are proposed for KI#2.

-	Solution #13 proposes to use N3IWF architecture to support service continuity between NPN and PLMN, and paging from both networks. This solution reuses Rel-16 and there is no architectural impact.

-	Solution #14 proposes to keep the UE in CM-CONNECTED to when the UE established NWu connection via N3IWF. However, necessity of this solution is not clear because NWu connection can be maintained even though the UE goes CM-IDLE in the serving PLMN.

-	Solution #15 proposes to support simultaneous communication only for Dual Radio UEs and leave it as an implementation. Simultaneous paging reception can be supported based on multi-SIM like implantation.

-	Solution #16 proposes to reuse existing procedures such as N2 handover or session continuity (SSC mode 2/3) procedures. However, this solution does not provide network level service continuity, i.e. IP address preservation and only applicable to specific deployment scenario.

-	Solution #17 proposes "inactive" mode for NWu interface. Benefit of introducing such mode and potential impact to IKEv2 tunnel is not clearly described.

-	Solution #18 proposes a mechanism for 2Rx / 1Tx UE. This solution has overall system impact while benefit of having this solution is not clear as 2Rx / 1Tx UE can use solution for 1Rx / 1T, i.e. Rel-16 N3IWF architecture.

-	Solution #X proposes that the SNPN provides PLMN List which includes the list of PLMNs that the SNPN has an SLA in the Registration Accept so that the UE can know whether serving PLMN can support QoS requirements for VIAPA services.

* * * * Start 2nd Change * * * *

8	Conclusions

8.2	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

Editor's note:	This clause will capture conclusions for Key Issue #<X>.

Solution #13 and #X is selected as a baseline for normative work.

* * * * End of Change * * * *