SA WG2 Meeting #140E (e-meeting)	S2-2005302

August 19 – September 1, 2020, Elbonia	(revision of S2-200xxxx)

Source:	LG Electronics

Title:	KI #1, Evaluation and conclusion

Document for:	Approval

Agenda Item:	8.10 Study on system enablers for multi-USIM devices

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: This paper proposes evaluation and conclusion for KI#1.

Background

Proposal

It is proposed to capture the following proposal to TR 23.761.

* * * * Start of 1st Change * * * *

7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

Evaluation for KI#1:

-	Total 10 solutions for KI#1, solution #1, #2, #3, #7, #8, #9, #10, #11, #12, #13.

-	Solution #1, #7, #8, #10 and #12 provides basic functionality.

-	Solution #1 enables the UE to selectively response to the paging considering user configuration or input.

-	Solution #10 enables the UE to provide interested service and consequently network can save paging resources by blocking paging for the non-interested services.

-	Solution #7 and #12 have similar mechanism but solution #7 uses SMS while solution #12 uses IP packet to send push notification. For solution#7, additional security mechanism is need while solution #12 can reuse existing NAS or IMS level security.

-	Solution #8 assumes deployment of N3IWF but re-uses existing architecture as much as possible.

-	Solution #2 and #3 have E-UTRA impact.

-	Solution #9, #11 and #13 are enhancement on top of basic functionality

* * * * Start of 2nd Change * * * *

8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

Interim Conclusion for KI#1:

-	Solution #1, #8, #10 and #12 are selected as a baseline solution for normative work.

-	Network provides "Paging Cause" to the UE via Paging message or Push Notification via SMS.

-	"Paging Cause" is a standardized value.

-	UE can provide "Paging Filtering Rules", which indicates services the UE wants to be notified.

-	Service in "Paging Filtering Rules" is aligned with "Paging Cause".

-	Network determines to page the UE based on "Paging Filtering Rules"

* * * * End of Change * * * *