3GPP TSG-WG SA2 Meeting #140E	S2-2005357

Aug 19 – Sep 01, 2020, Elbonia	(revision of S2-20xxxx)

Source:	China Mobile

Title:	KI #1, Sol #7: Update to clarify the solution

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes clarification on Solution #7.

1. Introduction

This contribution proposes clarification on Solution #7.

2. Text Proposal

It is proposed to capture the following clarification in TR 23.700-91.

* * * * Start of Changes * * * *

6.7	Solution #7: NWDAF functionality Split

6.7.1	Description

This solution addresses Key Issue #1: NWDAF functionality Split.

As shown in Figure 6.7.1-1, the 5GC could support data analytics as:

-	Training NWDAF could provide training services to 5GC NF(s).

-	Inference NWDAF could invoke the training service provided by Training NWDAF, if needed.

Figure 6.7.1-1: NWDAF based data analytics in 5GC

As shown in Figure 6.7.1-1, NWDAF is to support data analytics cross the 5GC NFs, and AF, which are composed of the following functionalities:

1)	Data collection;

2)	Training service provided by Training NWDAF: refer to the process of creating a machine learning model, which involves the use of a machine learning algorithm and a training data set;

3)	Exposing network work data analytics to the consumer NF;

4)	Inference function provided by Inference NWDAF: refer to the process of using the trained machine learning model to make predictions with new data.

As indicated in TS 23.288 [5], the NWDAF instance(s) can be collocated with a 5GS NF.

6.7.2	Procedures

Figure 6.7.2-1: Procedure for UE mobility analytics provided to NF/AF

1a-1b.	Training service provided by Training NWDAF sends registration request by invoking Nnrf_NFManagement_NFRegister to register its own service in the NRF, including some parameters, e.g. IP address, location, load, algorithm capability and algorithm type.

2a-2b.	The Consumer NF sends a request to the Inference service provided by Inference NWDAF for analytics, using either the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service.

3.	Inference NWDAF invokes Nnrf_NFDiscovery_Request service operation using parameters such as algorithm capability, algorithm type, and optionally Inference NWDAF address.

4.	NRF matches the requested query and sends this information via Nnrf_NFDiscovery_Request response service to Inference NWDAF including one or more Training services provided by Training NWDAF.

5.	The Inference NWDAF selects one Training service based on the output of NRF.

6.	The Inference NWDAF sends request including some parameters, e.g. Analytics ID for a machine learning model to invoke the Training service provided by the Training NWDAF for machine learning model generation.

7.	The Training service in the Training NWDDAF performs data collection and machine learning model training.

8.	The Inference NWDAF receives the response from the Training NWDAF including machine learning model response and some parameters, e.g. machine learning model identification, input parameter and output parameter.

9.	The Inference NWDAF completes the machine learning model deployment and inference.

10a.	The Inference NWDAF sends analytics response to the requested NF.

10b.	The Inference NWDAF may sends Analytics Registration Request to the NRF with analytics ID and address for further Analytics enquiry.

6.7.3	Impacts on services, entities and interfaces

NWDAF is decomposed into Training Function and Inference Function as a Training NWDAF and Inference NWDAF, respectively.

Training NWDAF:

-	Support to register its NF profile (IP address, location, load, algorithm capability and algorithm type) into the NRF;

-	Support to provision a machine learning model to the Inference Function.

Inference NWDAF:

-	Support to discover a suitable Training NWDAF via NRF;

-	 Support to request a machine learning model from the Training NWDAF.

NRF:

-	Support to accept the NF registration from the Training NWDAF;

-	Support to accept the discovery of the Training NWDAF from the Inference NWDAF.

* * * * End of Changes * * * *