3GPP TSG-WG SA2 Meeting #140E	S2-2005377

Aug 19 – Sep 01, 2020, Elbonia

Source:	China Mobile

Title:	 KI #11, New Sol: Solution for increasing efficiency of data collection

Document for:	Discussion / Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes a solution for KI#11 that how to increase efficiency of data collection for multiple NWDAF instances scenario.

- Discussion

TR 23.700-91 has discussed and given requirements for KI#11:

//Requirements extracted from KI #10//:

…

Similarly, an event that a NF may raise may be of interest to multiple NWDAFs that are supporting different Analytics IDs. In subscription/notification model, all of these NWDAFs need to make explicit subscription with the NF. NF has to send multiple notifications to NWDAFs i.e. one for each subscriber.

In a multiple NWDAF environment, NWDAF instance(s) may receive from different consumers the request for the same Analytics ID and trigger multiple times the data collection for the same type of data.

As the networks scale the above mentioned issues in the context of Network Analytics become manifold leading to inefficiencies and complexity and may cause important signalling load in data collection by the NWDAF.

Based on this discussion, the proposal as per this Key Issue is to investigate whether mechanisms are needed to:

-	Reduce signalling load for data collection;

-	Achieve communication efficiencies in large networks consisting of many NF instances and NWDAF instances.

-	Reduce dependency on managing subscriptions because of lifecycle events on NF/NWDAF.

-	Reduce dependency on managing subscriptions because of change in serving entities for a UE.

-	Reduce complexity at NWDAF in determining entities serving a UE or a group of UEs and entities serving an area at a particular time window.

-	Which architectural enhancement should be defined to minimize load for data collection, e.g. for a single NWDAF as well as in a multiple NWDAF environment?

-	Which enhanced mechanisms can be defined for NWDAF and NFs to minimize the load for data collection?

-	How to prevent NWDAF(s) triggering multiple times the data collection of the same data from the same NF(s)/AF(s)/OAM?

-	How to reduce frequency of notifications that are transmitted by the source NF.

Problem: if there are multiple NWDAF instances deployed in the network, they may trigger multiple times of data collection for the same type of data from the same 5GC NF(s)/AF(s)/OAM, this may impact the major performance of 5GC NF(s)/AF(s)/OAM.

Objective: this contribution is to provide solution for how to prevent NWDAF(s) triggering multiple times the data collection of the same data from the same NF(s)/AF(s)/OAM?

2.	Proposal

It is propose a new solution for KI#11 into the TR 23.700-91.

* * * * First Change * * * *

6.X	Solution for Key Issue #11: < increasing efficiency of data collection>

6.X.1	Description

This solution is proposed to address Key Issue #11: increasing efficiency of data collection.

For multiple NWDAF instances deployed in the network, if multiple NWDAF instances trigger multiple times of data collection for the same type of data from the same 5GC NF(s)/AF(s)/OAM, this may impact the major performance of 5GC NF(s)/AF(s)/OAM.

This solution specifies for how to prevent multiple NWDAF instances triggering multiple times of data collection of the same data from the same 5GC NF(s)/AF(s)/OAM. The procedure is illustrated in 6.x.2.

6.X.2	Procedure

Figure 6.X.2-1: Procedure for increasing efficiency of data collection

- NWDAF1 service consumer sends subscription request to NWDAF1 including analytics ID(s).

- According to the analytics ID(s), the NWDAF1 collects relevant input data from 5GC NF/AF/OAM via existing NF event exposure services and/or OAM services, including event ID(s), target of event (UE(s)), event filter and event reporting information etc.

- 5GC NF/AF/OAM reports related data according to the NWDAF1 data collection requirements.

- After data collection from 5GC NF/AF/OAM, the NWDAF1 stores the collected input data and generates a related data profile (e.g. UE list/group, event ID list, filter, reporting info etc.) for these data, which may contain one or more following parameters:

-	UE group/list

- Event ID list

- Reporting info

-	Timestamp e.g. collection time.

-	Data validity time e.g. validity of collected data.

-	Data validity area e.g. data is valid for specific TA or area or slice, etc.

- Based on the collected data, the NWDAF1 generates the data analytics and notifies it to the consumer.

- The NWDAF1 registers/updates its NF status in NRF with its NF profile (e.g. NWDAF instance ID, NWDAF FQDN/IP address, NWDAF serving area) and the generated data profile.

- NWDAF2 service consumer sends another subscription request to NWDAF2 including analytics ID(s).

- Then NWDAF2 needs to do data collection, it should inquire firstly the NRF using the NF discovery service including the related data profile (e.g. UE (s), event ID (s), filter, reporting info etc.), and discover if there is a NWDAF instance which has once obtained/stored the same or similar data.

- If yes, then NRF should prefer to feedback the NWDAF instance ID/NWDAF address (e.g. NWDAF1) rather than the 5GC NF/AF/OAM instance to the NWDAF2.

- NWDAF2 collects related data from the NWDAF instance ID/NWDAF address provided by NRF in step 9, e.g. NWDAF1.

- After data collection, the NWDAF2 also registers/updates its NF status in NRF with its NF profile and the data profile.

6.X.3	Impacts on services, entities and interfaces

Editor's note:	This clause lists impacts to services and interfaces.

NWDAF:

-	Supports to registers/updates its data profile in NRF.

-	Extends NWDAF subscription services to support interactions related to data collection among NWDAFs.

NRF:

-	Supports to register/update data profiles of NWDAF.

-	Supports NWDAF instance discovery based on NWDAF data profiles.

* * * * End of change * * * *