3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005474

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #2, Sol #7: Update MP-QUIC solution to solve ENs

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_ATSSS_ph2 / Rel-17

Abstract: This contribution solves the Editor’s Notes with solution #7.

- Introduction/Discussion

This paper is to solve the ENs added to the MP-QUIC solution #7. As the ENs for the MP-QUIC solution are the same as the ENs in the QUIC-based solution, please find the details in S2-205473 for the QUIC-based solution #8.

2. Text Proposal

It is proposed to capture the following changes in the TR 23.700.

# * * * * First change * * * *

6.7	Solution #7: Proposed solution based on MP-QUIC

6.7.1	Introduction

This solution addresses KI#2 on Additional Steering Functionalities.

The ATSSS feature is to enable UE simultaneously connecting to the 3GPP access and non-3GPP access. Then the traffic can take advantage of both accesses resource to raise the bandwidth or/and access reliability. In release-16, for UDP traffic, it can only be supported by the ATSSS-LL functionality, thus an IP packet flow cannot be split over both accesses, so actually the increase of bandwidth requirement is not achieved. This solution is proposed to introduce a new steering method based on MP-QUIC to resolve this issue, referring to the KI#2 on additional steering methods.

This solution applies to UDP based application traffic only, with IP-based MA PDU Sessions (IPv4, IPv6, IPv4v6).

6.7.2	High-level Description

The MP-QUIC protocol is being drafted in IETF draft-deconinck-quic-multipath-04 [y]: " Multipath Extensions for QUIC (MP-QUIC)". It is designed to support the multipath scenario, and except the QUIC connection ID, a Uniflow ID is defined for the MP-QUIC connection. When an uniflow is in use, each end host associates it with a network path. Each uniflow is an independent flow of packets over a given network path, it can experience very different network conditions (latency, packet loss rate, …). To handle this, each uniflow has its own packet sequence number space. When the MP-QUIC protocol is applied in 5G system, especially in the ATSSS architecture, the 3GPP and non-3GPP accesses are the different paths, and the different uniflows are bound to the different paths. With this uniflow, the MP-QUIC functionality can perform the congestion control on each path and support the traffic splitting per packet as MPTCP. Therefore, it is proposed to define the MP-QUIC functionality to be supported by the UE and UPF, see the following figure 6.7.2-1 for the model on the UE side.

-

Figure 6.7.2-1: Steering functionality of MP-QUIC in the UE

This MP-QUIC functionality can be applied to steer, switch and split the UDP traffic of applications allowed to use MP-QUIC. The MP-QUIC functionality in the UE communicates with an associated MP-QUIC Proxy functionality in the UPF, by using the MP-QUIC protocol over the 3GPP and/or the non-3GPP user plane. MP-QUIC functionality can support any steering modes as defined in Rel-16.

NOTE 1:	If the application applies MP-QUIC with the server, there is no need to enable the MP-QUIC functionality between the UE and UPF. If the application applies QUIC with the server, the solution #8 as described in sublcause 6.8.2 is applied.

The MP-QUIC connection is established between the UE and the UPF per traffic flow. One or more MP-QUIC connections between the UE and the UPF may be established based on information received in the procedure as described in clause 6.7.3. These MP-QUIC connections can be encrypted or NULL encrypted. The NULL encrypted solution is similar as solution #8, referring to the description in subclause 6.8.2.

The protocol stack for MP-QUIC is shown below, taking untrusted non-3GPP access as an example. (The "Application payload" in the protocol stack represents the application layer traffic carried on top of UDP, i.e. it does not include the UDP/IP headers):

Figure 6.7.2-3: Protocol stack of MP-QUIC

Similar as MPTCP functionality, the MP-QUIC proxy functionality is deployed in the UPF, and the UPF can be transparent MP-QUIC proxy, or non-transparent MP-QUIC proxy.

For both transparent and non-transparent MP-QUIC proxy, the UPF allocates the link-specific IP addresses to the UE, and these link-specific IP addresses may be the same or different with link-specific IP addresses for MPTCP. The UPF can detect the MP-QUIC traffic based on the UE link-specific IP address and forward it to the MP-QUIC functionality. The MP-QUIC proxy terminates an MP-QUIC connection with UE and apply regular UDP transport to the remote host. The protocol stack for both transparent and non-transparent QUIC proxy is as described in Figure 6.7.2-3.

In the case of non-transparent MP-QUIC proxy, similar with MPTCP solution defined in Rel-16, the network sends the MP-QUIC proxy information to the UE, i.e. the MP-QUIC functionality IP address, and port number. The UE uses this MP-QUIC IP address as the destination IP address to encapsulate the user packets. The following IETF protocol is needed in this non-transparent MP-QUIC solution, e.g. to transport the IP address of the remote server to the UPF:

-	Socks v5: IETF RFC1928 [11]. "SOCKS Protocol Version 5".

NOTE 2:	In the case of transparent MP-QUIC proxy, the packet from the UE is encapsulated with the destination IP address of the remote server.

NOTE 3: 	In case TLS 1.3 is applied, the UE will accept the credential of UPF during MP-QUIC connection establishment procedure for both transparent and non-transparent MP-QUIC proxy, as the UE knows the MP-QUIC connection is established between the UE and the UPF.

The UE instructs the proxy to forward the traffic to the target server, similar as described in subclause 6.8.2 of solution #8.

6.7.3	Procedures

The signalling flow for a MA PDU Session establishment when the UE is not roaming, or when the UE is roaming and the PDU Session Anchor (PSA) is located in the VPLMN, is described as below. The flow relates to the case of non transparent proxy.

Figure 6.7.3-1: MP-QUIC based MA PDU Sessioon establishment procedure

-	In step 1, the UE provides a "MA PDU Request" indication in UL NAS Transport message and an ATSSS Capability indicating support of "MP-QUIC Capability" in PDU Session Establishment Request message.

The "MA PDU Request" indicates to the network that this PDU Session Establishment Request is to establish a new MA PDU Session and to apply the MP-QUIC functionality, for traffic steering of this MA PDU session.

-	In step 2, if the AMF supports MA PDU sessions, then the AMF selects an SMF, which supports MA PDU sessions, and forwards the MA PDU Session Establishment Request to the SMF.

-	In step 3, if the MA PDU session is allowed and dynamic PCC is to be used for the MA PDU Session, the SMF sends an "MA PDU Request" indication and the ATSSS Capability of MA PDU Session to the PCF in the SM Policy Control Create message. The ATSSS Capability indicates the MP-QUIC functionality.

The PCF provides ATSSS policy if the MA PDU session is allowed. The PCF provides PCC rules for the MA PDU session, i.e. PCC rules that include ATSSS policy control information, which includes the MP-QUIC functionality indication. Additionally, the PCC rules may also indicate on whether the encryption of the MP-QUIC connection is needed or not based on operator policy and subscription data or access type for this MA PDU session.

-	In step 4, the SMF establishes the user-plane resources over the 3GPP access and/or non-3GPP.

-	the N4 rules derived by SMF for the MA PDU session are sent to UPF and one or two N3 UL CN tunnels info may be allocated by the UPF. If the ATSSS functionality for the MA PDU Session indicates "MP-QUIC functionality", the SMF includes MP-QUIC functionality indication into the N4 rule to instruct the UPF to activate the MP-QUIC functionality for the traffic. If the MP-QUIC connection needs encryption or NULL encryption, the SMF also indicates it to the UPF.

-	In step 5, the UPF allocates the UE "link-specific multipath" addresses/prefixes and MP-QUIC functionality information if the non-transparent MP-QUIC functionality is applied for this MA PDU session in the UPF. The UPF sends the "link-specific multipath" addresses/prefixes and MP-QUIC functionality information to the SMF. The MP-QUIC functionality information includes the MP-QUIC functionality IP address and UDP port number, uniflow ID with corresponding access type and the MP-QUIC connection ID.

-	In step 6, for the MA PDU session, the SMF includes an "MA PDU session Accepted" indication and PDU Session Establishment Accept message which includes ATSSS rules for MA PDU Session, the "link-specific multipath" addresses/prefixes and the MP-QUIC functionality information in the Namf_Communication_N1N2MessageTransfer message to the AMF and the AMF marks this PDU session as MA PDU session based on the received "MA PDU session Accepted" indication, same as defined in Rel-16 specifications.

-	In step 8, the UE receives a PDU Session Establishment Accept message, which indicates to the UE that the requested MA PDU session was successfully established. This message includes the ATSSS rules for the MA PDU Session, the "link-specific multipath" addresses/prefixes and the MP-QUIC functionality indication and encryption or NULL encryption indication for the traffic.

-	After step 8 in Figure 6.7.3-1, if the SMF was informed in step 2 that the UE is registered over both accesses, then the SMF initiates the establishment of user-plane resources over non-3GPP access too as specified in TS 23.502 [4] clause 4.22.2.1.

The last step above is not executed when the UE is registered over one access only, in which case the MA PDU Session is established with user-plane resources over one access only. How user-plane resources can be added over an access of the MA PDU Session is specified in TS 23.502 [4] clause 4.22.7.

6.7.4	Impacts on services, entities, interfaces and IETF protocols

This solution will impact the following entities in 5GS:

-	SMF: Supports to select the UPF based on its MP-QUIC capability.

-	PCF: Supports to authorize MP-QUIC functionality steering method for the SDF.

-	UPF: Supports the MP-QUIC proxy functionality, and establishes the MP-QUIC connection with the UE.

-	UE: Supports the MP-QUIC functionality, establishes the MP-QUIC connection with the UPF.

-	5G-AN/ NG RAN: No impact.

This solution needs to enhance the existing SM NAS protocol, N5 and N4 interfaces, Npcf service to support this MP-QUIC based steering functionality.

This solution has dependency on the following IETF drafts:

-	IETF draft-deconinck-quic-multipath-04: "Multipath Extensions for QUIC (MP-QUIC)".

-	IETF draft-ietf-aft-socks-protocol-v5-05: "SOCKS Protocol Version 5", in case the non-transparent MP-QUIC solution is to be supported.

This solution may have additional impact on IETF:

-	As the current MP-QUIC connection defined by IETF shall be encrypted based on the TLS 1.3, the NULL encryption MP-QUIC connection requires IETF support.

# * * * * End of changes * * * *