SA WG2 Meeting #140e	S2-2005562

Aug 19 – Sep 01, 2020, Elbonia

Source:	OPPO

Title:	KI#8, New sol: <MNO owned Data collection AF for UE data collection>

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution:  It is to add a new solution in KI#8 in TR 23700-91.

1. Introduction

This solution introduces a Data Collection AF (DC-AF). UE data is transmitted via user-plane to ASP server and then the ASP Server sends the data to the DC-AF.  The DC-AF is owned by MNO, and used to expose the collected UE data to consumers like NWDAF.

It can help to protect privacy since the UE data can be processed to remove sensitive information before sending to DC-AF.

2. Proposal

The following changes are proposed for solution#28 in TR 23.700-91.

* * * * Start of Change* * * *

6.28	Solution #x: UE assisted analysis for usage of network slice

6.x.1 	Description

This solution introduces a Data Collection AF (DC-AF). UE data is transmitted via user-plane to ASP server and then the ASP Server sends the data to the DC-AF.

6.x.1.1 	General

As the figure shown below, as a possible way, the M1d/M2d interface defined in SA6 TS 26.501 can be enhanced to realize that the ASP server sending UE data to ASP server. For example, a new API can be introduced in M1d/M2d which is used for ASP server provides UE data to DC-AF for UE data collection.

The ASP client and ASP server belongs to OTT application layer. The communication link between ASP client and ASP server is established over user-plane using existing PDU session.

NOTE: The communication between ASP client and ASP server is out of 3GPP scope.

The DC-AF (Data Collection AF) is introduced to collect the UE data from ASP server. Then it can expose the UE data to NWDAF consumer using a new interface (Nw) .

6.x.1.2 	DC-AF collects UE data from ASP server

Figure 6.x.1.2  DC-AF collects UE data from ASP sercer

1. UE ASP Client provides UE data via user-plane application layer, which is out of 3GPP scope. After the connectivity is established, ASP Server can interact with UE ASP Client to get UE data via application layer.

2. ASP Server sends DC-AF the UE data. ASP Server needs to interact with DC-AF to establish communication channel for UE data reporting. For example, by reusing 5GMS System in SA4, the ASP Server invokes an API in M1d in DC-AF to establish the channel between ASP Server and DC-AF, then ASP Server transmit the UE data to DC-AF via M2d.

NOTE: The specific functionality needs to be determined by SA4

6.x.1.3 	DC-AF provides UE data to NWDAF consumer

Figure 6.x.1.3  NWDAF consumer get UE data from DC-AF

0. DC-AF may have already collected UE data from ASP server as described in sub-clause 6.x.1.2.

1. NWDAF consumer sends Ndc-af_EventExposure_Subscribe for UE data collection provided by MNO to use.It may include Application id or UE id t oget the specific UE data.

2. Optionally, DC-AF may collects UE data upon receiving the request from NWDAF consumer using the channel already established as described in sub-clause 6.x.1.2.

3. DC-AF responds to Consumer for the requested UE data.

6.x.1.4 	Security consideration

The ASP Client and ASP server are transmitting data via user-plane. HTTPS can be used to protect data encryption and integrity.

For privacy consideration, the ASP can process the raw UE data before sending the data to DC-AF. Some examples of processing is as follow:

- Gathering a bunch of UE’s data and average the UE data;

- Remove the User identifier;

- Add noise to each single UE data;

Also, for DC-AF, the UE data provided by ASP server can be further processed and send to consumer according to some conditions. Some examples of processing is as follow:

- Sends the UE data to consumer considering network slice separation;

- Sends UE data of permitted users to consumer.

NOTE: All the behaviour for security aspect mentioned in this sub-clause is implementation specific and out of 3GPP scope.

6.x.2 	Impacts on services, entities and interfaces

UE:

None;

NWDAF:

Supporting a new interface between NWDAF and DC-AF for UE data collection;

DC-AF:

A new Function is introduced for UE data collection. Collecting UE data from ASP server and providing UE data to NWDAF consumer.

NOTE: The DC-AF and the interface between DC-AF and ASP Server shall be defined in SA4.

* * * * End of Change * * * *