SA WG2 Meeting #S2-140E	S2-2005583

19 Aug - 2 Sep, 2020, Elbonia

Source: 	Ericsson

Title: 	KI#2: Update to solution 13

Document for: 	Approval

Agenda Item: 	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This paper updates of solution#13 for Key Issue #2 to clarify the IKE/IPsec liveness checks.

# Discussion

The impact of solution #13 for key issue #2 is updated.

# Proposal

Add the following changes to TR 23.700-07.

*** BEGIN CHANGES ***

## 6.13	Solution #13: Solution using N3IWF to support service continuity between two networks and paging from both networks

### 6.13.1	Introduction

This solution proposes to use N3IWF architecture to address the key issue #2 service continuity between NPN and PLMN, and paging from both networks.

In this solution the UE has two subscription i.e. one PLMN (aka "PLMN UE" for PLMN parts of the UE) and one SNPN (aka "SNPN UE" for SNPN parts of the UE) subscription. The scenarios described uses PLMN and SNPN, but same principles can apply between SNPNs, if allowed.

### 6.13.2	Functional Description

#### 6.13.2.1	Service Continuity

There is an existing solution in Release 16 to address how to access the service of one network (e.g. PLMN) via the other network (e.g. SNPN), as described in clause 5.30.2.7 and 5.30.2.8 in TS 23.501 [4].

For single radio UE, it provides the basic function to achieve service continuity between PLMN and SNPN, by utilizing the handover procedure between non-3GPP access and 3GPP access described in clause 4.9.2 in TS 23.502 [6], with one network is acting as 3GPP access and the other network is acting as non-3GPP network.

But for single radio UE, it may take long time to complete such handover procedure between PLMN and SNPN, due to the following reasons:

-	UE needs to detect and trigger the procedure to activate/deactivate the SNPN access mode, otherwise UE may not be able to perform the corresponding network selection on the target side.

-	Depends on the handover direction, several extra signaling procedures are required prior to the actual PDU Session establishment procedure which initiates the handover of the PDU session:

-	If the handover target is 3GPP access (i.e. UE accesses the target directly via NG-RAN), UE needs to perform one initial registration procedure.

-	If the handover target is non-3GPP access (i.e. UE accesses the target via N3IWF), UE needs to perform two initial registration procedures (first a 3GPP access registration for the other network followed by non-3GPP access registration) and one PDU session establishment procedure.

For dual radio UE, the UE can use one radio operating in SNPN access mode and the other operating the normal PLMN selection, in order to avoid SNPN access mode switch. Optionally, dual radio UE can prepare a smoother service continuity by:

-	Register to both SNPN and PLMN via separated radio interface if the corresponding network is available.

-	Register to the same 5GC via both Uu and NWu interface and possibly establish MA-PDU session.

NOTE:	When dual radio is described in this solution, only 2Rx/2Tx dual radio UEs are assumed.

Figure 6.13.2.1-1 UPF PSA in SNPN

Figure 6.13.2.1-1 depicts a scenario where the UPF PSA of the PDU session is in SNPN. The dual radio UE uses one radio to register under the SNPN NG-RAN, and the other radio to register under the SNPN N3IWF via the PLMN NG-RAN and 5GC. From SNPN 5GC point of view, the UE is registered under both 3GPP access and non-3GPP access.

When the dual radio UE needs to do mobility, e.g. UE is leaving SNPN NG-RAN radio coverage, the second radio of the UE may have already performed the PLMN selection, cell selection, initial registration and PDU session establishment in PLMN. Then, UE can directly initiate the handover of the PDU session:

-	For single access PDU session, UE shall follow the procedure described in clause 4.9.2 in TS 23.502 [6].

-	For MA-PDU session, UE and UPF could switch the user plane resource to the corresponding access type.

If the UPF PSA of the PDU session is in PLMN, the following Figure 6.13.2.1-2 applies:

Figure 6.13.2.1-2: UPF PSA in PLMN

#### 6.13.2.2	Data services from both networks

The N3IWF solution enables the UE to receive data services from both network at the same time.

For single radio UE, it could register to both PLMN 5GC and SNPN 5GC, with option described in Figure 6.13.2.2-1 or Figure 6.13.2.2-2. But from one specific 5GC point of view, for example SNPN 5GC, the UE could only register to SNPN 5GC either via Uu or via NWu.

For dual radio UE, it could register to the same 5GC via both Uu and NWu interface, as illustrated by Figure 6.13.2.2-3. So that, it could take advantage of the MA-PDU session for data services and use ATSSS to steer traffic to desired interface.

Figure 6.13.2.2-1 single UE connects to PLMN via Uu

Figure 6.13.2.2-2 single radio UE connects to SNPN via Uu

Figure 6.13.2.2-3 Dual radio UE connects to 5GC via both Uu and NWu

#### 6.13.2.3	Paging

For single radio UE, The N3IWF solution described in clause 5.30.2.7 and 5.30.2.8 in TS 23.501 [4] can also be enhanced to address the paging aspect of the key issue.

The underlay network is taking the role of "Untrusted non-3GPP access" of the overlay network. UE is accessing the underlay network service via Uu interface and accessing the overlay network service via NWu interface. The paging aspect of this key issue is to address UE to receive paging from the overlay network.

According to NOTE 3 in clause 5.5.2 in TS 23.501 [4], it is assumed that the UE configured to receive services from the overlay 5GC network that is CM-IDLE over the NWu interface will attempt to establish the NWu connection and transition to CM-CONNECTED state in the overlay network, whenever the UE successfully connects to a non-3GPP access (i.e. the underlay network). So that both UE triggered and network triggered service request procedures in the overlay network can be performed as the UE is in CM-CONNECTED state.

It is proposed to always establish at least one PDU session in the underlay network, in order to simulate the scenario that UE successfully connects to a non-3GPP access of the overlay network, then the UE is expected to stay in CM-CONNECTED state in overlay network.

Existing liveness check procedure described in clause 7.8 and 7.9 in TS 24.502 [10] is also recommended, to avoid any NAT entries in the underlying network to timeout. Detailed procedure could be found in clause 6.13.3.5 and 6.13.3.6.

For dual radio (2Rx/2Tx), the UE listens and is prepared to receive MT data services on both networks.

#### 6.13.2.4	Dual radio UE architecture

A UE able to access both a PLMN and an SNPN has subscription to both networks (SNPN UE and PLMN UE). When a UE is dual radio capable, each radio can be associated to each subscription, e.g. as shown in figure 6.x.2.3-1.

NOTE:	When dual radio is described in this solution, only 2Rx/2Tx dual radio UEs are assumed.

Figure 6.13.2.4-1: Example of a dual radio capable UE

When the UE is dual radio capable, as shown in Figure 6.13.2.4-1, then the SNPN side of the UE (i.e. SNPN UE) can use the PLMN side (PLMN UE) for accessing the network, and the PLMN side of the UE (PLMN UE) can use the SNPN side (SNPN UE) for accessing the network.

NOTE:	The internal architecture of a dual radio UE is up to UE implementation.

### 6.13.3	Procedures

#### 6.13.3.1	Handover of PLMN anchored PDU Session from N3IWF to NG-RAN

Figure 6.13.3.1-1: Handover of PLMN anchored PDU Session from N3IWF to NG-RAN

This procedure assumes the UE has initially registered in SNPN NG-RAN/5GC and established a PDU session in SNPN.

1.	UE performs PLMN non-3GPP access registration via SNPN.

2.	UE establishes the non-3GPP access PDU session in PLMN and the corresponding user plane. This PDU session is anchored in PLMN UPF, and subject to the handover procedure described in the following steps.

3.	For single radio UE, after SNPN UE detects the coverage loss of SNPN NG-RAN, it first needs to activate the PLMN UE and switch its radio interface from SNPN NG-RAN to PLMN NG-RAN, and then performs 3GPP access registration in PLMN.

For dual radio UE, it is possible that this step of 3GPP access registration in PLMN is done at any time prior to the coverage loss of SNPN NG-RAN, if there is PLMN radio coverage available.

If it is MA-PDU session established in step 2, the user plane for 3GPP access could be activated due to the registration procedure.

4.	If it is a single access PDU session, UE performs handover of a PDU Session procedure from non-3GPP to 3GPP access as described in clause 4.9.2.1 in TS 23.502 [6].

If it is a MA-PDU session, UE shall add the user plane resource for 3GPP access by following the procedure in clause 4.22.7 in TS 23.502 [6], if 3GPP access user plane resource is not activated in step 3. Then UE shall trigger the release of non-3GPP access user plane resource by following clause 4.22.10.2 in TS 23.502 [6].

#### 6.13.3.2	Handover of PLMN anchored PDU Session from NG-RAN to N3IWF

Figure 6.13.3.2-1: Handover of PLMN anchored PDU Session from NG-RAN to N3IWF

1.	UE performs 3GPP access registration in PLMN.

2.	UE establishes the 3GPP access PDU session in PLMN and the corresponding user plane. This PDU session is anchored in PLMN UPF, and subject to the handover procedure described in the following steps.

3.	For single radio UE, after PLMN UE detects the coverage loss of PLMN NG-RAN, it first needs to activate the SNPN UE and switch its radio interface from PLMN NG-RAN to SNPN NG-RAN, and then perform initial registration and PDU session establishment with SNPN.

For dual radio UE, it is possible to register in SNPN, via its SNPN UE, and establish the SNPN PDU session whenever the radio coverage of SNPN is available, even prior to the PLMN radio coverage loss.

Once the SNPN PDU session is established, the PLMN UE performs the non-3GPP access registration in PLMN using the SNPN PDU Session connectivity of the SNPN UE.

If it is MA-PDU session established in step 2, the user plane for non-3GPP access could be activated due to the registration procedure.

4.	If it is a single access PDU session, UE performs handover of a PDU Session procedure from 3GPP to non-3GPP access as described in clause 4.9.2.2 in TS 23.502 [6].

If it is a MA-PDU session, UE shall add the user plane resource for non-3GPP access by following the procedure in clause 4.22.7 in TS 23.502 [6], if non-3GPP access user plane resource is not activated in step 3. Then UE shall trigger the release of 3GPP access user plane resource by following clause 4.22.10.2 in TS 23.502 [6].

#### 6.13.3.3	Handover of SNPN anchored PDU Session from N3IWF to NG-RAN

Figure 6.13.3.3-1: Handover of SNPN anchored PDU Session from N3IWF to NG-RAN

This procedure assumes the UE has initially registered in PLMN NG-RAN/5GC and established a PDU session in PLMN.

1.	UE performs SNPN non-3GPP access registration via PLMN.

2.	UE establishes the non-3GPP access PDU session in SNPN and the corresponding user plane. This PDU session is anchored in SNPN UPF, and subject to the handover procedure described in the following steps.

3.	For single radio UE, after PLMN UE detects the coverage loss of PLMN NG-RAN, it first needs to activate the SNPN UE and switch its radio interface from PLMN NG-RAN to SNPN NG-RAN, and then performs 3GPP access registration in SNPN.

For dual radio UE, it is possible that this step of 3GPP access registration in SNPN is done at any time prior to the coverage loss of PLMN NG-RAN, if there is SNPN radio coverage available.

If it is MA-PDU session established in step 2, the user plane for 3GPP access could be activated due to the registration procedure.

4.	If it is a single access PDU session, UE performs handover of a PDU Session procedure from non-3GPP to 3GPP access as described in clause 4.9.2.1 in TS 23.502 [6].

If it is a MA-PDU session, UE shall add the user plane resource for 3GPP access by following the procedure in clause 4.22.7 in TS 23.502 [6], if 3GPP access user plane resource is not activated in step 3. Then UE shall trigger the release of non-3GPP access user plane resource by following clause 4.22.10.2 in TS 23.502 [6].

#### 6.13.3.4	Handover of SNPN anchored PDU Session from NG-RAN to N3IWF

Figure 6.13.3.4-1: Handover of SNPN anchored PDU Session from NG-RAN to N3IWF

1.	UE performs 3GPP access registration in SNPN.

2.	UE establishes the 3GPP access PDU session in SNPN and the corresponding user plane. This PDU session is anchored in SNPN UPF, and subject to the handover procedure described in the following steps.

3.	For single radio UE, after SNPN UE detects the coverage loss of SNPN NG-RAN, it first needs to activate the PLMN UE and switch its radio interface from SNPN NG-RAN to PLMN NG-RAN, and then perform initial registration and PDU session establishment with PLMN.

For dual radio UE, it is possible to register in PLMN, via its PLMN UE, and establish the PLMN PDU session whenever the radio coverage of PLMN is available, even prior to the SNPN radio coverage loss.

Once the PLMN PDU session is established, the SNPN UE performs the non-3GPP access registration in SNPN using the SNPN PDU Session connectivity of the PLMN UE.

If it is MA-PDU session established in step 2, the user plane for non-3GPP access could be activated due to the registration procedure.

4.	If it is a single access PDU session, UE performs handover of a PDU Session procedure from 3GPP to non-3GPP access as described in clause 4.9.2.2 in TS 23.502 [6].

If it is a MA-PDU session, UE shall add the user plane resource for non-3GPP access by following the procedure in clause 4.22.7 in TS 23.502 [6], if non-3GPP access user plane resource is not activated in step 3. Then UE shall trigger the release of 3GPP access user plane resource by following clause 4.22.10.2 in TS 23.502 [6].

#### 6.13.3.5	UE initiated liveness check

NOTE 1:	This clause is a summary from existing TS 24.502 clause 7.3.2.2 and clause 7.8.

Figure 6.13.3.5-1 UE obtains liveness check timeout value from N3IWF and initiates liveness check

1.	During the IKE SA and Signaling IP Sec SA establishment(e.g. within the non-3GPP registration procedure), UE indicates its support of liveness check by including the TIMEOUT_PERIOD_FOR_LIVENESS_CHECK attribute in the CFG_REQUEST payload in the IKE_AUTH request message.

2.	N3IWF includes the TIMEOUT_PERIOD_FOR_LIVENESS_CHECK attribute in the CFG_REPLY payload in the IKE_AUTH response message, and then UE set its liveness check timeout value according to the TIMEOUT_PERIOD_FOR_LIVENESS_CHECK attribute value in the IKE_AUTH response message from N3IWF.

3.	Once UE has not received any cryptographically protected IKEv2 or IP Sec message for the duration of the timeout period, UE initiates the liveness check by sending a INFORMATIONAL Request message to N3IWF. N3IWF shall respond with INFORMATIONAL Response message to UE.

NOTE 2:	In case UE does not support TIMEOUT_PERIOD_FOR_LIVENESS_CHECK attribute, or the TIMEOUT_PERIOD_FOR_LIVENESS_CHECK attribute is not sent by N3IWF in the IKE_AUTH response message, UE shall use a pre-configured timeout value to perform liveness check. The pre-configured timeout value is implementation specific.

#### 6.13.3.6	Network initiated liveness check

NOTE:	This clause is a summary from existing TS 24.502 clause 7.9.

Figure 6.13.3.6-1 Network initiated liveness check

1.	N3IWF shall send an INFORMATIONAL request message to UE, if N3IWF has not received cryptographically protected IKEv2 or IPsec message from UE for the duration of the timeout period for liveness check selected according to the local policy.

2.	UE shall reply with INFORMATIONAL response message to N3IWF.

### 6.13.4	Impacts on services, entities and interfaces

UE:

-	Optionally support for Dual radio capability to be able to connect to both networks via Uu interface and achieve seamless continuity.

-	Support keeping one PDU session in underlay network, in order to simulate "successfully connected to the non-3GPP access network", where the underlay network is acting the non-3GPP access network.

-	Support establishing MA-PDU session where the non-3GPP access is simulated by a 3GPP access.

UE and N3IWF:

-	Liveness check procedure shall be used over NWu interface, either triggered from UE or N3IWF as described by TS 24.502 clause 7.3.2.2, 7.8 and 7.9.

*** END CHANGES ***