3GPP TSG-SA WG2 Meeting #140E 	S2-2005589

19 August – 2 September, 2020, Elbonia	 (revision of S2-200xxxx)

Source:	CATT

Title:	KI #3, Sol#22: Update the solution to add SMBR event notification procedure

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: this paper proposes to update solution #22 in TR 23.700-40.

Discussion

This is proposed to update solution#22 with following aspects:

- Add a new procedure to support SMBR event notify.

- Update the impacts clause.

Proposal

It is proposed to update solution #22.

First change

6.22.3.x	SMBR event notification

Figure 6.22.3.x-1 SMBR event notification

1.	The PCF sends Namf_EventExposure_Subscribe to AMF to subscribe the event that the SMBR of UE reaches a threshold.

2.	The AMF sends N2 message to RAN to subscribe the event that the SMBR of UE reaches a threshold.

3.	The RAN detects that the SMBR reaches the threshold.

4.	The RAN sends N2 message including S-NSSAI and indication of SMBR threshold reached to AMF.

5.	The AMF sends Namf_EventExposure_Notify (S-NSSAI, SUPI, SMBR threshold reached) to PCF.

6.	The PCF decides to update UE policy to reduce the network slice traffic via 3GPP access, e.g. steer network slice traffic to N3GPP access by setting the N3GPP access as preferred access type.

7.	The PCF triggers UE Configuration Update procedure in clause 4.2.4.3 in TS 23.502 to sends the UE policy container to the UE.

Second change

6.22.4	Impacts on services, entities and interfaces

AMF:

-	Support Subscribed SMBR transfer during initial registration via Nudm_SDM Service.

-	Support SMBR transfer via N2 interface and SMBR enforcement.

-	Support SMBR event notify.

UDM:

-	Support Subscribed SMBR transfer during initial registration via Nudm_SDM Service.

PCF:

-	FFS: Support authorized SMBR transfer via Npcf_AMPolicyControl Service.

-	Support SMBR event notify and update UE Policy when SMBR reaches a threshold.

RAN:

-	Support SMBR transfer via N2 interface and SMBR enforcement.

-	Support SMBR event notify when the SMBR reaches a threshold.

End of changes