3GPP TSG-WG SA2 Meeting #140E e-meeting	S2-2005645

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#4, New Sol: Negotiated UE-to-UE Relay Reselection

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract: This contribution introduces a solution for UE-to-UE Relay reselection.

# 1. Introduction/Discussion

In clause 5.4.1, for KI#4 Support of UE-to-UE Relay, the UE-to-UE Relay change aspect is to be studied

-	How to (re)-select a UE-to-UE Relay UE in proximity?

-	How to provide a mechanism for path changing in case of e.g. UE-to-UE Relay changes?

This solution provides a solution for UE-to-UE Relay reselection. Since the solution is not dependent on how does the relay forward traffic between Source UE and Target UE, this solution can be applied for both L2-based relay and L3-based relay mechanisms.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.752.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  | X |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X | X |  |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |  |
| 6 |  |  | X |  |  |  |  |  |
| 7 |  |  | X |  |  |  |  | X |
| 8 |  |  |  | X |  |  |  |  |
| 9 |  |  |  | X |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  |  | X |  |  |  |
| 13 |  |  |  |  |  |  | X |  |
| 14 |  |  |  |  |  |  | X |  |
| 15 |  |  |  |  |  |  | X |  |
| 16 |  |  |  |  |  |  |  | X |
| 17 |  |  |  |  |  |  |  | X |
| 18 | X |  |  |  |  |  | X |  |
| 19 | X |  | X |  |  |  |  |  |
| 20 |  | X |  |  |  |  |  |  |
| 21 |  | X |  |  |  |  |  |  |
| 22 | X | X |  |  |  |  |  |  |
| 23 |  |  | X |  |  |  |  |  |
| 24 |  |  | X |  |  |  |  |  |
| 25 |  |  | X |  |  |  |  |  |
| 26 |  |  | X |  |  |  |  |  |
| 27 |  |  | X |  |  |  |  |  |
| 28 | X |  | X |  |  |  |  |  |
| 29 |  |  | X |  |  |  |  |  |
| 30 |  |  | X |  |  |  |  | X |
| 31 |  |  |  | X |  |  |  |  |
| 32 |  |  |  | X |  |  |  |  |
| 33 | X |  |  | X |  |  |  |  |
| 34 |  |  |  |  |  |  | X |  |
| 35 |  |  | X |  |  |  |  | X |
| 36 |  |  |  | X |  |  |  | X |
| 37 |  | X |  |  |  |  |  |  |
| 38 |  |  | X |  |  |  |  |  |
| X |  |  |  | X |  |  |  |  |

# * * * * Second change (all new) * * * *

## 6.X	Solution #X: Negotiated UE-to-UE Relay Reselection

### 6.X.1	Description

This solution provides a solution for UE-to-UE Relay reselection in Key Issue #4. Since the solution is independent of how the relay forwards traffic between Source UE and Target UE this solution is applicable to both Layer 2 UE-to-UE Relays and Layer 3 UE-to-UE Relays.

Solution#8 provides a solution for UE-to-UE Relay Selection, and this solution is suitable for initial Relay selection and it could be extended for UE-to-UE Relay reselection. However, it will cause the relay UE in proximity broadcast the relay discovery message. Under some cases, these broadcast messages can be avoided, by Source UE and Target UE negotiating the relay reselection using the existing relay connection.

After the connection between Source UE and Target UE is setup, the Source UE and Target UE may receive relay discovery messages from the other Relay UEs. The Source UE or Target UE may find that the signal quality with other Relay UEs is better than that with the currently used Relay UE. Alternatively if the Source UE or Target UE finds that the signal quality with the selected Relay UE is not good enough, it will initiate the discover message to find the candidate Relay UEs which can provide a better connection. After new candidate Relay UEs have been identified, the Source UE or Target UE will initiate the relay reselection procedure and then these two UEs can negotiate Relay reselection using the existing relay connection.

### 6.X.2	Procedures

Figure 6.X.2-1: UE-to-UE Relay Reselection Procedure

1.	Connection between Source UE and Target UE via Relay 1 is setup by using the other solutions such as Solution #9 and Solution #10, and Relay 1 is selected as UE-to-UE Relay, such as by using the Solution #8.

2.	The Source UE decides to perform Relay UE reselection. This may be triggered by receiving the relay discovery message from another Relay UE, and the signal quality with this Relay UE is better than that with Relay 1. Alternatively the Source UE will initiate the discover message to find candidate Relay UEs which can provide a better connection, when it finds that the signal quality with Relay 1 is not good enough.

3.	After the Source UE identifies the candidate Relay UEs, the Source UE sends the Relay UE reselection request to the Target UE using the connection via Relay 1, and the request message includes the candidate Relay UE ID(s).

4.	The Target UE decides to change from Relay 1 to a new Relay UE. The new Relay UE is chosen from the candidate Relay UE ID(s) included in the reselection request. This decision can be based on a new Relay UE providing the best signal quality. If the Target UE has not received a relay discovery message from a candidate Relay UE or does not connect to the candidate Relay UE, the Target UE may perform the Relay UE discovery procedure with a candidate Relay UE ID in discovery message.

5.	The Target UE sends a response to the Source UE via Relay 1 that includes the new Relay ID. If no new Relay UE is chosen, the Target UE may not respond to Source UE or send a response indicating Relay reselection failure.

6.	[Optional] If the Target UE choses a new Relay UE in step 5, the Source UE initiates the connection setup procedure via the new Relay UE, and also releases the connection via Relay 1.

### 6.X.3	Impacts on services, entities and interfaces

Source UE:

-	Identifies the candidate Relay UEs, and initiates Relay reselection via the existing connection.

Target UE:

-	Decides the new Relay UE.

# * * * * End of changes * * * *