SA WG2 Meeting #140E (e-meeting)	S2-2005680

August 19 – September 1, 2020, Elbonia	(revision of S2-200xxxx)

Source:	Samsung

Title:	KI #3, Evaluation and Interim conclusion

Document for:	Approval

Agenda Item:	8.10 Study on system enablers for multi-USIM devices

Work Item / Release:	FS_MUSIM / Rel-17

# Background

This paper proposes evaluations and interim conclusion for KI#3.

# Proposal

It is proposed to capture the following proposal to TR 23.761.

* * * * Start of 1st Change * * * *

# 7	Evaluation

Evaluation for KI#3:

-	Total 4 solutions are available for KI#3 i.e. solution #4 #5, #6 and #22.

-	Solution #6 and #22 have E-UTRA impact. But as per RAN work item E-UTRA impacts are not allowed.

-	Solution #4 and #5,

-	for AS signaling solution, there are E-UTRA impact.

-	for NAS signaling solution, there are no E-UTRA impacts.

-	Solution having RAN impact are not applicable for EPS, because as per RAN work item E-UTRA impacts are not applicable. Thus we have only solution#4 and solution#5 (NAS based) applicable for EPS.

-	To have a common solution between 5GS and EPS, and to have a graceful release we propose NAS based solution as a way forward for both EPC and 5GC following sol#5 principles. On top of this for any other potential use case if RAN group deems necessary they can develop a new solution applicable only to 5GC as part of RAN WI.

* * * * Start of 2nd Change * * * *

# 8	Conclusions

Interim Conclusion for KI#3:

-	NAS based solution as described in Solution #5 is selected as a baseline solution for normative work for both 5GS and EPS.

* * * * End of Change * * * *