SA WG2 Meeting #140E	S2-2005683

19 August – 02 September, 2020

Source:	Samsung

Title:	KI#2: Interim Evaluation - Mobility / Reselection Aspects

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2/ Rel-17

Abstract of the contribution: This contribution proposes an interim evaluation for KI#2 regarding mobility/reselection aspects.

1. Introduction

Solutions #10 and #26 in the TR 23.700-91 address the mobility/ reselection aspect of KI #2 in the multiple NWDAF deployment scenario:

-	Solution #10: Handling of mixed and distributed NWDAF deployments

- 	Solution #26: Reselection of NWDAF

This contribution paper proposes an interim evaluation for the mobility/ reselection aspect of KI#2.

2. Text Proposal

It is proposed to adopt the following text within the TR 23.700-91.

*** Start of the change (all new text) ***

7.	Overall Evaluation

Editor's note: This clause will provide evaluation of different solutions.

7.x  Key Issue #2

7.x.x	Mobility/Reselection of NWDAF

Solutions #10 and #26 address the mobility/ reselection aspect of KI #2 in the multiple NWDAF deployment scenario:

-	Solution #10: Handling of mixed and distributed NWDAF deployments

- 	Solution #26: Reselection of NWDAF

Solution #10 defines a set of procedures to support change of the serving NWDAF instance when the change of NWDAF instance is triggered by the NF reallocation. When a new NWDAF instance is selected due to the AMF relocation, the new selected NWDAF may retrieve the analytics data and collected input data from the old serving NWDAF. The solution also defines a procedure which supports updating NWDAF service consumers with information of a new NWDAF instance.

Solution #26 defines a procedure to support change of the serving NWDAF instance when the change of NWDAF instance is triggered by the old serving NWDAF. The old serving NWDAF may select a new NWDAF instance based on NF profile received from the NRF. The new selected NWDAF may retrieve the analytics data and collected input data from the old serving NWDAF.

Solution #10 and #26 have commonality in that analytics data and input data collected by the old serving NWDAF can be transferred from the old serving NWDAF to the new target NWDAF. They can be adopted to cover different scenarios of mobility/reselection of NWDAF.

### *** End of the change ***