3GPP TSG-WG SA2 Meeting #140E e-meeting  	S2-2005836

Elbonia, August 19 – September 02, 2020

Source:	Lenovo, Motorola Mobility

Title:	KI#2, Way forward on KI#2

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution proposes the way forward on evaluation and conclusion of Key Issue #2: Edge relocation.

- Discussion on how to evaluate and conclude KI#2

According to the mapping table of the Solutions to Key Issues in the clause 6, there are 19 solutions for the KI#2 captured in the 3GPP TR 23.748. However, there are overlaps for some of the solutions. It's important to identify the sub-issues targeted by different solutions, and then evaluate the pro/cros of comparable solutions targeting same sub-issues.

Generally, solutions for KI#2 can be classified into the following three categories:

A)	Solutions for edge application server relocation triggered by CN(e.g. PSA relocation) or application, addressing the following sub-issues:

A1) Reducing packet loss for edge application server relocation;

A2) Connecting the UE with the relocated edge application server.

B)	Solutions for PSA relocation and UE IP relocation triggered by CN or application.

Proposal 1: Evaluation on each category of solutions can be discussed and compared together, to decide the conclusions for each sub-issues as the base for the normative work.

The following clauses propose how to evaluate and conclude for each of the above categories.

1.1 Solutions for edge application server relocation

For A solutions, there are 15 solutions.

1） The solutions for A1) Reducing packet loss for edge application server relocation are listed in the Table 1.1-1.

Table 1.1-1: Solutions for A1) Reducing packet loss for edge application server relocation

| NO# | Solutions | Solution Summary |
| --- | --- | --- |
| 1 | #27: Reducing packet loss during EAS relocation | The AF provides an indication to indicate the support of seamless edge application relocation and the need to buffer the uplink data in the local PSA. |
| 2 | #38: EAS change with reducing packet loss in uplink | 2 options: 1)Establish a forwarding tunnel between source PSA and target PSA to avoid loss of uplink packets 2)Introduce an AF-defined Flow End Maker between UL CL and source local DN/EAS to trigger EAS relocation. |

Proposal 2: Conclusion and evaluation on each A1 solution can be discussed together, to decide whether the above sub-issue need to be supported and whether the solution can be selected as baseline for the sub-issue.

- The A2 solutions can be classified more finely with the following sub-categories:

A2-1) UE aware Edge application server relocation

A2-2) UE unaware Edge application server relocation

A2-3) Using MPTCP mechanism for edge application server relocation

A2-4) NEF caching application status information for edge application server relocation

A2-5) To pre-flush the DNS cache for inactive connection."

A2-6) Using ICMP mechanism triggering new DNS query while edge application server relocation

A2-7) Using Anycast for stateful application relocation

A2-8) Multiple PDU sessions

All the A2 solutions are listed in the Table 1.1-2

Table 1.1-2: Solutions for A2) Connecting the UE with the relocated edge application server

| NO# | Solutions | Sub-class | Solution Summary |
| --- | --- | --- | --- |
| 3 | #28: Supporting application server change based on AF notification | A2-1) UE aware Edge application server relocation | SMF subscribes and is notified of the EAS relocation notification, and the SMF sends the changed IP address to the UE via NAS PCO. |
| 4 | #31: Application Relocation with UE assistance | A2-1) UE aware Edge application server relocation | AF provides either EAS IP address or indication for application server relocation, and FQDN to UE via SMF and indicates UE to suspend or resume the uplink traffic via SMF |
| 5 | #34: Local DN notification to the UE during ULCL operations | A2-1) UE aware Edge application server relocation | The local DN notification and the local DN binding context control information related with the local DN to the UE are send to the UE during local PSA insertion/relocation/removal. The local DN binding context control information includes Layer 4 context refresh indicator and DNS cache refresh indicator for the removed local DN (e.g. local DN 1 identified by DNAI-A). |
| 6 | #37: AF-based EAS End-Point-Address update via External Parameter Provisioning | A2-1) UE aware Edge application server relocation | The PCF gets updated EAS End-Point Address information from AF when Applications are migrated, or End-Point address are re-allocated. The AF communicates this to the PCF via a NEF Specific Parameter Provisioning Service. |
| 7 | #29: CN-based edge relocation | A2-2) UE unaware Edge application server relocation | CN gets information for the edge application server relocation at edge relocation and performs application server IP address replacement at edge application server relocation. |
| 8 | #30: UE Agnostic EAS IP address replacement for traffic subject to edge computing | A2-2) UE unaware Edge application server relocation | 0. Anchor EAS is configured. 1. CN gets information for the edge application server relocation at edge relocation and .CN performs application server IP address replacement at edge application server relocation. |
| 9 | #39: EAS relocation coordinated with PSA change | A2-3) Using MPTCP mechanism for edge application server relocation | Re-using some concepts of the MA-PDU session and path-switching mechanism based on MPTCP with Change MPTCP subflow's priority using MP_PRIO option B=0 to activate the MPTCP subflow between source PSA and target PSA. |
| 10 | #40: Seamless change of Edge Application Sever for stateful applications by caching application status information in NEF | A2-4) NEF caching application status information for edge application server relocation | Caching application status information in NEF triggered by early notificaiton and sending the application status infromation to the T-AF triggered by late notification |
| 11 | #32: UE DNS cache flush | A2-5) To pre-flush the DNS cache for inactive connection. | DNS flush When DNAI changes, i.e. the ULCL is changed or removed, the SMF sends a DNS flush indication to UE. This DNS flush indication tells UE to remove the cached DNS records. The active connection between the UE and the EAS is not impacted. |
| 12 | #24: Support of edge relocation, triggering of new DNS query by the UE | A2-6) Using ICMP mechanism triggering new DNS query while edge application server relocation | SMF or UPF generates an ICMP Destination unreachable message with code 3 (Port Unreachable) or 7 (destination host unknown). To trigger the new DNS query. |
| 13 | #25: Seamless Change of Edge for Stateful Applications | A2-7) Using Anycast for stateful application relocation | Using Anycast and application context: 1)5GC Assisted procedure 2)Application Domain aware procedure |
| 14 | #21: Provisioning URSP configuration to the UE to establish PDU Sessions for edge applications based on Provisioning Domains | A2-8) Multiple PDU sessions | For multiple PDU sessions: Provisioning Domains (PvDs) may be configure on UEs by PCF through enhanced URSP rules |
| 15 | #23: DNS for AS Discovery at Edge Relocation | A2-8) Multiple PDU sessions | Use DNS as a solution for new Edge Application Server Discovery at Edge Relocation for Distributed Anchor for both, SSC#3 and SSC#2 modes. |

Proposal 3: All the A2-1), A2-2), A2-3), A2-4), A2-5), A2-6) and A2-7) solutions addressing the session breakout model for A2) Connecting the UE with the relocated edge application server. One or more A2-x sub-class solutions should be chosen as baseline for normative work.

Proposal 4: Each solution of sub-classes with multiple alternative solutions (including A2-1), A2-1) and A2-8)) should be discussed together, to decide whether the solution or merged solution can be selected as baseline for the related sub-issue.

1.2 Solutions for PSA relocation

B solutions are listed in the Table 1.2-1.

Table 1.2-1: Solutions for PSA/UE IP relocation

| NO# | Solutions | Sub-class | Solution Summary |
| --- | --- | --- | --- |
| 16 | #33: IP preserving PSA relocation | Multiple PDU sessions | Two PDU Sessions are used temporarily with the same UE IP address. |
| 17 | #35: Edge relocation considering with user plane latency requirement (SMF decision) | PSA relocation considering user plane latency | The SMF decides to re-select UPF based on the user plane latency requirements received from the AF: 1)Maximum allowed user plane latency 2)Shortest user plane latency preference |
| 18 | #36: Edge relocation considering with user plane latency requirement (AF decision) | PSA relocation considering user plane latency | AF decides relocation based on the estimated user plane latency notified by SMF with PSA-UPF relocation. |
| 19 | #26: Persistent address allocation for mobile UEs that need MEC access | UE IP relocation: Using persistent UE IP address to avoid UE IP relocation | Allocate persistent IP addresses to UEs that need MEC access, then the problem of UE IP address change no longer exists |

Proposal 5: Conclusion and evaluation on each B solution can be discussed together, to decide whether the solution can be selected as baseline for the sub-issue.

2	Proposals

Proposal 1: Evaluation on each category of solutions can be discussed and compared together, to decide the conclusions for each sub-issues as the base for the normative work.

Proposal 2: Conclusion and evaluation on each A1 solution can be discussed together, to decide whether the above sub-issue need to be supported and whether the solution can be selected as baseline for the sub-issue.

Proposal 3: All the A2-1), A2-2), A2-3), A2-4), A2-5), A2-6) and A2-7) solutions addressing the session breakout model for A2) Connecting the UE with the relocated edge application server. One or more A2-x sub-class solutions should be chosen as baseline for normative work.

Proposal 4: Each solution of sub-classes with multiple alternative solutions (including A2-1), A2-1) and A2-8)) should be discussed together, to decide whether the solution or merged solution can be selected as baseline for the related sub-issue.

Proposal 5: Conclusion and evaluation on each B solution can be discussed together, to decide whether the solution can be selected as baseline for the sub-issue.