3GPP TSG-SA WG2 Meeting #140e	S2-2005846

# August 19 – September 02, 2020, Online	(revision of S2-200xxyxy)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2454 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying PSFP support for 5GS bridge |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical LAN |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 5.28.3.1 currently states "The use of PSFP information is mandatory at the TSN AF and is optional at both DS-TT and NW-TT." However, such partial support is not possible as IEEE 802.1Q assumes support for PSFP on a per bridge granularity. Furthermore, 5GS needs to indicate support of PSFP to CNC because CNC will only provide PSFP information to supporting bridges. PSFP information in turn is a prerequisite to enable the 5GS TSN functionality as specified in Rel-16, e.g. - deriving TSC Assistance Information (TSCAI) as defined in clause 5.27.2; - deriving service data flows and QoS information for TSN flows as defined in clause 5.28.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace " The use of PSFP information is mandatory at the TSN AF and is optional at both DS-TT and NW-TT " by " TSN AF only reports an NW-TT or DS-TT port to CNC as part of the bridge information and bridge capabilities defined in clause 5.28.2 if PSFP is supported by NW-TT or DS-TT, respectively. DS-TT indicates support of PSFP using port management capabilities, i.e. by indicating support for the Per-Stream Filtering and Policing information." |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSCAI information cannot be derived and uplink QoS flows cannot be derived for TSN flows if TSN AF does not indicate PSFP support for the 5GS bridge. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

#### 5.28.3.1	General

Port and bridge management information is exchanged between CNC and TSN AF. The port management information, is related to Ethernet ports located in DS-TT or NW-TT.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF and DS-TT or NW-TT, respectively inside a Port Management Information Container. NW-TT may support one or more ports. In this case, each port uses separate Port Management Information Container. 5GS shall also support transfer of standardized and deployment-specific bridge management information transparently between TSN AF and NW-TT, respectively inside a Bridge Management Information Container. Table 5.28.3.1-1 and Table 5.28.3.1-2 list standardized port management information and bridge management information, respectively.

Table 5.28.3.1-1: Standardized port management information

| Port management information | Applicability (see Note 6) |  | Supported operations by TSN AF (see Note 1) | Reference |
| --- | --- | --- | --- | --- |
|  | DS-TT | NW-TT |  |  |
| General |  |  |  |  |
| Port management capabilities (see Note 2) | X | X | R |  |
| Bridge delay related information |  |  |  |  |
| txPropagationDelay | X | X | R | IEEE 802.1Qcc [95] clause 12.32.2.1 |
| Traffic class related information |  |  |  |  |
| Traffic class table | X | X | RW | IEEE 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |
| GateEnabled | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminBaseTime | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminControlList | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminCycleTime (see Note 3) | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminControlListLength (see Note 3) | X | X | RW | IEEE 802.1Q [98] Table 12-28 |
| Tick granularity | X | X | R | IEEE 802.1Q [98] Table 12-29 |
| General Neighbor discovery configuration (NOTE 4) |  |  |  |  |
| adminStatus | D | X | RW | IEEE 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | D | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | D | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | D | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | D | X | RW | IEEE 802.1AB [97] Table 11-2 |
| NW-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X | RW | IEEE 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype | D |  | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId | D |  | RW | IEEE 802.1AB [97] Table 11-2 |
| Neighbor discovery information for each discovered neighbor of NW-TT |  |  |  |  |
| lldpV2RemChassisIdSubtype |  | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId |  | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype |  | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortId |  | X | R | IEEE 802.1AB [97] Table 11-2 |
| TTL |  | X | R | IEEE 802.1AB [97] clause 8.5.4 |
| Neighbor discovery information for each discovered neighbor of DS-TT (NOTE 5) |  |  |  |  |
| lldpV2RemChassisIdSubtype | D |  | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | D |  | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | D |  | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | D |  | R | IEEE 802.1AB [97] Table 11-2 |
| TTL | D |  | R | IEEE 802.1AB [97] clause 8.5.4.1 |
| Per-Stream Filtering and Policing information (NOTE 10) |  |  |  |  |
| Stream Filter Instance Table (NOTE 8) |  |  |  | IEEE 802.1Q [98] Table 12-32 |
| StreamHandleSpec | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| PrioritySpec | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| StreamGateInstanceID | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| Stream Gate Instance Table (NOTE 9) |  |  |  | IEEE 802.1Q [98] Table 12-33 |
| StreamGateInstance | X | X | R | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminBaseTime | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminControlList | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminCycleTime | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPTickGranularity | X | X | R | IEEE 802.1Q [98] Table 12-33 |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. NOTE 4: If DS-TT supports neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to NW-TT using the Bridge Management Information Container (refer to Table 5.28.3.1-2) and NW-TT performs neighbor discovery on behalf on DS-TT. NOTE 5: If DS-TT supports neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from DS-TT. If DS-TT does not support neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from NW-TT, using the Bridge Management Information Container (refer to Table 5.28.3.1-2), the NW-TT performing neighbor discovery on behalf on DS-TT. NOTE 6: X = applicable; D = applicable when validation and generation of LLDP frames is processed at the DS-TT. NOTE 7: Void. NOTE 8: There is a Stream Filter Instance Table per Stream. NOTE 9: There is a Stream Gate Instance Table per Gate. NOTE 10: TSN AF only reports an NW-TT or DS-TT port to CNC as part of the bridge information and bridge capabilities defined in clause 5.28.2 if PSFP is supported by NW-TT or DS-TT, respectively. DS-TT indicates support of PSFP using port management capabilities, i.e. by indicating support for the Per-Stream Filtering and Policing information.TSN AF uses the PSFP information at TSN bridge configuration time to identify the DS-TT MAC address of the PDU Session as described in clause 5.28.2 and for determination of the traffic pattern information as described in Annex I. The PSFP information can be used at the DS-TT (if supported) and at the NW-TT (if supported) for the purpose of per-stream filtering and policing as defined in IEEE 802.1Q [98] clause 8.6.5.1. |  |  |  |  |

Table 5.28.3.1-2: Standardized bridge management information

| Bridge management information | Supported operations by TSN AF (see NOTE 1) | Reference |
| --- | --- | --- |
| Information for 5GS Bridge |  |  |
| Bridge Address | R |  |
| Bridge Name | R |  |
| Bridge ID | R |  |
| Topology of 5GS Bridge |  |  |
| Chassis ID subtype and Chassis ID of the 5GS Bridge | R | IEEE 802.1AB [97] |
| Traffic forwarding information |  |  |
| Static Filtering Entry (NOTE 3) | RW | IEEE 802.1Q [98] clause 8.8.1 |
| General Neighbor discovery configuration (NOTE 2) |  |  |
| adminStatus | RW | IEEE 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | RW | IEEE 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration for DS-TT ports (NOTE 4) |  |  |
| >DS-TT port neighbor discovery configuration for each DS-TT port |  |  |
| >> DS-TT port number | RW |  |
| >> lldpV2LocPortIdSubtype | RW | IEEE 802.1AB [97] Table 11-2 |
| >> lldpV2LocPortId | RW | IEEE 802.1AB [97] Table 11-2 |
| Discovered neighbor information for DS-TT ports (NOTE 4) |  |  |
| >Discovered neighbor information for each DS-TT port (NOTE 4) |  |  |
| >> DS-TT port number | R |  |
| >> lldpV2RemChassisIdSubtype | R | IEEE 802.1AB [97] Table 11-2 |
| >> lldpV2RemChassisId | R | IEEE 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortIdSubtype | R | IEEE 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortId | R | IEEE 802.1AB [97] Table 11-2 |
| >> TTL | R | IEEE 802.1AB [97] clause 8.5.4.1 |
| Stream Parameters |  |  |
| Maximum number of filters, which defines the maximum number of streams that the bridge can handle | R | IEEE 802.1Q [98] |
| Maximum number of gates, which can be equal or less than the maximum number of filters | R | IEEE 802.1Q [98] |
| Maximum number of meters (optional) if measurements are required | R | IEEE 802.1Q [98] |
| Maximum length of the PSFPAdminControlList parameter that can be handled |  | IEEE 802.1Q [98] |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: General neighbor discovery information is included only when NW-TT performs neighbor discovery on behalf of DS-TT. NOTE 3: If the Static Filtering Entry information is present, NW-TT uses Static Filtering Entry information to determine the NW-TT egress port for forwarding UL TSC traffic. If the Static Filtering Entry information is not present, then the forwarding information as in clause 5.8.2.5.3 applies. NOTE 4: DS-TT discovery configuration and DS-TT discovery information are used only when DS-TT does not support LLDP and NW-TT performs neighbor discovery on behalf of DS-TT. These IEs are deliverered via the procedures for the PDU session for the DS-TT port, while the other IEs of the table are deliverered via the procedures for any of the PDU sessions of the 5GS TSN bridge. |  |  |

Exchange of port and bridge management information between TSN AF and NW-TT or DS-TT allows TSN AF to:

1)	retrieve port management information for a DS-TT or NW-TT Ethernet port or bridge management information for a 5GS TSN bridge;

2)	send port management information for a DS-TT or NW-TT Ethernet port or bridge management information for a 5GS TSN bridge;

3)	subscribe to and receive notifications if specific port management information for a DS-TT or NW-TT Ethernet port changes or bridge management information changes.

Exchange of port management information between TSN AF and NW-TT or DS-TT is initiated by DS-TT or NW-TT to:

-	notify TSN AF if port management information has changed that TSN AF has subscribed for.

Exchange of bridge management information between TSN AF and NW-TT is initiated by NW-TT to:

-	notify TSN AF if bridge management information has changed that TSN AF has subscribed for.

Exchange of port management information is initiated by DS-TT to:

-	provide port management capabilities, i.e. provide information indicating which standardized and deployment-specific port management information is supported by DS-TT.

TSN AF indicates inside the Port Management Information Container or Bridge Management Information Container whether it wants to retrieve or send port or bridge management information or intends to (un-)subscribe for notifications.

<<< End of changes >>>