SA WG2 Meeting SA2-140e	S2-2005926

19 August – 01 September, 2020, E-meeting	(revision of S2-2005094r07)

Source:	Intel

Title:	KI #1, Sol #1 – Solution update to address ENs

Document for:	Discussion / Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: Updates Sol#1 to address ENs.

# 1	Discussion

Solution #1 has the following EN:

Editor's note:	The different cases when the SP credentials are PLMN credentials and the case when the SP credentials are SNPN credentials need to be individually addressed in detail.

This EN calls for restructuring of the solution decription into two different cases:

-	Case where SP = PLMN and

-	Case where SP = SNPN.

This pCR proposes to restructure the solution description along this split providing separate architecture figures for the two cases.

Proposal 1: Restructure the text along the split where SP = SNPN and SP = PLMN. These changes are made throughout the pCR.

In SA2#139E all the reference point names for interfaces crossing the administrative boundaries in 6.1.1-1 and 6.2.1-1 have been replaced with “TBD” by a group of operators, the motivation for which we fail to understand even today.

We assume that the concerns may be related to th case where SP=PLMN, but may not be related to the case where SP=SNPN. It is therefore proposed to re-instaure the original interface names in the architecture

Proposal 2: Keep “TBD” on reference points crossing administrative boundaries for the case where SP=PLMN, but revert to existing reference point names for the case where SP=SNPN.

Regarding the EN on network selection we assume that the concern about service requirements (SA1) and feasibility (CT1) was related to the case where SP=PLMN, because for the case where SP=SNPN the configured lists that steer the network selection are homogeneous:

Editor's note:	SA WG1 needs to clarify the service requirements for the solution and CT WG1 needs to confirm the solution and therefore the network selection proposal in this solution is FFS.

It is therefore proposed to re-word the EN on network selection so that it refers to the case where SP=¨PLMN.

Proposal 3: Re-word the EN on network selection so that it refers to the case where SP=PLMN.

Regarding the EN on service continuity we propose to replace it with a NOTE identical to a related NOTE in Solution #2:

Editor's note:	Regarding service continuity: As non-3GPP credentials are limited to isolated networks per service requirement this clause needs to be updated accordingly TS 22.261 [2], clause 8.3.

The related NOTE in Solution #2 reads:

NOTE:	Service continuity even between SNPN requires 3GPP credentials being used as usage of non-3GPP credentials in this release is per service requirement for isolated networks only. Session establishment and continuity with an Home SP requires 3GPP credentials as usage of non-3GPP credentials in this release is per service requirement for isolated networks only.

Proposal 4: Replace the EN on service continuity with a NOTE.

The configured list in the UE currently refer to "Equivalent Home Service Provider" list and/or a "Service Provider Controlled Network Selector" list that assists the UE in network selection. To be fully aligned with TS 23.122 it is proposed to also add a "User Controlled Network Selector" list as an option.

Proposal 5: Add “User Controlled Network Selector” list in the UE as an option.

Regarding the EN on interface impacts:

Editor's note:	Other impact on N8, N12, N16, N24 and N31 reference points is FFS.

In our understanding there is no additional impact beyond support for the PlmnIdNid data type.

Proposal 6: Replace the EN on interface impact by listing the reference points that need to support the PlmnIdNid data type.

# 2	Proposal

It is proposed to update TR 23.700-07 as indicated below.

* * * * Start of Change * * * *

## 6.1	Solution #1: Standalone non-public network supporting service providers

### 6.1.0	Definitions

Visited-SNPN (V-SNPN): An SNPN for which the UE does not have a subscription associated with the SNPN's identity (PLMN ID and NID combination) and which supports access using credentials (Home SP credentials) owned by an entity separate from the SNPN.

Home Service Provider (Home SP): An entity separate from the SNPN that supports that its subscription is used to access a (different) SNPN. Also referred in the text simply as "Service Provider".

### 6.1.1	Introduction

The solution addresses key issue #1 (Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN). The solution also addresses the service continuity aspect in KI#2.

Figure 6.1.1-1: Standalone Non-public network supporting service providers with services provided by the Serving SNPN; Home SP = SNPN

Figure 6.1.1-2: Standalone Non-public network supporting service providers with services provided by the Serving SNPN; Home SP = PLMN

Figure 6.1.1-3: Standalone Non-public network supporting service providers with services provided by the Service Provider; Home SP =SNPN

Figure 6.1.1-4: Standalone Non-public network service providers with services provided by the Service Provider; Home SP =PLMN

NOTE 1:	The entity separate from the SNPN is referred to as service provider. The SP owns at least an AUSF and a UDM.

NOTE 2:	Reference points between the SNPN and the Home SP need not be roaming interfaces (e.g. N12, N8, N10, N16…).

The following are the main principles of the solution:

-	Depicted in Figure 6.1.1-1 and Figure 6.1.1-2 is the architecture for Standalone Non-Public Network (SNPN) supporting Service Providers with services provided by the Serving SNPN for the case where SP is an SNPN and a PLMN, respectively.

-	Depicted in Figure 6.1.1-3 and Figure 6.1.1-4 is the architecture for SNPN supporting Service Providers with services provided by the Service Provider for the case where SP is an SNPN and a PLMN, respectively.

-	It is assumed that there is a service level agreement between the SNPN owner and the Service Provider.

-	The Service Provider owns the subscription of the UE(s) and Standalone NPN provides access and connectivity for the UE. When the Service Provider uses PLMN credentials for its subscribers, the UE uses USIM credentials.

-	For each subscription with a service provider, the UE is configured with an "Equivalent Home Service Provider" list and/or a "Service Provider Controlled Network Selector" list and/or "User Controlled Network Selector" list that assists the UE in network selection.

### 6.1.2	Functional Description

The solution has the following assumptions:

-	A standalone non-public network has control plane and optionally user plane interfaces with one or more service providers. At the minimum, the 5GC of the SNPN needs to have an interface with the AUSF/UDM residing in the Service Provider.

-	For the case where SP=SNPN: the network selection is controlled via configured lists (refer below to the "Equivalent Home Service Provider" list, "Service Provider Controlled Network Selector" list and "User Controlled Network Selector" list) which include (PLMN ID, NID) tuples and the SNPN selection follows the principles of PLMN selection as specified in TS 23.122 [5] by replacing PLMN IDs with (PLMN ID, NID) tuples.

NOTE X:	Equivalent Home Service Provider list will not be pursued in Rel-17 as equivalent (H)SNPN functionality has been deprioritized, see also clause 5.5.1

-	For the case where SP=PLMN: the network selection is controlled via configured lists (refer below to the "Equivalent Home Service Provider" list, "Service Provider Controlled Network Selector" list and "User Controlled Network Selector" list) which include a mix of both PLMN IDs and (PLMN ID + NID) tuples and the SNPN selection is fully integrated into PLMN selection as specified in TS 23.122 [5].

Editor's note:	For the case where SP=PLMN, SA WG1 needs to clarify the service requirements for the solution and CT WG1 needs to confirm the solution and therefore the network selection proposal in this solution for the case where SP=PLMN is FFS.

NOTE 2:	When using SNPN credentials the device is in SNPN access mode. For devices configured with both SNPN and PLMN credentials, the SNPN access mode is turned on or off when the device uses SNPN or PLMN credentials, respectively.

The solution is based on the following principles:

-	The UE has subscription with Service Provider identified with a service provider ID. The service provider identifier may be identified by a PLMN ID or PLMN ID+NID.

-	When the Service Provider of the UE is identified by a PLMN ID, the UE subscription data and credentials are stored at the UDM/UDR and the UE uses USIM credentials. This is represented in Figure 6.1.1-2 and Figure 6.1.1-4.

-	The stand-alone non-public network can provide local services to the UE using the architecture in Figure 6.1.1-1 and Figure 6.1.1-2, or provide access to the Service Provider services using the architecture in Figure 6.1.1-3 and Figure 6.1.1-4.

-	When the Service Provider of the UE is identified by a PLMN ID, the standalone non-public network selects the session management function and PDU Session Anchor (PSA) in the SP to support PDU sessions that require service continuity using the architecture in Figure 6.1.1-3 or Figure 6.1.1-4.

-	Mobility in the absence of the N14 interface between SNPN and PLMN is handled by the "Existing PDU Session" indication in the PDU Session Establishment Request.

NOTE 4:	Service continuity even between SNPN requires 3GPP credentials being used as usage of non-3GPP credentials in this release is per service requirement for isolated networks only. Session establishment and continuity with an Home SP requires 3GPP credentials as usage of non-3GPP credentials in this release is per service requirement for isolated networks only.

-	When the Service Provider of the UE is a SNPN, the UE subscription data and credentials are maintained by the SNPN. This is represented in Figure 6.1.1-1, where UE has subscription with the Service Provider which is a SNPN and the UE is authorized to services provided by the SNPN.

-	The NG-RAN nodes in the standalone non-public network broadcasts the following:

-	List of combined PLMN IDs and NIDs identifying the standalone non-public networks to which the NG-RAN provides access.

-	The UE shall be configured with the following:

-	Subscriber identifier and credentials for the subscribed service provider (which is identified by PLMN ID or a combination of PLMN ID and NID).

-	A "Service Provider Controlled Network Selector" list which is modelled based on the Operator Controlled PLMN Selector with Access Technology list defined in TS 23.122 [5]. This list can include both PLMN IDs and (PLMN ID, NID) tuples (case where SP=PLMN) or only (PLMN ID, NID) tuples (case where SP=SNPN)  ranked in priority order.

-	A "User Controlled Network Selector" list which is modelled based on the User Controlled PLMN Selector with Access Technology list defined in TS 23.122 [5]. This list can include both PLMN IDs and (PLMN ID, NID) tuples (case where SP=PLMN) or only (PLMN ID, NID) tuples (case where SP=SNPN)  ranked in priority order.

-	In addition, the UE may be configured with an "Equivalent Home Service Provider" list which is modelled based on the EHPLMN list defined in TS 23.122 [5]. This list can include both PLMN IDs and (PLMN ID, NID) tuples (case where SP=PLMN) or only (PLMN ID, NID) tuples (case where SP=SNPN)  ranked in priority order.

-	The above information can be updated by a Home SP (for its subscription) using the UE Configuration Update procedure as defined in TS 23.502 [6] clause 4.2.4.2. An alternative is to use the UE parameter update procedure and other provisioning methods can be considered by Stage 3 WGs.

If the Rel-17 UE does not need to be prevented from attempting to register with a Rel-16 SNPN completely (i.e. not even make a single attempt), the Rel-17 UE may attempt to connect to a Rel-16 SNPN (e.g. because the serving SNPN identity is configured in the UE configured lists). In this case the SNPN rejects the registration attempt with an appropriate rejection cause specified in TS 24.501 [8] e.g. #74 (Temporarily not authorized for this SNPN) or #75 (Permanently not authorized for this SNPN). This will prevent the Rel-17 UEs (either temporarily or permanently) from future registration attempts with this SNPN.

If the Rel-17 UE needs to be prevented from attempting to register with a Rel-16 SNPN completely (i.e. not even make a single attempt), then it is proposed to use a new SIB indication (with the meaning "access using Home SP credentials is supported") so that Rel-17 UEs can only attempt to connect to an SNPN using Home SP credentials when this indication is advertised. For introducing this indication, co-ordination with RAN WG2 is required.

### 6.1.3	Procedures

The UE configuration is described in clause 6.1.2.

The principles for standalone non-public network discovery and selection with support for service providers is as given below:

-	UE has subscription with a service provider identified in the configured lists in the UE by its PLMN ID or combination of PLMN ID and NID. (For example - identified as SP#X).

-	SP#X has an agreement with standalone non-public network (NPN#A) that is identified in the information broadcasted on the radio interface by its combination of PLMN ID and NID (For example - identified as NPN#A), allowing the UEs that have subscription with SP#X to access the services of SP#X via NPN#A.

-	The UE has a stored "Service Provider Controlled Network Selector" list and/or a "User Controlled Network Selector" list, and optionally an "Equivalent Home Service Provider" list, that contains both SP#X and NPN#A.

-	The radio access network of NPN#A broadcasts its own ID i.e. NPN#A.

-	The UE which has subscription with SP #X selects NPN #A if NPN #A is configured in one of UE's configured lists. The UE evaluates the configured lists in the following precedence order for network selection: first "Equivalent Home Service Provider" list, then "User Controlled Network Selector" list, then "Service Provider Controlled Network Selector" list. This is similar as currently specified in TS 23.122 [5] where EHPLMN list has higher priority than Operator Controlled PLMN Selector list.

NOTE:	If the UE is in coverage of the service provider's SNPN (i.e. the information broadcasted on the radio interface includes the PLMN ID + NID tuple of the SNPN), the UE selects this SNPN.

-	Once the network selection is complete, the UE registers with the network by providing the UE identity, (including the SP identity e.g. as part of the "realm" part in the UE identity) and credential corresponding to the service provider. It is assumed that the authentication request is routed to UDM in the service provider's administrative domain.

-	If during network selection the UE has exhausted all the PLMN ID or PLMN ID + NID tuples in the configured list without finding a match with the information advertised on the radio interface, the UE does not attempt to register with any SNPN.

-	If the UE has subscriptions with multiple service providers, this solution assumes that the user selects the subscription which it wants to use before network selection is triggered.

### 6.1.4	Impacts on services, entities and interfaces

UE:

-	UE is configured with "Service Provider Controlled Network Selector" list and/or "User Controlled Network Selector", and optionally an "Equivalent Home Service Provider" list, that contains both SNPN identities and PLMN identities (case where SP=PLMN) or only SNPN identities (case where SP=SNPN).

-	Integrated PLMN and SNPN selection procedure if SP=PLMN.

-	SNPN selection procedure enhancements if SP=SNPN

NG-RAN:

-	None (if the use of rejection cause is deemed sufficient) or a new SIB indication with the meaning of "access using Home SP credentials is supported".

5GC:

-	Support for PlmnIdNid data type on N8, N12, N16, N24 and N31.

* * * * End of Change * * * *