3GPP TSG-SA2 Meeting #140E	S2-2005968

19 August - 2 September, 2020, Electronic	(revision of S2-205045)

Source:	NTT DOCOMO

Title:	KI #1, Sol #9: Update to resolve EN

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This pCR resolves the open EN in Solution#9.

# 1 Discussion

The Solution #9 in TR 23.748 has an EN:

Editor's note:	how the AF performs the mapping or receive the mapping between the GPSI and the UE IP address is FFS.

This paper clarifies how the AF determines the mapping in new clause 6.9.2.2.

# 2 Proposal

Start first change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 23.502: "Procedures for the 5G System".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[6]	3GPP TS 22.261: "Service requirements for the 5G system".

[7]	IETF RFC 7871: "Client Subnet in DNS Queries".

[8]	IETF RFC 1546: "Host Anycasting Service".

[9]	IETF RFC 1034: "Domain Names - implementation and specification".

[10]	IETF RFC 7858: "Specification for DNS over Transport Layer Security (TLS)".

[11]	IETF RFC 8484: "DNS Queries over HTTPS (DoH)".

[12]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[13]	IETF RFC 792: "INTERNET CONTROL MESSAGE PROTOCOL".

[14]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[15]	IETF RFC 1122: "Requirements for Internet Hosts - Communication Layer".

[16]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[17]	IETF RFC 8311: "Relaxing Restrictions on Explicit Congestion Notification (ECN) Experimentation".

[18]	IETF draft draft-ietf-tsvwg-l4s-arch-06: " Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[19]	IETF draft draft-ietf-tsvwg-ecn-l4s-id-10: "Identifying Modified Explicit Congestion Notification (ECN) Semantics for Ultra-Low Queuing Delay (L4S)".

[20]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[21]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[22]	3GPP TS 32.421: "Subscriber and equipment trace; Trace concepts and requirements".

[23]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[24]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[25]	IETF RFC 7556: "Multiple Provisioning Domain Architecture".

[26]	IETF RFC 6731: "Improved Recursive DNS Server Selection for Multi-Interfaced Nodes".

[27]	IETF RFC 8684: "TCP Extensions for Multipath Operation with Multiple Addresses".

[28]	IETF RFC 7681: "Email Exchange of Secondary School Transcripts".

[29]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[30]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[30]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[x]	3GPP TS 29.508: “Session Management Event Exposure Service; Stage 3”.

Start second change

## 6.9	Solution #9: Assist DNS resolution without connectivity between local and central data network

KI#1 on Discovery of Edge Application Server involves aspects related to assisting the DNS resolution.

The proposed solution supports the "Session Breakout" connectivity model with dynamic insertion of local PSA for Edge Computing.

In particular, the solution considers the scenario determined by the following requirement in clause 4.3 (General Requirements and Assumption):

-	It shall be possible for an edge computing hosting environment to have no connectivity with the central data network.

### 6.9.1	Description

In the scenario considered by this solution, the UE has established a PDU Session with a DN to perform communication with Edge Application Servers (EAS) deployed in local Data Networks (LDNs). For the edge application, Authoritative DNS Server(s) are reachable via the LDNs. A DNS Resolver in the central DN is used to forward a DNS query from the UE to the Authoritative DNS Server. The address of the DNS Resolver is provided to the UE via PCO during the PDU Session Establishment.

If direct connectivity between the central DN and the LDNs is supported, the DNS Resolver forwards the DNS query to the Authoritative DNS Server directly.

The proposed solution addresses the scenario where direct connectivity between the DNS Resolver and the LDNs is not supported: in this case the DNS Resolver, acting as an AF, requests the 5GC to forward the DNS query. A relevant scenario where the is no connectivity between the DNS Resolver and the LDNs is depicted in Figure 6.9.1-1. In this scenario the local DN belongs to an Enterprise and the DNS Resolver is in the operator's network (i.e. N6 network). In this case maintaining direct connectivity (e.g. a VPN) between the operator's network and the LDN for all the LDNs of a multiplicity of Enterprises is complex/expensive.

Figure 6.9.1-1: "no connectivity" scenario

NOTE: The ADNS in this scenario may be located in a private DN reachable from a multiplicity of local N6 interfaces and local DNs belonging to the Enterprise.

### 6.9.2	Procedures

#### 6.9.2.1	DNS query forwarding via 5GC

Figure 6.9.2.1-1 shows the procedure to support the application layer in forwarding via 5GC Control Plane the DNS query and the related DNS response. The procedure applies in non-roaming and LBO scenarios. The procedure applies when the DNS resolver has no direct interface with the authoritative server.

In step 0, the UE establishes a PDU Session to perform communication for an edge application. Via UPF1, the PDU provides connectivity to the DN where the DNS Resolver resides. Via UPF2, the PDU Session may provide also connectivity to a local N6 to the local DN where EAS and Authoritative DNS Server are deployed.

1.	Via the UP of the PDU Session, the UE sends (step 1a) a DNS query to resolve the FQDN of a EAS. Via UPF1, the DNS query is forwarded to the DNS Resolver. Based on the knowledge of the UE location, the DNS Resolver selects the Authoritative DNS Server (based on configuration in the DNS resolver, which maps the UE location to the Authoritative DNS IP address) and needs to forward the DNS query. Since there is no direct connectivity between the DNS Resolver and the local DN, the DNS Resolver acts as AF (step 1b) to request the forwarding via 5GC.

NOTE 1:	How the DNS Resolver gets the information on the UE location is not in the scope of this solution. The DNS Resolver can get this is information from the AF, which can get it by subscribing for UE location change notification from the 5GS.

2.	Acting as an AF, the DNS Resolver invokes an SMF service operation to request forwarding the DNS query to the local DN. The parameters of the SMF service operation are the UP packet used to send the DNS query and a destination identifier. The destination identifier can be e.g. a DNAI of the local N6 if the AF interacts directly with the SMF, or an AF-Service-ID (see TS 23.501 [2], clause 5.6.7) if the AF interacts with the SMF via NEF. The AF indicates also an address for notifications on the DNS response. The implementation of the SMF service operation (and of the related NEF service operation) is discussed in clause 6.9.3.

NOTE 2:	AF influence on traffic routing in Rel-16 applies only to non-roaming and LBO scenarios. The same applies also for the proposed solution, which assumes that the AF is in the V-PLMN. In the LBO scenario, the address of the DNS Resolver is assigned by the V-SMF to the UE.

NOTE 3:	The AF can determine the SMF ID and map the GPSI to the UE IP address as described in clause 6.9.2.2.

3.	If not yet inserted, the SMF inserts UPF2 and configured the ULCL and local PSA. Via N4, the SMF sends the DNS query to the UPF2 and configures the UPF2 to forward the DNS query to the local N6 and to intercept the DNS response and forward it to the SMF. Control of User Plane Forwarding in order to forward traffic to/from the SMF is described in TS 23.501 [2], clauses 5.8.2.5.1 and 5.8.2.5.2 and in TS 29.244 [31], clause 5.3: the SMF can create a PDI (packet detection information) in a PDR with a filter that matches e.g. with dst IP address = DNS resolver address and src IP address = L-DNS server address; the UPF forwards the matching packets to the tunnel towards SMF.

4.	The UPF2 intercepts the DNS response by matching the PDI configured by the SMF in step 2and forwards it to the SMF.

5.	The SMF forwards the DNS response to the address for notifications provided by the AF in step 2. This can be done by a notification. The implementation of the notification to the AF (and the related NEF service operation) is discussed in clause 6.9.3. The DNS Resolver forwards the DNS response to the UE via UP of the PDU Session.

Figure 6.9.2.1-1: DNS query forwarding via 5GC

#### 6.9.2.2 Mapping of GPSI to UE IP address

The AF subscribes to the NEF on “PDU Session Status” events for the UE (TS 23.502 [3] Table 4.15.3.1-1); the AF uses the GPSI to identify the UE (TS 23.502 [3] clause 5.2.6.2.2 Nnef_EventExposure_Subscribe operation). The AF subscribes to receive the notifications directly from the SMF. The subscription can be done contextually with AF influence on traffic routing (TS 23.502 [3] clause 4.3.6.1).

At PDU Session establishment the SMF notifies the LDNSR AF on the PduSessionStatus including the SMF ID, the UE IP address (TS 29.508 [x] clause 4.2.2.2) and the GPSI. Finally, the AF stores the association between GPSI and UE IP address and the SMF ID. This option does not have any new requirement to 3GPP specifications.

### 6.9.3	Impacts on services, entities and interfaces

The proposed solution introduces new SMF and NEF service operation to request forwarding of UP packets via 5GC and uses notifications to carry UP packets to the AF:

- Step 2, implementation of the SMF service operation and of the related NEF service operation:

- The SMF service operation can be implemented as an extension of Nsmf_PDUSession_SendMOData (TS 23.502 [3] clause 5.2.8.2.12) or as a new SMF service operation.

- The NEF service can be implemented as an as an extension of the Nnef_TrafficInfluence service operation (TS 23.502 [3] clause 5.2.6.7), or as an extension of the Nnef_NIDD service operation (TS 23.502 [3] clause 5.2.6.14) or as a new NEF service operation.

The parameters of the SMF and NEF service operations need to include the UP packet used to send the DNS query, the destination identifier (e.g. a DNAI of the local N6 if the AF interacts directly with the SMF, or an AF-Service-ID if the AF interacts with the SMF via NEF) and an address for notifications on the DNS response.

- Step 5, implementation of the notification to the AF and the related NEF service operation:

- The notification to the AF from the SMF can be implemented as an extension of the Nsmf_EventExposure service operation or as a new SMF service operation

- The notification to the AF from the NEF can be implemented as an extension of the Nnef_EventExposure service operation or as a new NEF service operation.

End of changes